3GPP TSG-RAN WG4 Meeting # 112 	R4-2412071

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the co-existence simulation assumptions for NTN HPUE

Agenda Item:	8.8.2.1

Document for:	Approval

- Introduction

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

In [1], a new WID is approved for NTN HPUE and the co-existence related work is listed below:

In this contribution, we provide our views on simulation assumptions for NTN HPUE.

- Discussion

2.1 Evaluation scenarios

In [2], 3 type of satellite (GEO, LEO-600km, LEO-1200km) and 2 type of deployment (Urban macro and Rural) are considered for 6 cases as listed below:

Table 1: Aggressor and victim for NTN co-existence

| No. | Combination | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL |
| 2 | TN with NTN | TN UL | NTN UL |
| 3 | TN with NTN | NTN DL | TN DL |
| 4 | TN with NTN | NTN UL | TN UL |
| 5 | TN with NTN | NTN UL | TN DL |
| 6 | TN with NTN | TN DL | NTN UL |

There will be total 36 cases but for NTN HPUE, the interference of NTN UE increases and only the scenarios NTN UL interferer TN needs to be evaluated, i.e., case4 and case5 in the table above.

Observation 1: For NTN HPUE, only the case 4 and case 5 need to be re-evaluated.

After checking the simulation results in [2], for the NTN UL interferer TN, case 4 with GEO in Uma is the worst case to derive the ACLR/ACS requirement. To save effort, only re-evaluate this case is enough.

Observation 2: In the previous PC3 NTN co-existence study, NTN GEO UL interferer Uma TN UL is the worst case for NTN UL interferer TN.

Proposal 1: Only the NTN GEO UL interferer Uma TN UL need to be evaluated for NTN HPUE.

  - Simulation assumptions

Considering the co-existence of both NR NTN and IoT NTN are already evaluated in previous release, the simulation assumption should be same and only NTN UE power/power control parameters need to be modified

Proposal 2: For NR NTN HPUE, the simulation assumptions in TR38.863 can be used and for IoT NTN HPUE, the assumptions in WF R4-2217473 can be used.

As for the power class to be evaluated, since from simulation perspective, the only changes are power related parameter and the overhead is negligible, we prefer all higher power should be evaluated.

Proposal 3: All PC1/PC1.5/PC2 are suggested to be evaluated

Another issue we want to point out is the ratio of NTN HPUE. In our understanding, it may not be a practical scenario that all NTN UE support high power, so we suggest to discuss whether it is possible to have a ratio of NTN HPUE in the simulation to make the evaluation more realistic.

Proposal 4: It is suggested to discuss the ratio of NTN HPUE in all NTN UE in the simulation assumption.

- Conclusion

Observation 1: For NTN HPUE, only the case 4 and case 5 need to be re-evaluated.

Observation 2: In the previous PC3 NTN co-existence study, NTN GEO UL interferer Uma TN UL is the worst case for NTN UL interferer TN.

Proposal 1: Only the NTN GEO UL interferer Uma TN UL need to be evaluated for NTN HPUE.

Proposal 2: For NR NTN HPUE, the simulation assumptions in TR38.863 can be used and for IoT NTN HPUE, the assumptions in WF R4-2217473 can be used.

Proposal 3: All PC1/PC1.5/PC2 are suggested to be evaluated

Proposal 4: It is suggested to discuss the ratio of NTN HPUE in all NTN UE in the simulation assumption.

- References

- RP-241281, WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

- TR 38.863