3GPP TSG-RAN WG4 Meeting # 112 	R4-2412072

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the test method for sTxMP

Agenda Item:	8.13.2

Document for:	Approval

- Introduction

In this contribution, we further provide our views on how to verify the UEs that support sTxMP.

- Discussion

  - MPR/A-MPR verification in the Pumax

In current TS 38.521-2, the test description for configured transmitted power for single carrier is defined as below:

6.2.4.4	Test description

This test is covered by clause 6.2.1 Maximum output power, 6.2.2 Maximum output power reduction and 6.2.3 UE maximum output power with additional requirements.

This means that the configured transmission power doesn’t has a dedicated test procedure, and only if the MOP, MPR, A-MPR pass the test, the Pcmax can be validated.

Observation 1: The configured transmission power is verified by testing MOP, MPR, A-MPR respectively.

In TS 38.101-2, it is noted that MPR/A-MPR are relaxed by 3dB when UE active sTxMP:

PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP  ≤ PUMAX,f,c,k ≤ EIRPmax

ΔMPRSTxMP is [3.0] dB if two TCI states are indicated for simultaneous transmission to multiple directions, 0.0 dB otherwise.

The MPR requirement is derived to ensure UE can meet the emission requirement, e.g., SEM, ACLR, etc. By applying 3dB MPR, the total radiated power of UE will not be greater than if the UE only activated one antenna module and at this stage, the UE can meet the MPR requirement of single carrier without any effort, since it is already verified.

Observation 2: The MPR requirement is relaxed by 3dB in RF spec to ensure the total radiated power of UE will not exceed the situation when only one antenna module is activated, so the UE can meet MPR of single carrier without any additional effort.

Proposal 1: The MPR/A-MPR is not need to be verified during the test of the configured transmission power.

  - UE orientation and AoA offset

In the last meeting, we propose a method to skip the peak search of sTxMP and choose same peak when only single antenna module are activated, as the option 2 that recorded in [1]

Issue 1-4-1: AoA separation and UE orientation for EIRP PUMAX,f,c,k testing

- Agreements:

  - FFS on AoA separation and UE orientation for EIRP PUMAX,f,c,k testing considering the following options

    - Option 1: Consider similar as Multi-Rx UE declaration approach for one AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and one proper UE orientation listed in TS38.101-2 for EIRP per TCI measurement

      - How to process the measurement data is FFS

      - Data processing should avoid repeated selection of peak EIRP per TCI.

    - Option 2: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2), another AoA is decided by UE declared orientation (listed in TS38.101-2) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

To validate our method, a set of simulation results are shown in below. In this simulation, a UE equipped with two 1x4 panels in opposite side is used.

| Orientation#1 AoA offset | panel#1 peak angle [theta, phi] | panel#1 peak EIRP /dBm | panel#2 peak angle [theta, phi] | panel#2 peak EIRP /dBm | AoA pair when AoA1 EIRP is maximum [theta, phi] | AoA pair EIRP [AoA1,AoA2]/dBm | AoA pair when AoA2 EIRP is maximum [theta, phi] | AoA pair EIRP [AoA1,AoA2]/dBm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30° | [165,90] | 26.49 | [10,90] | 26.4 | [170,90] [170,120] | [26.44, 19.75] | [165,60][165,90] | [19.29,26.49] |
| 60° |  |  |  |  | [165,90][165,150] | [26.49,17.82] | [165,30][165,90] | [15.34,26.49] |
| 90° |  |  |  |  | [165,90][165,180] | [26.49,14.09] | [165,0][165,90] | [16.48,26.49] |
| 120° |  |  |  |  | [165,90][345,150] | [26.48,19.16] | [345,30][165,90] | [17.84,26.49] |
| 150° |  |  |  |  | [165,90][345,120] | [26.49,23.64] | [345,60][165,90] | [23.00,23.49] |

| Orientation#2 AoA offset | panel#1 peak angle [theta, phi] | panel#1 peak EIRP /dBm | panel#2 peak angle [theta, phi] | panel#2 peak EIRP /dBm | AoA pair when AoA1 EIRP is maximum [theta, phi] | AoA pair EIRP [AoA1,AoA2]/dBm | AoA pair when AoA2 EIRP is maximum [theta, phi] | AoA pair EIRP [AoA1,AoA2]/dBm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30° | [90,105] | 26.49 | [270,100] | 26.4 | [90,100] [90,130] | [26.44, 18.87] | [270,130] [270,100] | [18.87,26.40] |
| 60° |  |  |  |  | [90,105] [90,165] | [26.49,19.39] | [90,45] [90,105] | [19.43,26.49] |
| 90° |  |  |  |  | [90,105] [270,165] | [26.49,20.49] | [90,15] [90,105] | [21.37,26.49] |
| 120° |  |  |  |  | [90,105] [270,135] | [26.49,24.26] | [270,15] [90,105] | [23.00,26.49] |
| 150° |  |  |  |  | [90,105] [270,105] | [26.49,26.31] | [270,45] [90,105] | [23.79,23.49] |

The following observations can be found:

Observation 3:

- Even with peak search of sTxMP, the peak direction always aligns with peak direction of single antenna module.

- Due to the TE can only implemented with several fixed AoA offset, it is impossible to ensure TE can find two peak directions from arbitrary AoAs. For example, the case above require AoA offset =155°

- Under proper UE orientation and AoA offset, a AoA pair that close to the peak for each antenna module can be found, as the green part above.

Based on the simulation result and analysis above, we believe our proposal can used for sTxMP testing to save test time.

Proposal 2: The following method is agreed to be used for sTxMP testing: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2), another AoA is decided by UE declared orientation (listed in TS38.101-2) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}

- Conclusion

Observation 1: The configured transmission power is verified by testing MOP, MPR, A-MPR respectively.

Observation 2: The MPR requirement is relaxed by 3dB in RF spec to ensure the total radiated power of UE will not exceed the situation when only one antenna module is activated, so the UE can meet MPR of single carrier without any additional effort.

Proposal 1: The MPR/A-MPR is not need to be verified during the test of the configured transmission power.

Observation 3:

- Even with peak search of sTxMP, the peak direction always aligns with peak direction of single antenna module.

- Due to the TE can only implemented with several fixed AoA offset, it is impossible to ensure TE can find two peak directions from arbitrary AoAs. For example, the case above require AoA offset =155°

- Under proper UE orientation and AoA offset, a AoA pair that close to the peak for each antenna module can be found, as the green part above.

Proposal 2: The following method is agreed to be used for sTxMP testing: For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enable (from Beam Peak Search for MOP in 6.2 of TS 38.101-2), another AoA is decided by UE declared orientation (listed in TS38.101-2) and AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}

- References

- R4-2406087	Way Forward for [110bis][339] NR_FR2_OTA	Qualcomm