# 3GPP TSG-RAN WG4 Meeting #112	R4-2412073

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Further discussion on HPUE for intra-band contiguous and non-contiguous CA

Agenda Item:	8.1.1.1.1

Document for:	Approval

Introduction

In the last meeting, RAN4 decided UE architectures on PC1.5 for intra-band contiguous and non-contiguous and the WF was captured in [1]. We reached the WF that to first evaluate MPR/A-MPR for UL contiguous CA, and Architecture 2 (2x26 dBm PA + 1 LO with 200MHz BW) was prioritized. Some RF requirements related issues were also discussed, such as Pcmax,c/Pcmax. In this contribution, we will continue on the discussion on PC1.5 for intra-band contiguous and non-contiguous CA.

Discussion

MPR evaluation assumptions

In Rel-17, the MPR analysis of contiguous and non-contiguous CA is different. For contiguous CA, with the baseline architecture of one RF chain and one PA, the evaluation is basically based on simulation as in [2]. However, for non-contiguous CA, the evaluation is based on measurements [3], considering that the reverse IMD introduced by multiple Tx is difficult to be simulated. With the previous analysis of reference architectures, it seems that the multiple Tx is inevitable for achieving PC1.5, and it is likely that measurements is more likely to be preferred way if abide by what PC2 has done.

However, since testing may have more work and more challenging, more detailed solution need to be discussed, based on the reference architecture discussion.

In the following chapter, we tried to give a detailed MPR framework given the experience of 2Tx MPR measurements:

RF assumptions:

  - 4dB post PA losses

  - 10dB/20dB antenna isolation

  - Equal power spectral density and Equal back-off power split for the two antennas

  - Equal power for architecture 2 between 2PA (TxD)

- Usual 3GPP PA calibration for 20MHz QPSK DFT-s-OFDM 100RB0 waveform based on 4dB post PA losses and 1dB MPR.

  - For 26dBm PA: 29dBm at 31dB ACLR

- For RF impairments, assumptions may be not needed for measurements

- Measurements is used where two PA are coupled at their outputs recreating the 10/20dB antenna isolation assumption with the Reverse IMD

- To recreate the effect of CDD the two signal on each antennas can simply have a small delay between each other (a fraction of CP)

For the Equal PSD and Equal back-off power split assumption, it is necessary for intra-band contiguous and non-contiguous CA. Based on this assumption, we think the equation in the WF [1] is meaningful. This equation ensures the equal PSD between two different CCs. When the BW between two CCs are balanced (LCRB1*SCS1=LCRB2*SCS2), the Pcmax can achieve PC 1.5 (26dBm +26dBm). When the BW between too CCs are seriously imbalanced (e.g. the BW ratio between CC1 and CC2 =100), the Pcmax can be approximately be only 26dBm which is not reachable for PC1.5.

FFS For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances

PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10))

Requirements for back-off evaluation:

- Emission requirements (ACLR/SEM/spurious emissions of the targeted power class) are checked by summing the power of the two transmit paths

- EVM is checked for the agreed composite EVM equation

- MPR is provided in the form of back off of total power versus power class nominal power level

- EVM budget for PA:

  - QPSK             10%

  - 16QAM          8%

  - 64QAM          4%

  - 256QAM        1.8%

- Evaluation scenarios:

  - Both CP-OFDM and DFT-s-OFDM waveforms are evaluated

  - Since simulation may not be available, at least some worst case corners are evaluated for inner/outer and edge allocations

  - Since it has the tighter requirements and highest PSD the lowest valid SCS should be used.

  - All modulation orders should be checked:

    - Pi/2 BPSK (no shaping), QPSK, 16QAM, 64QAM, 256QAM (for 256QAM mostly EVM with proper image level)

  - Channel BW configurations should cover the entire channel bandwidth range:

    - At least 5, 20, 50, 100MHz channel bandwidths (depends on the supported CBW of the operating band)

We suggest to adopt above RF assumptions, requirements for back-off evaluation, evaluation scenarios as a starting point for MPR measurement campaign for PC1.5 intra-band contiguous and non-contiguous CA.

Proposal 1：To adopt above RF assumptions, requirements for back-off evaluation, evaluation scenarios as a starting point for MPR measurement campaign for PC1.5 intra-band contiguous and non-contiguous CA.

SAR solutions

For SAR solutions, we had an agreement in the last meeting:

Online agreement: Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC

Considering the duty cycle solution for PC2 of intra-band CA is mandatory and the duty cycle restriction is default, when maxUplinkDutyCycle-PC2-FR1 is absent, the default threshold is 50%. If the duty cycle exceeds the threshold, the power reduction is expected.

Solution for PC1.5 of intra-band CA is extend for the similar way and compatible with PC2, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 2: For PC1.5 of intra-band CA, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Pcmax,C and Pcmax

- PCMAX,C Proposals: - Proposal 1: Define PCMAX,c limitation for each component carrier considering PC1.5 UE architecture of intra-band UL CA. (LGE) - For dualPA-architecture - PCMAX,C limitation for each component carrier is 26dBm - For TxD (dualTx) - PCMAX,CC1 = 26+10*log10( LCRB1*SCS1LCRB1*SCS1+LCRB2*SCS2)+3 - PCMAX,CC2 = 26+10*log10( LCRB2*SCS2LCRB1*SCS1+LCRB2*SCS2)+3 - Proposal 2: Changes to TS 38.101-1 for dual-PA architecture, inset new equation for PCMAX,c (Ericsson, more details refer to R4-2407721) Way forward: FFS in future meetings ## 2.5 PCMAX Proposals: - Option 1: (Skyworks) - MOP for 2Tx architectures (TxD w/wo UL MIMO) is: 29 dBm - MOP for Dual-PA architectures (one PA/CC, 2LO) is: PCmax= 29 + 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dBm - Option 2: (Skyworks) MPR for Dual-PA architectures (one PA/CC, 2LO) is modified by: 10*log(1/2*(1+Min(LCRB1*SCS1,LCRB2*SCS2)/Max(LCRB1*SCS1,LCRB2*SCS2))) dB - Option 3: (Captured in last meeting’s WF) For R19 PC1.5 intra-band non-contiguous /contiguous ULCA with two 26dBm PAs and one PA per CC, the PCMAX is modified as follows to account for RB BW imbalances PCmax=10*log(10^(26/10) + 10^((26-10*log(LCRB1*SCS1/(LCRB2*SCS2)))/10)) - Option 4: For UEs indicating dualPA-Architecture (Architecture #1) the total configured output power PCMAX for the band combination is given by clause 6.2A.4.1.3 for inter-band CA in the current version of 38.101-1, the same applies for TxD (Ericsson) Way forward: FFS in future meetings

For Pcmax,c/Pcmax, there were a few proposals as above. Generally, we prefer not to introduce the BW imbalance item in the Pcmax,c or Pcmax equation. This is just an evaluation assumption we used in the MPR revaluation considering equal PSD. In 2Tx PC2, we also had the same assumption in the MPR evaluation, however, we did not put the restriction on the Pcmax/Pcmax,c equation. Also, in the field, it is not guaranteed whether the two scheduled CC has the same PSD. Therefore, we suggest to only consider equal PSD assumption in the MPR evaluation, and put no restriction on the Pcmax,c/Pcmax equation.

Proposal 3: It is suggested to only consider equal PSD assumption in the MPR evaluation, and put no restriction on the Pcmax,c/Pcmax equation.

Conclusion

This contribution discusses MPR evaluation framework for PC1.5 intra-band contiguous and non-contiguous CA. The following proposals are made:

Proposal 1：To adopt above RF assumptions, requirements for back-off evaluation, evaluation scenarios as a starting point for MPR measurement campaign for PC1.5 intra-band contiguous and non-contiguous CA.

Proposal 2: For PC1.5 of intra-band CA, the default threshold is 25% when maxUplinkDutyCycle-PC2-FR1 is absent, if 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 3: It is suggested to only consider equal PSD assumption in the MPR evaluation, and put no restriction on the Pcmax,c/Pcmax equation.

- References

[1] R4-2410750, WF on high power UE, Samsung, RAN4#111.

[2] R4-2016910, WF on MPR&AMPR simulation assumption for PC2 intra-band contiguous UL, Skyworks Solutions Inc., Qualcomm, RAN4#97-e

[3] R4-2103371, Way forward on NC UL CA PC2 evaluation assumptions and scenarios, RAN4#98-e