# 3GPP TSG-RAN WG4 Meeting #112	R4-2412074

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Further discussion on 6Rx REFSENS requirements for FWA and Handheld UE

Agenda Item:	8.1.1.3.1

Document for:	Approval

Introduction

In the last meeting, we discussed REFSENS requirements and ΔRIB,6R values for 6Rx and the WF was captured in [1]. Some general considerations and topics were presented on the table, based on that, the agenda for 6Rx requirements is further spitted as REFSENS, MIMO layer evaluation, SRS antenna switching requirements and SRS IL imbalance issue. In this contribution, we further discuss the REFSENS requirements for 6Rx.

Discussion

The WF for 6Rx REFSENS requirements in RAN4#111 meeting were captured as follows:

RAN4#111 Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: For ΔRIB,6R, include n77, n78, n79 and [n104] into the same group. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA Issue 1-1-3: Release independence of ΔRIB,6R value Way Forward: Follow RAN4 Chair guidance and defer the release independence discussions until the requirements are stable. Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

For Issue 1-1-1, we think the same ΔRIB,6R value can be applied for band n77, n78, n79 and n104. Firstly, the ΔRIB,6R is estimated as around -3dB, which is mainly from the difference of diversity gain between 6Rx and 2Rx, maybe some implementation margin difference included. We think this value is relaxed enough and achievable for 6Rx in band n104 and other high bands. Secondly, the diversity gain mainly depends on the number of Rx antenna, instead of different frequency bands. Thus, it is suggested to adopt the same ΔRIB,6R for band n104 and n77/n78/n79.

Proposal 1: For ΔRIB,6R requirement, include band n104 into the same group with band n77, n78, n79.

In the last meeting, companies proposed ΔRIB,6R values for handheld UE and FWA and the numbers are summarized as the range in the WF. It can be observed that the range of ΔRIB,6R are within 0.3dB for band n77, n78, n79, [n104] and 1dB for band n41. The differences among companies’ values are very small. We think either pick a middle value or average value from the range is fine for ΔRIB,6R.

Currently, 4Rx and 8Rx ΔRIB,6R values are specified for FWA UE. Only 4Rx ΔRIB,4R is specified for Handheld UE. These requirements are summarized in Table 1. They can be a starting point for deriving the requirements for 6Rx.

Table 1. Existing ΔRIB values for 4Rx and 8Rx

| Operating bands | ΔRIB,4R for FWA | ΔRIB,4R for Handheld | ΔRIB,8R for FWA |
| --- | --- | --- | --- |
| n77, n78, n79, n104 | -2.2 dB | -2.2 dB | -4.0 dB |
| n41 | -2.7 dB | -2.7 dB | -4.3 dB |

For FWA UE, the ΔRIB,6R for 6Rx we suggest is -3.2dB for band n77, n78, n79, n104 and -3.4dB for band n41, which are the middle values between ΔRIB,4R and ΔRIB,8R for FWA.

For handheld UE, some relaxation can be considered based on ΔRIB,6R for FWA, due to tighter form factors. It is suggested to go with 0.2 dB, that is -3dB ΔRIB,6R for band n77, n78, n79, n104 and -3.2 dB for band n41.

For ΔRIB,6R values for different bands or different UE types are of small difference, we are OK to use -3dB for all these bands, for FWA and Handheld UE, which is the most relaxed number can be achieved in all cases.

Proposal 2: It is suggested to define ΔRIB,6R as:

- -3dB for ΔRIB,6R value for all bands n41, n77, n78, n79, n104, for FWA and Handheld UE

Or

- different values for FWA and Handheld

| Operating bands | ΔRIB,6R for FWA | ΔRIB,6R for Handheld |
| --- | --- | --- |
| n77, n78, n79, n104 | -3.2 dB | -3.0 dB |
| n41 | -3.4 dB | -3.2 dB |

Conclusion

This contribution discusses 6Rx REFSENS requirements for both handheld and FWA UEs. The following proposals are made:

Proposal 1：For ΔRIB,6R requirement, include band n104 into the same group with band n77, n78, n79.

Proposal 2: It is suggested to define ΔRIB,6R as:

- -3dB for ΔRIB,6R value for all bands n41, n77, n78, n79, n104, for FWA and Handheld UE

Or

- different values for FWA and Handheld

| Operating bands | ΔRIB,6R for FWA | ΔRIB,6R for Handheld |
| --- | --- | --- |
| n77, n78, n79, n104 | -3.2 dB | -3.0 dB |
| n41 | -3.4 dB | -3.2 dB |

- References

[1] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4#111.