# 3GPP TSG-RAN WG4 Meeting #112	R4-2412076

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Further discussion on SRS antenna switching requirements

Agenda Item:	8.1.1.3.3

Document for:	Approval

Introduction

SRS antenna switching requirements are one of important aspects for 6Rx requirements. In the last meeting, we further clarified the SRS antenna switching configurations and ΔTRxSRS values. The WFs are captured in [1]. In this contribution, we will further discuss the SRS antenna switching requirements.

Discussion

For SRS antenna switching configurations, there was one update in RAN#104:

RAN#104 Conclusion

- RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability.

- Note 1: SRS antenna switching to support 4T6R is precluded.

- Note 2: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

For t3r6, whether to support it needs further confirmation on RAN1 discussion. For t4r6, it is clearly precluded from the scope. At this time being, we can focus on the SRS antenna switching requirements for t1r6 and t2r6.

The WF for SRS antenna switching and ΔTRxSRS of last meeting was captured as follows:

- Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: SRS antenna switching configurations Way forward: RAN4 to focus on discussing ∆TRxSRS for 1tr6 and t2r6 until RAN1 concludes work on 3t6r and 4t6r. Pending RAN1 feedback, RAN4 can consider any necessary updates to the work scope in a Revised WID for the SRS antenna configurations subject to RAN Plenary approval. Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. - Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

For SRS antenna switching requirements, we need to study the ΔTRxSRS requirements based on possible RF architectures for t1r6, t2r6 and t1r6-t2r6. This value is the additional insertion loss when UE transmits SRS on Rx antenna port and it is highly related to the RF architecture for SRS antenna switching. Figure 1~3 shows the example RF architectures for supporting t1r6, t2r6 and t1r6-t2r6.

Fig 1. Example RF architecture for t1r6

Fig 2. Example RF architecture for t2r6

Fig 3. Example RF architecture for supporting both t16r and t2r6

In all, the value of ΔTRxSRS is highly related to the RF architecture for SRS antenna switching, including the insertion loss of RF switch (e.g. SP4T, SP2T) and additional SRS trace. For the insertion loss, we give the numbers for each item:

- SP4T: 1/1.3dB

- SP3T: 0.8/1dB

- DP3T: 1.1/1.4dB

- SP2T: 0.5/0.6dB

- The IL of additional SRS Trace: 1.8/2.4 dB

The number before the dash is the insertion loss for band n41, n77, n78, and the number after the dash is for band n79.

Take t1r6 as an example, the insertion loss for Ant 6 is assumed as the worst, which is 4.9 dB (2*0.6+1.3+2.4=4.9dB) for band n79. The insertion loss for Ant 1 is TRx path with minimum insertion loss, which is 1.3 dB. Then the ΔTRxSRS value is 3.6dB for t1r6 for band n79.

Base on our calculation, we derive the ΔTRxSRS values for t1r6, t2r6 and t1r6-t2r6.

| Operating bands | ΔTRxSRS value for t1r6 | ΔTRxSRS value for t1r6 | ΔTRxSRS value for t1r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 2.8 dB | 2.3dB | 3.4dB |
| Band n79 | 3.6 dB | 3dB | 4.4 dB |

The actual RFFE is more complex than the above example RF architectures considering supporting multiple bands and other factors. It is reasonable to consider some implementation margins for ΔTRxSRS requirements for 6Rx. Thus we suggest to define ΔTRxSRS requirements for 6Rx as follows:

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t1r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4 dB | 3.5 dB | 5.5 dB |
| Band n79 | 5 dB | 4.0 dB | 6.5 dB |

For ΔTRxSRS requirements, we suggest not to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. The insertion loss of RF switch and additional SRS trace mainly contributes to ΔTRxSRS requirements. Although the frequency of n104 is higher than n79, the IL of these two aspects is of little difference. We investigated some commercial RF switch whose operating frequency covers band n79 and n104, the IL difference is very small, 0.1~0.2 dB at most. For additional SRS trace, the IL difference for band n79 and n104 is estimated as 1.06dB. When considering the ΔTRxSRS requirements for band n79, we already consider additional margin for the IL of RF switch and additional SRS trace and large implementation margin and we think this can cover the IL difference between band n79 and n104.

Proposal 1: For ΔTRxSRS requirements, no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 2: Define ΔTRxSRS requirements for 6Rx as follows

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t1r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4 dB | 3.5 dB | 5.5 dB |
| Band n79, n104 | 5 dB | 4.0 dB | 6.5 dB |

Conclusion

This contribution discusses ΔTRxSRS requirements for 6Rx considering SRS antenna switching configurations t1r6, t2r6 and t1r6-t2r6. The following proposal is made:

Proposal 1: For ΔTRxSRS requirements, no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 2: Define ΔTRxSRS requirements for 6Rx as follows

| Operating bands | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t1r6 | ΔTRxSRS requirement for t1r6 |
| --- | --- | --- | --- |
| Band n41, n77, n78 | 4 dB | 3.5 dB | 5.5 dB |
| Band n79, n104 | 5 dB | 4.0 dB | 6.5 dB |

- References

[1] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4#111.