# 3GPP TSG-RAN WG4 Meeting #112	R4-2412077

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on system parameters for less than 5MHz in FR1-NTN bands

Agenda Item:	8.8.3.1

Document for:	Approval

Introduction

A new WID [1] was approved for enhanced requirements and test methodology for NR and IoT NTN. There are three parts of work for this WID: HPUE for NTN, NTN testing for NGSO and less than 5MHz for NTN. In this contribution, we discuss system parameters for less than 5MHz for NTN.

Discussion

According to the WID, the objectives for less than 5MHz for NTN are captured as follows:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

RAN4 has specified less than 5MHz for TN in sub-1GHz. For NTN with less than 5MHz, it is suggested to reuse the outcome of TN with less than 5MHz as much as possible. According to the objectives, we summarized the work for system parameters for less than 5MHz for NTN.

- Add 3MHz with 15kHz SCS for NR NTN bands n254, n255, n256

- Reuse NRB number (15RB) and minimum guard band (142.5kHz) for 3MHz

- GSCN parameters need to be defined from 1000MHz to 3000MHz for FR1-NTN bands

For channel bandwidth, we aim to add 3MHz with 15kHz SCS for FR1-NTN bands n254, n255, n256. For the NRB number and minimum guard band, we suggest to reuse the numbers defined for TN. That is 15RB with 142.5kHz minimum guard band for 3MHz for NTN.

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN

| NTN satellite band | SCS kHz |  | UE Channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 30 (NOTE) |
|  | 15 | 3 | 5 | 10 | 15 | 20 |  |
| n256 | 30 |  |  | 10 | 15 | 20 |  |
|  | 60 |  |  | 10 | 15 | 20 |  |
|  | 15 | 3 | 5 | 10 | 15 | 20 |  |
| n255 | 30 |  |  | 10 | 15 | 20 |  |
|  | 60 |  |  | 10 | 15 | 20 |  |
|  | 15 | 3 | 5 | 10 | 15 |  |  |
| n254 | 30 |  |  | 10 | 15 |  |  |
|  | 60 |  |  | 10 | 15 |  |  |

For now, GSCN parameters for 3MHz are defined for 0-1000MHz frequency range in TN as shown in Table 5.4.3.1-2, however, FR1-NTN bands span from L band to S band, that is 1000MHz~3000MHz. If RAN4 introduces 3MHz for FR1-NTN bands, the GSCN parameters need to be extended to 3000MHz.

Table 5.4.3.1-2: GSCN parameters for the global frequency for 3 MHz channel bandwidth

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 1000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 31634 |
| NOTE 1: Only applicable for 15 PRB DCH transmission within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

We’ve done some calculations and some changes need to be made for GSCN parameters for 3MHz:

- Range of frequencies：0-3000MHz

- N=1:4999

- Range of GSCN: 26640 – 41636

To support 3MHz in FR1-NTN bands, the GSCN parameters for 3MHz need to be updated as follows:

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |
| NOTE 1: Only applicable for 15 PRB DCH transmission within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

Conclusion

This contribution gives our initial views on system parameters for FR1-NTN supporting less than 5MHz channel bandwidth. The following proposal is made:

Proposal 1: For FR-1 NTN supporting 3MHz, we propose these for system parameters:

- Add 3MHz with 15kHz SCS for NR NTN bands n254, n255, n256

- Reuse NRB number (15RB) and minimum guard band (142.5kHz) for 3MHz

- Extend GSCN parameters for 3MHz channel raster from 1000MHz to 3000MHz for FR1-NTN bands

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 600 kHz + M * 50 kHz + 300 kHz, N = 4999, M ϵ {1,3,5} (Note 1) | 26638+3N + (M-3)/2 | 26640 – 41636 |
| NOTE 1: Only applicable for 15 PRB DCH transmission within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1. |  |  |  |

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103.