# 3GPP TSG-RAN WG4 Meeting #112	R4-2412079

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on system parameters for Ku band supporting NTN

Agenda Item:	8.9.3

Document for:	Approval

Introduction

In RAN#104, a new WID on introduction of Ku band for NR NTN was approved. In this contribution, we give our initial views on system parameters for Ku band.

Discussion

According to the WID, the objectives for NTN Ku band are captured as follows:

- Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios:

  - For ITU regions 1 and 3:

    - Downlink 10.70 – 12.75 GHz

    - Uplink 13.75-14.5 GHz

  - For ITU regions 2 excluding US:

    - Downlink 10.70 – 12.70 GHz

    - Uplink 13.75-14.5 GHz

  - For ITU regions 1 and 3

    - Downlink 10.70 – 12.75 GHz

    - Uplink 12.75 - 13.25 GHz

  - For ITU regions 2 excluding US

    - Downlink 10.70 – 12.70 GHz

    - Uplink 12.70 - 13.25 GHz

  - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required.

- Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services.

- Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to:

  - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

  - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing

- RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106.

- Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths.

Issue 1.FR extension

According to the objectives, the Ku band is neither FR1-NTN nor FR2-NTN for now. One issue is that how to define the frequency range for this Ku band. From our perspective, the channel bandwidths for Ku band is likely to be FR1-NTN like since existing Ku band system use typical channel bandwidth 18/36/54 MHz. However, radiated RF requirements are likely to be defined for Ku band, which is FR2-NTN like. It is difficult to say this Ku band belongs to FR1-NTN or FR2-NTN. Maybe define a new frequency range for NTN (like FR3-NTN) is not precluded.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Proposal 1. Discuss whether to define FR3-NTN for Ku band.

Issue 2. Band plan

For the band plan for Ku band, we suggest to define two different bands considering different frequency usages in different regions according to the regulation.

FDD band nX: UL 13.75-14.5 GHz DL 10.70-12.75 GHz

FDD band nY: UL 13.75-14.5 GHz DL 10.70-12.70 GHz

The band numbers will be dependent on which frequency range this Ku band belongs to.

Proposal 2. Consider define two Ku bands for different regions as first priority:

FDD band nX: UL 13.75-14.5 GHz DL 10.70-12.75 GHz

FDD band nY: UL 13.75-14.5 GHz DL 10.70-12.70 GHz

Issue 3. Channel bandwidth

Ku band are typically used for fixed satellite television data services and some typical channel bandwidth is 18，36，54MHz for non-3GPP services. Figure 1 shows a typical frequency plan used in Ku band in a particular region. It can be observed that existing ku-band satellite systems use FR1-NTN like channel bandwidth. If there are no scenarios for larger channel bandwidth like FR2-NTN channel bandwidth, we suggest to adopt FR1-NTN like channel bandwidth.

Figure 1. Satellite downlink frequency plans. (a) C band. (b) Plan for Ku band of AMC-1 dual-band satellite. (c) Ku-band plan for AMC-5 Ku band only.

  - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

Observation 1: It can be observed that existing Ku band satellite systems use FR1-NTN like channel bandwidth.

Proposal 3: To adopt channel bandwidth as FR1-NTN.

Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

Issue 4. Channel raster and sync raster

For channel raster and sync raster issue, they are related to the channel bandwidth definition for Ku band. For channel raster, if SCS-based channel raster is adopted, the stepsize for Ku-band is 2 if we consider 30kHz SCS. For GSCN parameters, we can calculate the numbers based on the minimum channel bandwidth supported by the Ku band.

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 – 30000 | 60 | 24250.08 | 2016667 | 2016667 – 2112499 |

In a word, the channel raster and sync raster can be decided after the definition of channel bandwidth for the Ku band.

Proposal 4：The channel raster and sync raster can be decided after the definition of channel bandwidth for the Ku band.

Conclusion

This contribution gives our initial views on system parameters for Ku band NTN. The following proposals are made:

Proposal 1. Discuss whether to define FR3-NTN for Ku band.

Proposal 2. Consider define two Ku bands for different regions as first priority:

FDD band nX: UL 13.75-14.5 GHz DL 10.70-12.75 GHz

FDD band nY: UL 13.75-14.5 GHz DL 10.70-12.70 GHz

Observation 1: It can be observed that existing Ku band satellite systems use FR1-NTN like channel bandwidth.

Proposal 3: To adopt channel bandwidth as FR1-NTN.

Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing

Proposal 4：The channel raster and sync raster can be decided after the definition of channel bandwidth for the Ku band.

- References

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104.