# 3GPP TSG-RAN WG4 Meeting #112	R4-2412080

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on the subband configurations and guardbands for gNB SBFD

Agenda Item:	8.19.1

Document for:	Approval

Introduction

In the last meeting, we further discussed system parameters and the impact on existing BS RF requirements and potentially new requirements for gNB SBFD. The WF for gNB SBFD was captured in [1]. In this contribution, we focus on subband configurations and guard band issues for gNB SBFD.

Discussion

In the last meeting, we discussed operating bands for SBFD operation, however, there was no agreement on this issue. The consensus is that, all TDD bands for FR1 and FR2-1 are potentially to be used for SBFD operations. The agreements are captured as follows:

Issue 1-1-1: SBFD as band specific or general feature to all TDD bands

- Agreement:

  - SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations:

    - Option 1: Declaration based method

      - The band supported for SBFD shall be manufacturer declaration based

      - The channel bandwidth supported for SBFD shall be manufacturer declaration based

        - The supported channel bandwidth can be impacted by the subband/guard band size discussion

    - Option 2: General principle but with consideration(s) for a specific band

      - General principle: The channel bandwidth shall be larger than X MHz

        - FFS the value of X

          - X can be different for high and middle TDD bands

        - For a specific band, the following further restriction(s) can be considered:

          - Certain restriction can be provided by operator(s) for certain band

          - Other restrictions are not precluded.

Only band agnostic BS RF requirements are going to be specified in this WI. It is not preferred to restrict the operating bands for gNB SBFD, however, some general principles for gNB SBFD are fine.

From our perspective, the above two options do not contradict with each other. The gNB venders can always use the declare based method even there are some general principles introduced for a specific band for SBFD operation. We suggest to adopt Option 1 and further discuss Option 2.

For SBFD configurations and guard band issues, we reached following agreements:

Issue 1-2-2: How to handle guard band and subband configurations in specification

- Agreement:

  - Add clarification that only DUD and DU patterns are specified for the sub-band configuration.

  - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements

  - For a certain channel bandwidth which RAN4 agree to introduce RF requirements:

    - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements

      - FFS which UL/DL subbands configuration(s) will be defined in RAN4

      - FFS Guard band size is declaration based and can be different for different BS classes

      - FFS the limitation on the maximum guard band

      - FFS possible range for UL/DL subband sizes

    - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except:

      - 1RB granularity (already introduced in RAN1)

From our perspective, we tend to not restrict RAN1 definition on subband sizes for SBFD. RAN4 only define some typical DUD/DU configurations only for deriving RF requirements purposes. For SBFD configurations, we had some basic assumptions during the study item phase:

- Only DUD and DU configurations are considered

- UL subband occupies 20% BS channel bandwidth

These assumptions were used in the gNB self-interference cancellation and co-existence study. At least these configurations were verified in the study item. Therefore, we still suggest to follow these basic assumptions in the normative phase.

For UL subband, the minimum regular channel bandwidth supported by UE is 5MHz. Since the maximum BS channel bandwidth is 100MHz in FR1, the UL occupies 20% and the maximum UL subband is 20MHz. So, we suggest the UL subband is 5MHz, 10MHz, 15MHz, 20MHz, which are aligned with the regular channel bandwidth sizes.

- When the UL subband is 5 MHz, the DUD and DU configurations are [10, 5, 10] and [20, 5]

- When the UL subband is 10 MHz, the DUD and DU configurations are [20, 10, 20] and [40, 10]

- When the UL subband is 15 MHz, the DUD and DU configurations are [30, 15, 30] and [60, 15]

- When the UL subband is 20 MHz, the DUD and DU configurations are [40, 20, 40] and [80, 20]

For the case 15MHz UL subband occupying 20% BS channel bandwidth, the channel bandwidth is 75 MHz, which is not supported by the BS in FR1. So, we suggest to remove this case.

For FR2-1, we follow the basic SBFD assumptions and the subband size can be calculated for each BS channel bandwidth. In a summary, we create a table for gNB SBFD configurations:

Table 1. BS SBFD configurations in FR1 and FR2-1

| SBFD configuration | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25 | 25 | 50 | 50 | 100 | 100 | 200 | 200 | … |
| DL Subband 1 | 10 | 20 | 20 | 40 | 40 | 80 | 80 | 160 | … |
| UL Subband | 5 | 5 | 10 | 10 | 20 | 20 | 40 | 40 | … |
| DL subband 2 | 10 | N/A | 20 | N/A | 40 | N/A | 80 | N/A | … |

Proposal 1: It is suggested to adopt Table 1 only for deriving BS RF requirements purpose and no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth.

Another issue when we decide the SBFD configurations, the transmission bandwidth configurations for DL and UL subband can also be discussed. The one merit of using Table 1 is that the sizes of DL and UL subband are regular channel bandwidth. The typical NRB configuration for subbands can still follow the transmission bandwidth configuration in TS 38.104.

Table 2: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Proposal 2: The typical NRB configuration for DL or UL subbands can still follow the transmission bandwidth configuration in TS 38.104.

Another tricky issue for SBFD configuration is that whether or not to specify guard band. In the study phase, most companies gave their GB assumptions in the self-interference analysis table, that is 5RB for 100MHz BS channel bandwidths in FR1. Guard band can be seen as frequency isolation technique for self-interference cancellation technique, which is related to gNB implementations. With less GB, it puts more stringent performance requirement on other isolation techniques in order to achieve the required RSIC. Especially for FR1 WA BS, the required RSIC can not be achieved by many gNB vendors, we think reserving some GB can reduce the effort to control the self-interference.

As discussed in the last meeting, if the self-interference level can be controlled well, some gNB vendor can use 0 GB. However, not all gNB vendors can achieve 0 GB especially for FR1-WA BS. If DL/UL subband sizes go with the Table 2, the guard band is implied since the transmission configuration NRB and the minimum GB are two linked requirements. We suggest that the maximum guard band between DL and UL subband can follow Option 3.

Table 3. The maximum guard band between DL and UL subband for SBFD

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

Proposal 3: The guardband between UL/DL subbands can be in the range of [0, GBmax], and GBmax can refer to Table 3.

Conclusion

This contribution discusses SBFD configurations and guardbands for gNB SBFD. The following observations and proposals are made:

Proposal 1: It is suggested to adopt Table 1 only for deriving BS RF requirements purpose and no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth.

Table 1. BS SBFD configurations in FR1 and FR2-1

| SBFD configuration | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25 | 25 | 50 | 50 | 100 | 100 | 200 | 200 | … |
| DL Subband 1 | 10 | 20 | 20 | 40 | 40 | 80 | 80 | 160 | … |
| UL Subband | 5 | 5 | 10 | 10 | 20 | 20 | 40 | 40 | … |
| DL subband 2 | 10 | N/A | 20 | N/A | 40 | N/A | 80 | N/A | … |

Proposal 2: The typical NRB configurations for DL or UL subbands can still follow the transmission bandwidth configuration in TS 38.104.

Proposal 3: The guardband between UL/DL subbands can be in the range of [0, GBmax], and GBmax can refer to Table 3.

Table 3. The maximum guard band between DL and UL subband for SBFD

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

- References

[1] R4-2409958, Way Forward for [111][313] NR_duplex_evo, Samsung, RAN4#111.