# 3GPP TSG-RAN WG4 Meeting #112	R4-2412081

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on RF requirements for RedCap and eRedCap UE supporting FR1-NTN

Agenda Item:	8.25.2.1

Document for:	Approval

Introduction

In the last meeting, we mainly discussed UE RF requirements for (e)RedCap NTN in HD-FDD mode. The WFs were captured in [1]. Two remaining issues for HD-FDD are REFSENS and Maximum output power. In this contribution, we further discuss the remaining issues for (e)RedCap NTN in HD-FDD mode.

Discussion

We had following open issues not being fully discussed in the last meeting. In this contribution, we would like to share our views on these open items.

- Issue 1-1 UL RB allocation for HD-FDD Refsens, both for 1Rx and 2Rx

Agreement: Full RB allocation.

- Issue 1-2: HD-FDD refsens for 2 Rx

Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112.

Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

- Issue 1-3: HD-FDD refsens for 1 Rx

Agreement: 2.5 dB relaxation compared to 2Rx HD-FDD

- Issue 1-4: Frequency error

Agreement: Discuss changes common to non-Redcap NTN and (e)RedCap NTN NR UE frequency error in maintenance agenda. (e)RedCap UE frequency error requirement may be impacted by this maintenance discussion.

Option for further discussionn, not an agreement:

- As a starting point, consider (e)Redcap UE frequency error based on existing NR UE requirements.

- Issue 1-5: Maximum output power

options for further discussion:

Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band.

Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

- Issue 1-6: Specification structure and baseline changes

Agreement: Use R4-2408816 as starting point for (e)RedCap NR NTN UE RF requirements specification structure. Further changes on requirement values and additional impacted clauses (e.g. according to issue 1-4 and 1-5) as well as other improvements can be added.

FFS for suffix letter.

For UE Rx RF requirements for (e)RedCap NTN in Half-Duplex mode, the only focus is on REFSENS requirements. There are two considerations for HD REFSENS:

- Compared to FD-FDD, RF front end my adopt filters with switch for HD-FDD, instead of using duplexer; the insertion loss for HD-FDD may be smaller than FD-FDD due to the difference in RFFE.

- For HD-FDD, receiving DL will not have the interference from UL transmission;

Base on these two considerations, the REFSENS may need tightening for HD, compared with FD-FDD. However, the tightening values need to be studies case by case. In the discussion of REFSENS for RedCap UE, we had following agreements for deriving the requirements:

- The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

Table 1. HD-FDD 5MHz REFSENS tightening from FD-FDD

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70, n91~n94 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

- HD-FDD REFSENS for channel BW wider than 5 MHz can be calculated by REFSENS(5MHz) + 10log10(n x NRB/25), where NRB is the maximum transmission bandwidth configuration with n=1 for 15kHz SCS and n=2 for 30kHz SCS.and n=4 for 60kHz SCS

For 2Rx FR1-NTN FDD bands, the REFSENS is defined as follows:

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.8 |
| n256 | 30 |  | -96.6 | -94.6 | -94.0 |
|  | 60 |  | -97.0 | -94.9 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -99.5 | -96.3 | -94.5 |  |
| n254 | 30 |  | -96.6 | -94.6 |  |
|  | 60 |  | -97.0 | -94.9 |  |

According to above table 1, RFSENS for HD need to tightened by 0 dB for band n255 and 0.5 dB for band n256 and n254.

Then the REFSENS for FR1-NTN HD-FDD bands can be calculated as:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -94.3 |
| n256 | 30 |  | -97.1 | -95.1 | -94.5 |
|  | 60 |  | -97.5 | -95.4 | -94.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Proposal 1: With 0.5 dB tightening for n254 and n256, 0 dB for n255, the 2Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -94.3 |
| n256 | 30 |  | -97.1 | -95.1 | -94.5 |
|  | 60 |  | -97.5 | -95.4 | -94.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

For 1Rx, with additional Delta_1R 2.5dB, we can derive 1Rx REFSENS for FR1-NTN HD-FDD bands.

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -94.3 | -92.5 | -91.8 |
| n256 | 30 |  | -94.6 | -92.6 | -92.0 |
|  | 60 |  | -95.0 | -92.9 | -92.2 |
|  | 15 | --97.5 | -94.3 | -92.5 | -91.3 |
| n255 | 30 |  | -94.6 | -92.6 | -91.5 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
|  | 15 | -97.5 | -94.3 | -92.5 |  |
| n254 | 30 |  | -94.6 | -92.6 |  |
|  | 60 |  | -95.0 | -92.9 |  |

Proposal 2: The 1Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -94.3 | -92.5 | -91.8 |
| n256 | 30 |  | -94.6 | -92.6 | -92.0 |
|  | 60 |  | -95.0 | -92.9 | -92.2 |
|  | 15 | --97.5 | -94.3 | -92.5 | -91.3 |
| n255 | 30 |  | -94.6 | -92.6 | -91.5 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
|  | 15 | -97.5 | -94.3 | -92.5 |  |
| n254 | 30 |  | -94.6 | -92.6 |  |
|  | 60 |  | -95.0 | -92.9 |  |

For Issue 1-5, we still prefer Option 2 for maximum output power for NTN (e)RedCap UE only in HD-FDD operation for a band. That is to use PC3 as the power class for UE only in HD-FDD operation for a band. Firstly, we think this is a general issue for UE only supports HD-FDD, not only restricted to RedCap NTN. In RedCap, we only defined PC3 for UE only in HD-FDD mode. We didn’t consider the power increase due to the insertion loss difference between duplexer and RF switch. For RedCap NTN, we still suggest to follow the same way with RedCap. Secondly, we need to clarify how much power increase for this UE only in HD-FDD mode. In the last meeting, some company mentioned 1dB power increase, however, there is no discussion on this value. From our perspective, the IL difference between duplexer and RF switch will be no larger than 1dB. In the HD-FDD REFSENS discussion, only 0.5dB RFSENS tightening are considered for HD-FDD compared to FD-FDD. Considering the Tx and Rx symmetry, the power increase will be around 0.5dB for some specific bands. Anyway, if we want to further discuss Option 1, we need to clarify the value of power increase. Thirdly, we are not sure about how much standard work will be made for this power increase. It may have some impact on the MPR and out of band emission requirements if we want to study the power increase for RedCap NTN.

In a word, we still think there are a lot of issues for this power increase for RedCap NTN for in only HD-FDD mode. We suggest to go with Option 2.

Proposal 3：Not to consider increase in PC3 power level for NTN (e)RedCap UE which only supports HD-FDD operation.

Conclusion

This contribution discusses some general issues for RedCap or eRedCap UE supporting FR1-NTN bands and the REFSENS requirements for FR1-NTN HD-FDD bands. The following proposals are made:

Proposal 1: The 2Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.0 | -96.8 | -95.0 | -94.3 |
| n256 | 30 |  | -97.1 | -95.1 | -94.5 |
|  | 60 |  | -97.5 | -95.4 | -94.7 |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
|  | 15 | -100.0 | -96.8 | -95.0 |  |
| n254 | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Proposal 2: The 1Rx REFSENS for FR1-NTN HD-FDD bands are proposed:

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -97.5 | -94.3 | -92.5 | -91.8 |
| n256 | 30 |  | -94.6 | -92.6 | -92.0 |
|  | 60 |  | -95.0 | -92.9 | -92.2 |
|  | 15 | --97.5 | -94.3 | -92.5 | -91.3 |
| n255 | 30 |  | -94.6 | -92.6 | -91.5 |
|  | 60 |  | -95.0 | -92.9 | -91.7 |
|  | 15 | -97.5 | -94.3 | -92.5 |  |
| n254 | 30 |  | -94.6 | -92.6 |  |
|  | 60 |  | -95.0 | -92.9 |  |

Proposal 3：Not to consider increase in PC3 power level for NTN (e)RedCap UE which only supports HD-FDD operation.

- References

[1] R4-2410574, WF on Rel-19 phase 3 NR-NTN UE RF requirements, Qualcomm, RAN4#111.