3GPP TSG-RAN4 Meeting #112	R4-2412085

# Maastricht, The Netherlands, Aug 19 – 23, 2024

Agenda item:			8.1.1.2.1

Source:	vivo

Title:	Discussion on power domain enhancement for NR single carrier

Document for:	Discussion

- Introduction

In RAN#111 meeting, the WF[1] has been agreed and the details about power domain enhancement for single carrier are as follows:

1.1 Consideration on relaxation of requirements for identified scenarios Agreement in main session: - No relaxation of level of spurious emission requirements - FFS on whether to change the spurious emission boundary ## 1.2. Mechanisms for enabling MPR reduction and/or power boosting Way forward: - Study the approach to convert full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW, e.g. each side of the UE BW is equal to ½ of the UE BW, inside a larger BS CBW as starting point with consideration of the following aspects - The edge of the extended UE BW, i.e. the UE CBW plus the shifted frequency symmetrically at each side of the UE CBW, should be inside BS CBW or at least aligned with the BS CBW edge - ① FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW - ② FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW - ③ FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB - ④ FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW - Other mechanisms are not precluded ## 1.3 Simulation assumption for power boosting and/or MPR reduction Way forward: Agree with the preliminary simulation assumptions for following evaluation. Assumptions are subject to revisions with further study. - PA model calibration - DFT-s-OFDM QPSK 20MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier Leakage: 28dBc - IQ Image: 28dBc - CIM3: 60dBc - EVM: 17.5% - ACLR: 30dB for PC3, 31dB for PC2

- Discussion: Power domain enhancements for single carrier

## 2.1. Consideration on relaxation of requirements

Considering the relaxation of requirements, there are a few of issues need to be discussed.

### 2.1.1 Output RF spectrum emissions

1. The change of the spurious emission boundary

According to the WF of meeting #111, not relaxing the level of spurious emission requirements has become a consensus. However, the application range of SE still should be altered with the edge of the UE CBW shifting and the change of ACLR and SEM application range. Otherwise, SE would become the limiting factor for MPR reduction, and the application range will partially overlap with SEM, and it would not adheres to the definition and motivation of SE evaluation: Spurious emissions are emissions which are caused by unwanted transmitter effects but exclude out of band emissions unless otherwise stated.

Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW and the change in the application range of ACLR and SEM.

2. The start point of ΔfOOB & integral region and boundary of OOBE

ACLR and SEM would be applicable from the edge of extended CBW, and the integral region and the boundary of OOBE should be based on UE CBW. From one side, SE need to be evaluated from the edge of SEM and according to ITU-R regulations, the SE should apply from 250% of the centre frequency of the “necessary bandwidth”. If the integral region is based on extended UE CBW, the boundary of OOBE would be extended to not only farer than current 3GPP requirements but also disobey the 250% requirement of ITU.

For example, in Figure 1(a), the integral CBW of OOBE is equal to the UE CBW. The SE applies from the edge of SEM and the frequency spacing with centre of UE CBW is 200% * UE CBW+5M, which meets with the requirement of ITU. However, in Figure 1(b), the integral CBW of OOBE is equal to the extended UE CBW. And the SE applies from 300% * UE CBW+5M, which is too much far away from the UE centre especially for the wider UEs and the protection effect of UE co-existence would be weaken a lot.

Proposal 2: The integral region and the boundary of OOBE should be based on UE CBW.

(a)

(b)

Fig.1 Comparison of two integral bandwidths

### 2.2. Mechanisms for enabling MPR reduction and/or power boosting

1. Limitation of the edge of extended UE BW

The extension of the UE boundary is conditional, that is, the distance between the UE CBW or RB configuration and the edge of the BS CBW should be far enough, so that the guardband between the transmitted RBs and the BS edge could be wide enough to prevent the in-band signals from interfering with the UEs outside the BS. Correspondingly, the extended UE edge should not exceed the boundary of the BS CBW, otherwise this would lead to excessive relaxation and increased interference outside the BS.

Proposal 3: The edge of the extended UE BW should be inside BS CBW or at least aligned with the BS CBW edge.

2. Applying range of IBE

After extending the UE CBW, the applied evaluation requirement between the edges of UE CBW and extended UE CBW should be discussed. Although the RBs in this “new guardband” could not be utilized in the actual UE, the frequency range still belongs to “in-band” concept as it is a kind of virtual bandwidth set for the UE, so IBE should be applied. Further, the general part of IBE consists of four equations and the max of them is filtered out as the final requirement:

IBE_general=max(PRB - 30 dB, )

Therefore, there are two elements to consider: NRB and ΔRB. They are both related to UE CBW and corresponding RB allocations: NRB represents the maximum transmission bandwidth configuration for each UE channel bandwidth, ΔRB is the starting frequency offset between the allocated RB and the measured non-allocated RB.

According to [2], the NRB of the IBE requirement refers to the total number of RBs in the uplink, which should be based on the actual UE bandwidth indicated by the NW, and should not change with the virtual extension of UE CBW edge. Similarly, the calculation of ΔRB should also be limited within the actual UE CBW. The IBE requirement at the edge of actual UE CBW could be used as a reference to evaluate the radiation power level of the entire extended part. The illustration is as follows:

Fig. 2 IBE applying range after bandwidth expansion

Based on the assumption above, there is another situation need to be discussed: Full RB allocation cases.

Since there are no remaining unallocated RBs, there is no IBE requirement in the full-RB allocation of the UE CBW. Therefore, how to evaluate the emission level between edges of UE CBW and extended UE CBW in this situation is a problem. Two methods can be applied: 1) Not evaluating the emission level in that area. 2) Using IBE requirement to evaluate the emission level, with ΔRB set to 0, indicating that the frequency difference between the allocated RB and the UE CBW edge is 0. The second method is preferable. The original requirements for this area used to be ACLR and SEM, so IBE is a kind of relaxation, and the nearest radiation leakage outside the UE CBW also needs to be evaluated to control the transmit modulation quality of the UE.

Proposal 4: IBE should be used between edges of UE CBW and extended UE CBW. And the IBE of this part should be equal to that of the original UE CBW edge.

3. The required extended UE CBW

In the WF[1], it has been concluded that converting full RB allocation in UE CBW to “inner RB allocation” could be achieved through extending each side of UE edge by 1/2 UE CBW. However, efficient conversion and using the inner MPR requirement is based on the premise that all the in-band and out-of-band requirements of the new extended UE CBW case should be the same as those for a wider bandwidth UE CBW originally. However, based on the analysis of IBE above, it can be seen that the IBE between the UE CBW and the edge of the virtual extended UE CBW is relaxed. ACLR and SEM are also relaxed because the evaluation region also refers to the original UE bandwidth. For example,

Case 1: UE CBW=20M, full RB allocation, extended UE CBW=40M

After UE edge shifting, the original full bandwidth configuration would become an inner configuration. However, the IBE within the extended frequency part is equal to those at the edge of the 20M bandwidth with ΔRB=0 and the corresponding NRB is also based on 20M, as shown in Fig 2. At the same time, the integral bandwidth of ACLR and SEM applying range also refer to 20M.

Case 2: UE CBW=40M, inner allocation: full RB allocation in the middle 20M

The NRB refers to 40M, and ΔRB keeps increasing from the edge of the middle 20M to the edge of 40M. Meanwhile, the integral bandwidth of OOBE is also based on 40M. Therefore, IBE, ACLR, and SEM are all stricter than that in the first case of UE CBW extension.

Comparing case 1 and case 2, it can be seen that extending the bandwidth by less than 1/2 UE CBW is sufficient to reduce the MPR of full RB allocation to the inner level.

Observation 1: Extending the bandwidth by less than 1/2 UE CBW is sufficient to reduce the MPR of full RB allocation to the inner level.

The following table is the simulation results of MPR with different extending bandwidth, original UE CBW 10\20\25\50MHz is configured with full RB allocation. As can be seen, to reduce the MPR of the same UE CBW with full bandwidth configuration to 0, the required bandwidth extension for SCS=30KHz is narrower than that for SCS=15KHz. To be conservative, the required extended bandwidth of the both sides can be set to 2/5 UE CBW as a starting point. Other configurations and cases can be further checked by other companies.

Proposal 5: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point.

Table 1. MPR with different UE CBW extending bandwidth, DFT-s-OFDM，QPSK

| original/extended UE CBW (MHz) SCS(kHz) | 10 |  | 20 |  |  |  | 25 |  |  |  | 50 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 15 | 20 | 25 | 30 | 35 | 40 | 35 | 40 | 45 | 50 | 70 | 80 | 90 | 100 |
| 15 | 0.1 | -0.4 | 0.8 | 0.6 | 0 | -0.2 | 0.4 | 0.1 | 0 | 0 |  |  |  |  |
| 30 | -0.1 | -1 | 0.4 | 0 | -0.4 | -0.5 | 0.3 | 0 | 0 | 0 | 0.4 | 0.1 | 0 | 0 |

According to the simulation results, it can also be concluded that the output power of outer RB allocation is mainly limited by ACLR, and the extension of UE CBW could change the integral region of ACLR to some further range, so the observed out-of-band emission level could be reduced and ACLR could be higher.

But when shifting the edge of UE to some extent the limit factor would change to some other requirements, e.g., IBE. Since for the in-band part, the interference falling in a specific non-allocated RB and the corresponding IBE level will not change with the expansion of the virtual UE bandwidth, continuing to expand the bandwidth outward is not effective in reducing IBE and MPR further. And the MPR has already reduced to inner level earlier. Therefore, excessive bandwidth expansion is a waste of resources.

4. The judgement of shifting UE edge

The new mechanism of the UE to reduce MPR should be discussed, and there are two options could be considered as a starting point. The corresponding indication from the NW to the UE is different and the mapping to MPR will be also decided by different sides:

1) The NW directly indicates that the UE's MPR reduces from outer to inner level:

The NW would judgement the deployment environment, when the RB configuration belongs to outer region and the UE CBW is far enough away from the edge of BS CBW, the NW should send signaling to the UE and indicate MPR reduction. Then UE would refer to the inner MPR requirement instead.

2) The NW indicates the extended bandwidth to the UE, and the mapping of the extended BW to MPR is decided by the UE:

The NW would judgement the deployment environment, when the RB configuration belongs to outer region and the UE CBW is far enough away from the edge of BS CBW, the NW should send signaling to indicate a virtual extended UE CBW to UE, then UE would refer to a new MPR according to the corresponding indication information.

Comparing option1 to option2, option1 is preferred, as there is no need to redefine a new RB region according to the relationship of original UE CBW and the extended CBW. The frequency interval of the original UE CBW and the BS CBW could be easily judged by the NW as it decides the RB allocation of the UEs, the process is more succinct. And when the interval is far enough then a simple indication including changing the MPR to an inner level instead could be send to the UE, and the response could be quicker when the RB allocation changes.

Proposal 6: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met.

How to present the mapping relationship is another issue. As it can be seen that under certain conditions a UE with outer allocation could also refer to the MPR of an inner level, then how could the setting conditions of UE BW shifting and mapping relationship between the original UE CBW, the extended UE CBW and the MPR be covered in the spec and in which spec it should be covered needs to be discussed. From UE’s perspective, it is not necessary to know the virtual UE CBW and the MPR change could also be indicated by the NW, so it seems that clarifying the MPR reduction with the indication from the NW is enough in TS38.101-1. However, the mapping table is still needed as the judgement from the NW also needs a clear guidance. At the same time, whether the relaxation of ACLR,SEM and IBE should be presented in 38.101 or the change should only be known by the testing part, it also needs to be discussed.

Proposal 7: Where and how to present the mapping relationship between the original UE CBW, the extended UE CBW and the MPR table should be clarified.

- Conclusion

Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW and the change in the application range of ACLR and SEM.

Proposal 2: The integral region and the boundary of OOBE should be based on UE CBW.

Proposal 3: The edge of the extended UE BW should be inside BS CBW or at least aligned with the BS CBW edge.

Proposal 4: IBE should be used between edges of UE CBW and extended UE CBW. And the IBE of this part should be equal to that of the original UE CBW edge.

Observation 1: Extending the bandwidth by less than 1/2 UE CBW is sufficient to reduce the MPR of full RB allocation to the inner level.

Proposal 5: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point.

Proposal 6: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met.

Proposal 7: Where and how to present the mapping relationship between the original UE CBW, the extended UE CBW and the MPR table should be clarified.

- References

- R4_2410749 WF on power domain enhancements, Huawei, HiSilicon, RAN4#111.

- R4_080678 UE In-band Emission Requirements, Qualcomm Europe, RAN4#46-bis.