3GPP TSG-RAN4 Meeting #112	R4-2412086

# Maastricht, The Netherlands, Aug 19 – 23, 2024

Agenda item:			8.1.1.2.2

Source:	vivo

Title:	Discussion on MPR applicability for FR1 intra-band UL CA

Document for:	Discussion

- Introduction

In RAN#111 meeting, the WF[1] has been agreed and the details about MPR applicability for FR1 intra-band UL CA are as follows:

2.1. MPR applicability for FR1 intra-band contiguous UL CA Way forward: - For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated. - FFS: MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. - FFS: MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. - FFS: MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. FFS whether spurious emissions/ACLR/SEM are kept for aggregated CBW for CA 2.2. MPR applicability for FR1 intra-band UL non-contiguous CA Way forward: Enhancements not aligned with the WID will not be considered. Further clarify the scope for NC CA in next meeting based on the WID objectives.

- Discussion: MPR applicability for FR1 intra-band UL CA

As it has been agreed that for PC3 and PC2 contiguous UL CA the corresponding single CC MPR would be used when only 1 CC is activated. Based on the premise, the BW basis of the out-of-band emission requirements is still controversial, it is recommended that the evaluation bandwidth of ACLR\SEM\SE should be based on the BW of the activated CC rather than the aggregated BW.

The reasons are as follows: The MPR reduction from CA to single carrier means the transmission power is increased. On this promise if the evaluation bandwidth of EVM and ACLR still keeps as wide as aggregated BW including multiple CCs activated in CA, these requirements are relaxed compared to single-carrier. Then the MPR would be further reduced to a lower level compared to single-carrier, and the transmit power could be much higher and the emission is more likely to cause interference with the adjacent UEs. At the same time, it is kind of similar as the discussion on power domain enhancements for NR single carrier in [2], in which the applying range of ACLR\SEM shifts from the edge of UE CBW to a further frequency range to produce a wider guardband and the MPR is much lower. Based on the corresponding relaxation maybe a new MPR table or a new mapping relationship need to be simulated and defined.

Proposal: When only 1 CC is activated in CA, spurious emissions/ACLR/SEM should be based on the bandwidth of the activated carrier.

- Conclusion

Proposal: When only 1 CC is activated in CA, spurious emissions/ACLR/SEM should be based on the bandwidth of the activated carrier.

- References

- R4-2410749 WF on power domain enhancements, Huawei, HiSilicon, RAN4#111.

- R4-2412085 Discussion on power domain enhancements for NR single carrier, vivo, RAN4#112.