3GPP TSG-RAN4 Meeting #112	R4-2412087

# Maastricht, The Netherlands, Aug 19 – 23, 2024

Agenda item:			8.6.2

Source:	vivo

Title:	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

- Introduction

In RAN#104 meeting, the new SID[1] has been approved and the objective are as follows:

The objective of this study is as follows: - Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers. - Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

- Discussion

According to the SID, the methods for reducing the number of UE RX chains would be identified, one main direction is the change of the number of CCs supported by a RX chain, e.g., from separate RF chains per CC to shared RF chains. And there are some related issues to be discussed. First of all, what does “shared RX chain” really mean, as part of the current RX chain is already in the shared state.

Assuming one LO will be shared between the two non-contiguous CCs to down-convert the two CCs simultaneously, and the filter will be also shared between the aggregated CCs. Then the in-gap interference from adjacent channel operator would become a new problem and may interfere with the target aggregated carriers, as the in-gap interference between the non-contiguous CCs could not be filtered effectually. At the same time, the single LO could not be set in the middle of every CC but in the middle of the aggregated CCs. When the sub-blocks are asymmetrically allocated, the image of a high-power interference will also fall into another CC’s region, as illustrated in [2]. Therefore, the RX requirements and DL performance would be impacted a lot.

Fig 1. Contrast between separate architecture and shared architecture

Observation 1: The sharing of LO, filters and some other devices would introduce some new interference and degradation of DL performance compared to separate state.

Proposal 1: The “shared RX chain” should be clarified clearly that which part of the hardware resources would be switched to a shared state compared to the current technology.

“The ability to semi-statically switch hardware resources” means the state of RX chain and the number of RX chains per band of a CA combination could be dynamically determined according to the need and the environment. So it has certain requirements for the hard equipment, which may be stricter than the traditional version in order to accommodate functional changes. From [3], it could be seen that in the study of RX architecture in LTE period, one of the reasons for choosing the dual receiver architecture is that with single receiver architecture it is not possible to cover all possible channel arrangement as the technical constraints limit the possible receiver bandwidth to a finite number. And if operator owns frequency blocks that are separated more than Y MHz then single receiver UE’s can access only part of the carrier combinations and operator cannot deploy NC-intra-band CA in all markets.

As to the current technical development, the hardware equipment has been much more advanced than the LTE period. But considering the need of shared RX chain and the possibility of aggregating more CCs in the future, the demand for better-performing devices should still be higher than before. For example, the filter bandwidth of shared chains should be much wider than the separate chains to receive more non-contiguous CCs simultaneously. At the same time, maybe the design and production costs of the device will also be higher, which should be considered as well.

Observation 2: Considering the need of shared RX chain and the possibility of aggregating more CCs, the demand for better-performing devices should be higher than before, which should be considered as well.

In addition, whether RX chain sharing is only applied to a portion of non-contiguous subcarriers within a frequency band or to all non-contiguous subcarriers, the requirements for the device and the impact on RF needs are different:

1) If RX chain sharing is only applied to a portion of non-contiguous subcarriers within a frequency band, the device needs to support simpler signal processing algorithms. Additionally, the device may not need to support a too wide frequency range or high dynamic range. And the interference introduced by the in-gap channel would be less and simpler.

2) If RX chain sharing is applied to all non-contiguous subcarriers within a frequency band, the device needs to support a wider frequency range and higher dynamic range. And the in-gap interference would be more complex because they are more numerous and may originate from different operators. Additionally, the device may require more complex signal processing algorithms to handle a large number of subcarriers simultaneously. This approach may increase power consumption and cost, also reducing the sensitivity and selectivity of the receiver. However, it can also improve spectral efficiency and system capacity by processing more subcarriers simultaneously.

Therefore, the expansion of the bandwidth capability supported by shared RX chains needs to be approached with caution. Before the impact on DL performance is fully studied, at least at this stage the study of shared RX chains should be concentrated to the supporting for only 2 non-contiguous CCs instead of more CCs or even an entire frequency band.

Observation 3: Before fully studying the specific impact on DL performance, the level of RX chain sharing should not be expanded to the entire frequency band.

According to the analysis above, the power of the in-gap interference of the adjacent channel operator should be controlled under a certain level, or a relative low level compared to the two target non-contiguous CCs, otherwise the RX chain could be hardly shared between the two sub-blocks. There are two possible power control methods: 1) The in-gap signal would automatically have lower power, which could be achieved in certain cases, e.g., inter-operator co-located BS scenario. 2) The power of the in-gap signal is dynamic and needs to be continuously detected or indicated to seize the correct timing of the proper low level.

Observation 4: The power of the in-gap signal of the adjacent channel operator should be controlled to a certain level, or a relative low level compared to the two target non-contiguous CCs, otherwise the RX chain could not be shared between the two sub-blocks.

The reason of controlling the power imbalance between the two non-contiguous CCs is that: When the power of some carriers is significantly higher or lower than that of other carriers, this difference will directly affect the processing ability of the receiving signal. The relatively stronger power carrier may mask or interfere with the weaker power carrier, making it difficult for the latter to be correctly received and demodulated. But as for the in-gap signal which is kind of different, as it is the interference in the aggregated BW, so the power of the signal should be controlled as low as possible to confirm the correct receiving of the useful CCs. So the discussion of the “Determine a reasonable level of the power spectral density difference between carriers of co-located adjacent channel operators” could be simplified to “determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs”.

Proposal 2: The fourth sub-bullet in the SID should be simplified to “Determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs”.

So the analysis of power control would rely to the meaning of the inter-operator co-located BS scenario, however, the concept of which is not clear, and there are two kinds of understanding:

1) The power difference transmitted from different operators in the co-located BS is very small.

Assuming that BS will always transmit at their maximum power/PSD level within the allowed range, then the power difference transmitted from the target operator and the other one in the same band could be controlled to a relatively low level when they are co-located. Then the interference from the adjacent band of another operator would not be a big problem, so the fragmented carriers of the target operator can be utilized more fully, as some carriers can share part of the RX chains and free up corresponding resources for more carriers to use.

2) The power difference transmitted from different operators in the co-located BS is uncertain.

The power difference could not be controlled within different operators, which means the RX chain sharing between different CCs of the target operator is not always achievable, only when the power in the “gap” of the adjacent band belonging to the other operator is low enough then the sharing could be possible. So it should be relied on the UE to detect the power of the in-gap interference. And the judgement of the relative or absolute level of the power could be carried out by the UE or the NW.

Proposal 3: The inter-operator co-located scenario should be clarified: In the same band, whether the power difference transmitted from different operators in the co-located BS is very small.

Furthermore, as the final judgement information is the PSD difference, then the evaluation of in-gap interference is not only related to the power but also related to the bandwidth. If only the power difference could be controlled under a certain level in the inter-operator co-located BS scenario, then the sharing of the RX chains should be based on the premise that the BW of the in-gap signal should not be narrower than a certain range so that a relative low PSD level could be guaranteed.

Observation 5: If only the power difference transmitted from different operators could be guaranteed at a relatively small level, then the sharing of the RX chains should be based on the premise that the BW of the in-gap signal should not be narrower than a certain range.

With the RX chains switching to shared architecture, the RF requirements should be adjusted and some RF chains could be released, potentially allowing for the aggregation of more CCs. This would result in a wider variety of CA configurations supported by the UE. There are two kinds of methods for a UE to inform the network of new CA configurations it can support with adjusted RF requirements:

- The new CA configuration could be reported to the NW together with other default supported CA configuration, as they are related to the stable hardware capabilities. And it could be reported through a relative static mechanism, such as RRC signaling, as the current technology does. The RX chain state and CA configuration indication could be dynamically switched according to the environment.

- The new CA configuration could be separately reported to the NW when or after the RX chains switching to be shared, which is more dynamic but complex. When the RX chains could be switched to the shared architecture, the UE could report to the NW that the new CA configuration it supports with the sharing architecture, after which the NW could send a new CA configuration to indicate to the UE that there are more CCs to be aggregated.

Proposal 4: The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration.

- Conclusion

Observation 1: The sharing of LO, filters and some other devices would introduce some new interference and degradation of DL performance compared to separate state.

Proposal 1: The “shared RX chain” should be clarified clearly that which part of the hardware resources would be switched to a shared state compared to the current technology.

Observation 2: Considering the need of shared RX chain and the possibility of aggregating more CCs, the demand for better-performing devices should be higher than before, which should be considered as well.

Observation 3: Before fully studying the specific impact on DL performance, the level of RX chain sharing should not be expanded to the entire frequency band.

Observation 4: The power of the in-gap signal of the adjacent channel operator should be controlled to a certain level, or a relative low level compared to the two target non-contiguous CCs, otherwise the RX chain could not be shared between the two sub-blocks.

Proposal 2: The fourth sub-bullet in the SID should be simplified to “Determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs”.

Proposal 3: The inter-operator co-located scenario should be clarified: In the same band, whether the power difference transmitted from different operators in the co-located BS is very small.

Observation 5: If only the power difference transmitted from different operators could be guaranteed at a relatively small level, then the sharing of the RX chains should be based on the premise that the BW of the in-gap signal should not be narrower than a certain range.

Proposal 4: The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration.

- References

- RP-241669, Study on NR FR1 DL Fragmented Carriers, MediaTek Inc. RAN#104.

- R4-2412088, Discussion on impacts on UE RF requirements and DL performance, vivo, RAN#112.

- R4-120232, Open issues for UE receiver requirements for non-contiguous intra-band CA, Ericsson, ST-Ericsson, RAN #62.