3GPP TSG-RAN4 Meeting #112	R4-2412088

# Maastricht, The Netherlands, Aug 19 – 23, 2024

Agenda item:			8.6.3

Source:	vivo

Title:	Discussion on impacts on UE RF requirements and DL performance

Document for:	Discussion

- Introduction

In RAN#104 meeting, the new SID[1] has been approved and the objective are as follows:

The objective of this study is as follows: - Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering: - Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4]; - Other requirements are not precluded, if identified - The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2]; - Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4]; - Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4] - Impacts on DL performance [RAN4] ; - Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2]. NOTE 1: No RAN1 impact is foreseen NOTE 2: RAN2 work, if necessary, will be triggered by RAN4 LS NOTE 3: This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers. Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

- Discussion

According to the SID, the methods for reducing the number of UE RX chains would be identified, one main direction is the change of the number of CCs supported by a RX chain, e.g., from separate RF chains per CC to shared RF chains. And there are some related issues to be discussed. In this configuration, the impacts on UE RF requirements and DL performance would be studied.

ACS and in-band blocking

First of all, assuming the “shared RX chain” in the objective means in one RX chain there would not be dedicated filter for each CC, which not only presents a great challenge to the bandwidth and filtering effect of the shared filter, but also introduces some interference signal of the adjacent channel operators as the interference is allocated in the in-gap position of the two aggregated CCs and also within the filter passband. Also, a strong in-gap interfering signal may cause unnecessary power consumption and circuit saturation, it may have severe impacts into the dynamic range of the UE as well.

As mentioned in [2], currently the ACS and in-band blocking requirements are developed based on separate RF chain assumption, the in-gap requirements are the same as out-of-gap requirements, and they are too stringent for the in-band cases. So if the power of the in-gap interference could not be controlled to a relative low level contiguously, then the two RF requirements should be relaxed.

Observation 1: According to the interference of the in-gap signal, ACS and in-band blocking should be relaxed.

Image rejection of the in-gap interference

In the analysis of [3], due to imbalanced LO, two adjacent RF carriers would overlap causing the “image” problem after down-conversion, and this cannot be removed by any filtering scheme, since both carriers contain useful information. Therefore, the image rejection (IRR) is given as the ratio between the squared amplitudes of the original component and the image signal.

In the RX sharing scenario, a similar but not identical problem arises. When the LO is shared between two non-contiguous CCs to down-convert the two CCs simultaneously, the single LO could not be set in the middle of every CC but in the middle of the aggregated CCs instead. The strong interference from the in-gap signal would also cause another important problem: the image issue. As shown in Fig 1, when a strong interfering signal is located inside the sub-block gap, and the CCs are allocated asymmetric, considering the shared LO is not allocated in the middle of in-gap interference then the image of the interference may fall into one of the CC. Especially when the interference power is significant, the power of the image can also be high enough to disrupt the effective reception of the CC, and there is no related requirement of a conventional UE receiver to guarantee reasonable UE performance. A new requirement should be set to assess the interference suppression of the image signal, and a relatively reasonable level should be considered.

Fig 1. Contrast between separate architecture and shared architecture

Proposal 1: A new requirement should be set to assess the interference suppression of the image of the in-gap signal, and a relatively reasonable level should be considered.

REFSENSE

Sometimes an UL CC may cause stronger interference than an in-band signal, especially considering a relative weak duplexer isolation and a very narrow frequency separation between the UL CC and the DL CC, as analyzed in [4]. In some cases the frequency separation between an UL CC and a DL CC could be as narrow as the gap bandwidth between the transmit band and the receive band. Therefore, the impact of UL CCs on the UE receiver performance should be considered carefully. For example, the relaxation of REFSENSE should be further studied in this situation, e.g., ΔRIBNC. When the RX chain is switched to a shared state, the filtering effect is relatively reduced compared to the separated state, so the DL CC closer to the UL side is more severely affected and may not easily meet the current REFSENSE.

Observation 2: The ΔRIBNC should be relaxed in the shared RX chain structure especially when the frequency separation between the UL CC and the DL CC is too narrow.

- Conclusion

Observation 1: According to the interference of the in-gap signal, ACS and in-band blocking should be relaxed.

Proposal 1: A new requirement should be set to assess the interference suppression of the image of the in-gap signal, and a relatively reasonable level should be considered.

Observation 2: The ΔRIBNC should be relaxed in the shared RX chain structure especially when the frequency separation between the UL CC and the DL CC is too narrow.

- References

- RP-241669, Study on NR FR1 DL Fragmented Carriers, MediaTek Inc. RAN#104.

- RP-241257, Views on SI of Fragmented Carriers in Rel-19, Huawei, HiSilicon. RAN#104.

- R4-103677, Image Rejection in intraband carrier aggregation, Nokia, Nokia Siemens Networks, RAN4# AH#4.

- R4-115584, Impact of non-contiguous intra-band CA on the UE receiver requirements, Ericsson, ST-Ericsson, RAN#61.