3GPP TSG-RAN WG4 Meeting # 112	R4-2412089

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	On 4Tx power degradation for SRS antenna switching

Agenda Item:	5.9.1

Document for:	Approval

Introduction

In RAN4#110, an issue has been raised for ΔPPowerClass for 4Tx for SRS antenna switching in [1], for this parameter has not been considered as what has been specified for 2Tx. In RAN4#110bis, a set of tentative solutions were raised in [2] with an accompanying draft CR in [3]. However, that scheme was deemed too complicated and not agreed, while a WF has been agreed in [4]. In RAN4#111, several papers were submitted in [5][6][7][8] on this topic, but still no agreements could be reached.

In this paper, the history of the topic was re-summarized, but no new proposal were raised.

Discussion

In [5], it is illustrated that the root course of the current problem is that 4Tx has many variations of architectures, thus can not be bounded by ΔPPowerClass to reflect the restriction of architecture implementation like in 2Tx TxD. Even for 4Tx TxD we do not have an assumption for the architecture, let alone other 4Tx cases.

In [4], the following WF including a very brief pros and cons analysis was agreed and also copied as following:

Proposal: Companies are encouraged to consider the following options for ΔPPowerClass for SRS antenna switching for 4Tx.

Option 1: Combine the ΔPPowerClass and ΔTRxSRS to achieve the needed backoff with some flexibility for 4Tx.

- Details and related issues according to [1] and [2]

- Pros: Leaving the flexibility for UE implementation, while do not restrict the UL power;

- Cons: Complicated; Network not aware of power class used for sounding SRS; It allows unnecessary relaxation for certain architectures.

Option 2: Using fixed ΔPPowerClass for 4Tx based on the minimum capability architecture, i.e. 4*23 dBm.

- Define 3dB for t2ry, 6dB for t1ry, for all cases without considering other architectures.

- Pros: Simple and power achievable irrespective of architectures. Network aware of power class used for sounding SRS.

- Cons: Unnecessarily restrict output power for architectures with at least a 26dBm RF chain.

Option 3: Keep it as it is and do not define new requirements for 4Tx. (Default)

- Current wording already provide a 3dB value for PC1.5 which would imply a 26dBm RF chain should be used for 4Tx SRS antenna switching, and might be problematic for 4*23dBm architecture.

- No dedicated conformance testing for this part.

Option 4: Other solutions or variants of previous solutions not precluded.

Among those options, Option1 was the most complicated one, and was later not further pursued by original proponent.  In RAN4#111, the four proposals were summarized in [9] as following:

Issue 1-1-1: How to solve the 4Tx ΔPPowerClass issue?

- Proposals

  - Proposal 1: Do not define new ΔPPowerClass requirements for 4Tx to restrict the architectures or power in SRS antenna switching, and to refine the wording of current requirements to insure only appliable for 2Tx. (vivo)

  - Proposal 2: For 4 Tx architectures defined in Rel-18 use a 26 dBm PA to perform SRS transmission. If for a given architecture only a 23 dBm PA is available for SRS transmission, then limit SRS antenna switching to t4ry. (Qualcomm)

  - Proposal 3: (Huawei)

    - Proposal 3-1: Do not make ΔTRxSRS include PA ability, i.e., decouple ΔPPowerClass and ΔTRxSRS

    - Proposal 3-2: Do not lower PCMAX_H,f,c depending on the number of SRS ports in each SRS transmission occasion due to variation of PA configurations

    - Proposal 3-3: Apply ΔPPowerClass to only PCMAX_L,f,c as exception in case of SRS antenna switching as follows.

    - Proposal 3-4: Remove following texts and similar from the specification if Proposal 3 is agreed.

  - Proposal 4: (Ericsson):

    - Proposal 1: To address the power limitation issue for SRS transmissions for AS for UEs equipped with 4 Tx chains, avoid combining ΔPPowerClass and ΔTRxSRS requirements since those two have a different nature and introduce uncertainty at the gNB and thus degrade the channel estimation accuracy.

    - Proposal 2: We propose that different values should be applicable for ΔPPowerClass as a function of the indicated AS capability:

      - For ‘t4ry’ AS capability, ΔPPowerClass = 0 dB.

      - For ‘t2ry’ and similar AS capabilities, ΔPPowerClass = 3 dB.

      - For ‘t1ry’ and similar AS capabilities, ΔPPowerClass = 6 dB.

    - Proposal 3: Let the UE report the appropriate ΔPPowerClass depending on its PA configuration. That would allow reporting a different ΔPPowerClass for different SRS ports.

  - Proposal 5: Others.

Among those proposals, the implication of the corresponding brief pros and cons analysis is provided:

Option 1: Similar to “do nothing”. (Proposal 1)

Pros: No need for further discuss anymore.

Cons: Basically a 3dB fixed relaxation for 4Tx case, while not quite reasonable or aligned to certain architecture such as 2*23dBm;

Option 2: Restrict certain implementation.(Proposal 2)

Pros: Simple and bring much simplicity to requirements;

Cons: Restrictions to UE implementations

Option 3: Decouple ΔPPowerClass and ΔTRxSRS; (Proposal 3)

Pros: Flexible for implementation, while keep the requirements still simple

Cons: Not quite consistent with 2Tx;

Option 4: Fixed ΔPPowerClass based on SRS antenna switching configurations(Proposal 4:)

Pros: More consistent with 2Tx and strict requirements for the power

Cons: More stringent requirements for power;

It could be seen that no proposals currently is perfect. Currently we have slightly preference for option 3, while also can consider other proposals.

Proposal: Discuss the previous proposals and options, based on the pros and cons analysis, if certain option can be agreed.

Conclusion

In this paper, the history of the topic were re-summarized, but no new proposals were raised.

Proposal: Discuss the previous proposals and options, based on the pros and cons analysis, if certain option can be agreed.

- References

[1] R4-2400341, Delta PpowerClsss for 4Tx for SRS antenna switching, Nokia, RAN4#110

[2] R4-2404658, 4Tx power degradation for SRS antenna switching, vivo, RAN4#110bis

[3] R4-2404659, Draft CR for 4Tx power degradation for SRS antenna switching, vivo, RAN4#110bis

[4] R4-2406590, WF on 4Tx SRS issues, vivo, RAN4#110bis

[5] R4-2408113, Further discussion on 4Tx power degradation for SRS antenna switching, vivo, RAN4#111

[6] R4-2407583, 4 Tx DeltaP_PowerClass for SRS antenna switching, Qualcomm, RAN4#111

[7] R4-2407682, Delta Ppowerclass and Delta TRxSRS for 4Tx for SRS antenna switching, Huawei, HiSilicon, RAN4#111

[8] R4-2409772, On DeltaP_PowerClass for SRS antenna switching for 4Tx, Ericsson, RAN4#111

[9] R4-2408928, Topic summary for [111][117] FR1_enh2_R18, Moderator (vivo), RAN4#111