3GPP TSG-RAN WG4 Meeting # 112		R4-2412092

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.1.1.1.2

Source:	vivo

Title:	Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx

Document for:	Approval

- Introduction

In RAN#110bis, a WF[1] has been agreed and the details about 2-band inter-band UL NR-CA/EN-DC with 2Tx and/or 3Tx were included. In RAN4#111, a further WF is agreed in [2]:

1.1 Duty cycle solution for SAR compliance

Online agreement: Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC

3.1 Requirements between handheld UE and FWA

  - For PC2 and PC1.5 2-band inter-band UL CA with 2Tx or 3Tx, only define one set of UE RF requirements for both handheld UE and FWA

  - For PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx, only define one set of UE RF requirements for both handheld UE and FWA.

3.2 Whether to collect configurations to derive general requirements?

  - General requirements can be specified including considerations of different duplex mode configurations:

  - For PC2, FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx

  - For PC1.5, [FDD+FDD,] FDD+TDD, TDD+TDD with 2Tx and 3Tx

3.3 3UL CC with 3Tx

RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19.

3.4 Release independence

Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI

  - For FWA, only discuss the configurations that were not introduced in Rel-18

In addition, how to derive MSD requirements have been extensively discussed as documented in the summary [3]. There were a lot of discussions, though no contents have been written in the WF yet.

In this paper, those issues that is related to HPUE for UL inter-band UL CA or DC with 2Tx or 3Tx were discussed.

- Discussion

  - Configurations for general requirements

Actually, apart from SAR solution part and MSD, there are not too much duplex mode related requirements. If those issues have dedicated discussion, this issue may not be necessary to have sperate specific discussion.

Observation 1: Dedicated discussion on configurations for general requirements may not be necessary if SAR solution and MSD are already considered.

  - 3UL CC with 3Tx

In the WF [5], there is the following issue:

“RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19.”

It is quite obvious that this 3UL CC with 3Tx is quite different from what have been studied and specified in Rel-18, in which only inter-band CA+ULMIMIO/TxD considered. The implementation and requirements would be different, since UL-MIMO/TxD would make sure 2Tx would operate in the same frequency, thus bring simplicity in interference etc. 3UL CC with 3Tx would bring considerably more complex issues at least in interference, and also maybe other implementation factors.

In Rel-19, the current scope are extends from Rel-18 to higher power and handheld type, while the basic structure and scenario of inter-band UL-MIMO/TxD has kept. This kind of proposal for new scenarios belong to RAN level discussion and is an up scoping.

Proposal 1: Do not discuss 3UL CC with 3Tx since it is out of WI scope. Any up scoping would belong to RAN level discussion.

  - Duty cycle solution

Scenarios

As the MOP keep increasing with power class, not just SAR compliance will be an issue, the heat dissipation of the device and the stability of RF components in the high-power mode will also be a challenge. If there is no duty cycle limit, the device may keep working in the high-power mode for 100% duty cycle.

As discussed in [4], PC1.5 of inter-band CA, PC1.5 of inter band EN-DC, PC2 of Inter-band EN-DC are proposed to compatible with the legacy duty cycle solution.

PC1.5 of inter-band CA

The duty cycle solution for PC2 of inter-band CA is optional and also specified for 3Tx. When maxUplinkDutyCycle-interBandCA-PC2 is absent, there is no default threshold. The duty cycle solution is only applied when maxUplinkDutyCycle-interBandCA-PC2 is reported. If the capacity maxUplinkDutyCycle-interBandCA-PC2 is reported the duty cycle exceeds it, the power reduction is expected.

The solution for PC1.5 of inter-band CA is proposed to follow the similar way and compatible with PC2. There is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 2a: To compatible with PC2 of inter-band CA, for PC1.5 of inter-band CA, there is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

PC1.5 of Inter-band EN-DC(TDD+TDD)

The duty cycle solution for PC2 of Inter-band EN-DC(TDD+TDD) is mandatory and the duty cycle restriction is default, when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, the default threshold is 30%. If the duty cycle exceeds the threshold, the power reduction is expected.

The solution for PC1.5 of Inter-band EN-DC(TDD+TDD) could be extended for the similar way and compatible with PC2, the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

Proposal 2b: To compatible with PC2 of inter-band EN-DC(TDD+TDD), for PC1.5 of Inter-band EN-DC(TDD+TDD) , the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

PC2 of inter-band EN-DC(FDD+FDD)

As PC2 of inter-band EN-DC(FDD+TDD) PC2 is already specified in 38.101-3, the scenario is FDD on LTE and TDD on NR.  On LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2. These two capabilities could be easily reused for FDD+FDD with editorial changes.

The solution for PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

Proposal 2c: For PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

  - MSD Issues

Currently there were a lot of discussion on how to consider MSD requirements and verification for higher power class BCs in case lower power class requirements are already exist.

Observation 2: There are some basic options for MSD derivation for HPUE BC:

- Re-Calculate the MSD requirements.

This will introduce new test points and requirements, the BC would be verified under High power. One set of way is proposed in [5] including the following proposals:

Proposal 1: If MSD of 2TX PC2 is higher than 10dB, MSD of 2TX PC1.5 (26+26 architecture) = MSD of 2TX PC2 + 3n, where n is the order of IMDn, n<=5.

Proposal 2: For the 23+29 3TX architecture of PC1.5, the MSD of IMD2 for 2TX PC1.5 could be reused.

Proposal 3: If the MSD of 2TX PC2 is higher than 10dB, for the intermodulation product of the 23+29 3TX architecture higher than the second order, MSD = 2TX PC2 MSD + 6|y| where y is the coefficient of the 29dBm band.

Proposal 4: If the IIPn of PA increases linearly with the gain and the MSD of PC2 is higher than 10dB, MSD of PC1.5 2TX(26+26) = MSD of PC2 2TX +3. MSD of PC1.5 2TX(23+29) = MSD of PC2 2TX + 6|y| -3(n-1), where y is the coefficient of the 29dBm band and n is the order of IMDn.

This is by far the most clean solution, and also consistent with some legacy cases. However, this would also means that new requirements need to be developed, which might be a tedious and high loading work with so many band combinations.

- Using High Power + Default power class requirements

This basically use high power, but re-use legacy requirements developed for default power class. This would save the effort of new requirements, but could be more stringent, since relaxations are generally should be assumed. This method just give up all the relaxations compared to previous method.

- Using Default Power + Default power class requirements

This would means High power of BC would not be tested, and UE would only verified in fall back mode. This would also involve no new requirements definition. However, could be a bit too loose, since power fall back mode should involve better MSD.

In all, there are no perfect solutions, and more discussion is needed.

- Conclusion

In this paper, the following views provided:

Observation 1: Dedicated discussion on configurations for general requirements may not be necessary if SAR solution and MSD are already considered.

Proposal 1: Do not discuss 3UL CC with 3Tx since it is out of WI scope. Any up scoping would belong to RAN level discussion.

Proposal 2a: To compatible with PC2 of inter-band CA, for PC1.5 of inter-band CA, there is no default threshold when maxUplinkDutyCycle-interBandCA-PC2 is absent, if maxUplinkDutyCycle-PC2-FR1 is reported and 0.5* maxUplinkDutyCycle-PC2-FR1 is exceeded, power reduction is expected.

Proposal 2b: To compatible with PC2 of inter-band EN-DC(TDD+TDD), for PC1.5 of Inter-band EN-DC(TDD+TDD) , the default threshold is 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent, if 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is exceeded, power reduction is expected.

Proposal 2c: For PC2 of inter-band EN-DC(FDD+FDD), on LTE side, there are two threshold 40% and 70% is hardcoded. On NR side, UE will report two thresholds: maxUplinkDutyCycle-FDD-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-FDD-EN-DC2.

Observation 2: There are some basic options for MSD derivation for HPUE BC:

- Re-Calculate the MSD requirements.

- Using High Power + Default power class requirements

- Using Default Power + Default power class requirements

- References

- R4-2406583, WF on HPUE UE for UL CA and EN-DC, Samsung, RAN4#110bis.

- R4-2410750, WF on higher power UE, Samsung, RAN4#111

- R4-2408936, Topic summary for [111][125] NR_ENDC_RF_Ph4_part2, Moderator (Samsung), RAN4#111

- R4-2408136, Further discussion on SAR solution of HPUE, vivo, RAN4#111

- R4-2408131, Discussion on UL inter-band UL CA or DC with 2Tx or 3Tx, vivo, RAN4#111