3GPP TSG-RAN WG4 Meeting # 112	R4-2412093

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion on increasing UE transmission high power limit for CA HPUE

Agenda Item:	8.1.1.1.3

Document for:	Approval

Introduction

In RAN#103, a new WID [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC. Among the scopes there is the following:

  - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified

In RAN4#110bis, there were contributions discussing these issues documented in [2][3]. However, due to limited time, no discussion has been taken regarding this issue in this meeting yet.

In RAN4#111, significant progress were made and the following key agreements were documented in WF [6]:

Online agreement: Use the higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability as the starting point.

Adhoc agreement: The high-power limit feature (higherPowerLimit-R17/higherPowerLimtMRDC-R17) only applies with ΔPPowerClass,CA /ΔPPowerClass,EN-DC = 0 dB, which is aligned with the Rel-17 agreement

Way forward:

  - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific,

- For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es)

- No new duty-cycle solution/mechanism for SAR compliance is considered

- P-MPR can be used for SAR mitigation

However, there are still a lot of open issues, particularity the scenarios need to be considered, and some various other related features are still need to be discussed.

In this contribution, some issues were discussed on this topic.

Discussion

Scenarios

There are a lot of dispute on what power classes should be restricted, or whether there should be no power class restriction at all as some companies proposed. Admittedly, totally remove the power class restriction may seems to be simple, but at current stage, already two releases have passed and the current scheme works still well, we do not think that flexibility may really help much. Some sort of restriction would be still useful, since many combinations do not really make too much sense, as discussed before in Rel-18 stage.

Observation 1: There were already in Rel-18 discussions about the useful applicable power classes, and let increase power limit to apply to all power classes may not be needed.

As in [4][5], The following proposal is still kept here:

- Option 1: For 2Tx, if based on existing specified combos, there seems no new scenarios to be considered so far. For 3Tx, the following scenarios may could be considered. (Samsung)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.8 | One CC per band; For FWA only |

We think this is a good compromise between need and workload, and if other operator interests were raised, more scenarios can be considered.

Proposal 1: Consider PC3+PC2 and PC3+PC1.5 for 3Tx as scenarios for increasing power limit Rel-19.

UE Type consideration

There are several proposals on making some restriction on UE type for the case of increasing the total Tx power limit beyond PC1.5. We think this is worth to be study more, since total power beyond 29dBm may bring some difficulty for NR handheld UE .

Proposal 2: Consider to add some restrictions of UE type such as FWA only in case the total Tx power limit beyond PC1.5.

Relationship with Power Boosting

About power boosting, it should be noted that the latest agreements [8] in power boosting is, in case it is used in UL CA, it should be used with increase power limit and applied in only one of the band.

So currently we do not see a clear need to discuss power boosting anymore, since there is already latest agreement in the relation of boosting and increase power limit already. Any more discussion or enhancement on more power boosting such as on multiple bands at the same time would also clearly not supported in the current stage.

Proposal 3: No need to discuss relationship with power boosting since already have agreements.

Conclusion

In this paper, some considerations for increasing power limit were discussed, and the following observations and proposals were provided:

Observation 1: There were already in Rel-18 discussions about the useful applicable power classes, and let increase power limit to apply to all power classes may not be needed.

Proposal 1: Consider PC3+PC2 and PC3+PC1.5 for 3Tx as scenarios for increasing power limit Rel-19.

Proposal 2: Consider to add some restrictions of UE type such as FWA only in case the total Tx power limit beyond PC1.5.

Proposal 3: No need to discuss relationship with power boosting since already have agreements.

- References

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, RAN#103

[2] R4-2405282, Topic summary for [110bis][129] NR_ENDC_RF_Ph4, Moderator (Huawei, HiSilicon), RAN4 #110bis

[3] R4-2406583, WF on HPUE UE for UL CA and EN-DC, Samsung, RAN4 #110bis

[4] R4-2404986, Views on HPUE, Samsung, RAN4 #110bis

[5] R4-2408121, Discussion on increasing UE transmission high power limit for CA HPUE, vivo, RAN4#111;

[6] R4-2410750, WF on higher power UE, Samsung, RAN4#111;

[7] R4-2408936, Topic summary for [111][125] NR_ENDC_RF_Ph4_part2, Moderator (Samsung), RAN4#111

[8] R4-2410759	Draft CR to 38.101-1 for powerr boosting feature supporting CA, Ericsson, Qualcomm, Intel, Huawei, RAN4#111