3GPP TSG-RAN WG4 Meeting # 112	R4- 2412095

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Discussion of UE RF impact for NR MIMO phase 5

Agenda Item:	8.18.2

Document for:	Approval

Introduction

In RAN#102, a new WID [1] was approved on NR MIMO Phase 5. There were already three RAN1 meetings  and significant progress have been made, and this is the first meeting for RAN4 to discuss this.

This paper provides an initial impact analysis on the UE RF impact. RAN4 impact mainly comes from3Tx related scope. Detailed analysis is provided below.

Discussion

Overall Scope analysis

There are a total of five objectives within the scopes of this WI.

Beam reporting enhancement

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

  - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching

  - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

This scope is targeting to enhance the design of Beam reporting to improve efficiency.

Specifically, traditional mechanisms involve network-configured reporting (periodic or aperiodic), which can lead to inefficiencies. To address this, a new approach is introduced where the UE (User Equipment) initiates the reporting trigger, although the specific conditions are being discussed in RAN1 details are yet to be finalized.

This area has no impact on UE RF.

CSI-RS enhancement

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1

  - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks

  - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s)

  - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

This is for CSI-RS enhancement, to increase the ports number etc. The basic background is that there are still potentials to use large number of antennas and make finer beams.

This area has no impact on UE RF.

CJT reporting Enhancement

- Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD

- Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

This CJT (coherent joint transmission) reporting enhancement is for inter-TRP operation. RAN4 currently do not have any related verification for multi-TRP, and it is believed that this has no UE RF impact.

3Tx Codebook Transmission

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

This is an enhancement for UL-MIMO, where 3-antenna-port codebook-based transmissions is introduced.

In addition, full power mode is also considered. Mode-full power should be considered since it is not precluded.

This part clearly has RAN4 RF impact, and would be discussed later.

Asymmetric DL sTRP/UL mTRP

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

This scope involves a new scenario, that has uplink-only TRP. This uplink-only TRPs are subject to less stringent regulatory and deployment restrictions, or may allow for energy savings under specific conditions, thus could be useful in the field. However, this would introduce new challenges for closed-loop power control, necessitating the definition of parameters related to path loss offset and other relevant information.

After some analysis, it is felt that this scope would not change any UE RF requirements, or introduce new requirements. As said, RAN4 do not have multi-TRP related RF requirements.

Based on the previous analysis, the following proposal is provided:

Observation 1: Among the objectives within the WID, only 3Tx Codebook Transmission has clear RAN4 UE RF impact.

Proposal 1: Defining requirements for 3Tx Codebook Transmission.

RF Impact by 3Tx

General

Based on the general analysis in previous section, the RAN1 agreements were considered for 3Tx codebook transmission, and RAN4 UE RF impact would be studied in this section.

The main references could be [2][3] in which the agreements from previous 3 RAN1 meetings were included. As described in [3]:

Following the agreed description of the objective for 3TX UE, the focus of the discussion in Rel-19 NR MIMO is restricted to,

- design of non-coherent UL 3TX codebook,

- reuse of existing SRS resource definition and dimensions,

- exclusion of full power modes 1 and 2.

RAN1 has already completed those features. In this section, we investigate those agreed features and potential RAN4UE RF impact.

RAN1 agreements study

After study, it is found these RAN1 agreements that may have some direct RAN4 UE RF impact.

The non-coherent precoding codebook have been agreed as following:

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission.

13100,13010,13001

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission.

13100100, 13100001, 13 001001

For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission.

13100010001

Although “at least” have been used, actually no more codebook are defined, and this is currently already a complete set. It is noted that not necessarily  all the codebook would be used for verification.

For the power splitting behaviour, the following proposals were made:

For a 3-antenna-port codebook-based UL transmission, study power split for each port of SRS and PUSCH.

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource.

This basically means that the power splitting behaviour is aligned with current 38.213 defines. This has been discussed quite a lot before for this behaviour, and it should be noted that the power splitting behaviour is never been verified..

The full power mode 0 (Mode-full power) is formally agreed as following:

For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability.

It is noted that the full power mode is always verified with full power, non-full power configurations are not verified.

Regarding 3-port SRS transmission, according to RAN1 agreement the 4-port SRS resource would be reused, where the last port is muted.

For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource,

- Option 3: Always a same port is muted, e.g., the 4th port

This would have an impact to how to indicate the 3Tx operation, this would be explained more later in the document.

This indication in DCI might be used in some configurations, and also listed here:

To indicate precoding information for codebook-based UL transmission by a 3TX UE,

- Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated

- TPMI field is 2 or 3bits for 3-antenna-port transmission

  - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM

  - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM

For others, such as detailed SRS resource configurations, PTRS related and M-TRP related agreements, they generally do not have direct RAN4 UE RF impact. Thus, the detailed reference and explanations are omitted.

Observation 2: RAN1 agreements related to RAN4 RF are codebook definition, power splitting behavior, full power mode and DCI indication. Others such as SRS resource configurations, PTRS related and M-TRP related do not have direct impact.

RAN4 impact analysis

Based on the brief analysis above, the RAN4 UE RF impact is focus on MOP part.

Proposal 2: RAN4 impact for 3Tx are mainly for MOP for UL-MIMO.

Specifically speaking, for non full power mode, in order to verify the MOP, the 3 layer UL transmission with codebook 13100010001 is suggested to be used, and one tentative suggestion is made below:

Table 6.2D.1-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| Codebook based uplink | DCI format 0_1 | 4 | 02 |
| Codebook based uplink | DCI format 0_1 | 3 | 0TBD |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |

It should be noted that the configured “nrofSRS-Ports” would be still 4 for 3Tx case, since it has been already agreed that 4-port SRS resource would be reused. So, new mechanism should be used to distinguish with 4Tx case, and this will be discussed in RAN1 in upcoming meetings.

For Full power mode, at least layer = 1 for 3 ports should be verified. This corresponds to the codebooks: 13100,13010,13001

Table 6.2D.1-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
|  |  |  |  |  | 4 NOTE4 | 13 |
|  |  |  | CP-OFDM NOTE3 | 2 | 4 | 6 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
|  |  |  |  |  | 4 NOTE4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE2 |
|  |  |  | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
|  |  |  |  | 1 | 3 TBD | 0,1,2 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ul-FullPowerTransmission set to fullpowerMode1, all the transmitter requirement for CP-OFDM based modulation does not need to be verified if the requirements for 2-layer or 4-layer UL MIMO according to Table 6.2D.1-2 has been verified. NOTE 4: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |  |  |  |

Similarly, currently “nrofSRS-Ports” cannot be used to distinguish 3Tx and 4Tx, and new schemes would depend on future RAN1 discussion.

Proposal 3: Introduce MOP requirements based on following configuration:

Non-full power mode: 3 Layers, TPMI 0, 13100010001

Full power mode 0: 1 layer, TPMI 0,1,2, 13100,13010,13001

Proposal 4: Wait for further clarification by RAN1 on how to differentiate 3Tx and 4Tx in MOP configuration since legacy method of using “nrofSRS-Ports” cannot be used anymore.

Others

2Tx can be implemented for handheld. While for 4Tx, it is common understanding that this is a FWA feature that not applied to handheld UE. Now for the study of 3Tx single carrier, it might be worth to discuss whether there should be some restriction of UE type.

Admittedly, the 3Tx is already introduced in Rel-18 for inter-band CA+ULMIMO, and on going work in Rel-19 work assume to extend that to handheld UE. However, there are still new considerations in that currently full power mode 0 is also considered. This may somehow increase the RF chain implementation complexity in that all the RF chain should be full power.

Based on this situation, it is proposed:

Proposal 5: Discuss if some UE type restriction should be made on the UE type for 3Tx, especially for the case of full power mode 0.

Recently 3T6R is under discussion in RAN1 and may be introduced. However, since this was triggered in another RAN4 WI (NR_ENDC_RF_Ph4) and already update the corresponding WID, it is proposed not to be considered here.

Proposal 6: Do not consider RAN4 impact of 3T6R in MIMO Phase 5 WI since it already covered by another RAN4 WI.

In addition, possible up scoping is under discussion in RAN1 and might be introduced in upcoming RAN plenary. It is suggested to keep track on this in RAN4.

Proposal 7: Keep track on possible up scoping of WI in RAN.

Conclusion

In this paper, the initial UE RF impact of MIMO phase 5 is provided:

Observation 1: Among the objectives within the WID, only 3Tx Codebook Transmission has clear RAN4 UE RF impact.

Proposal 1: Defining requirements for 3Tx Codebook Transmission.

Observation 2: RAN1 agreements related to RAN4 RF are codebook definition, power splitting behavior, full power mode and DCI indication. Others such as SRS resource configurations, PTRS related and M-TRP related do not have direct impact.

Proposal 2: RAN4 impact for 3Tx are mainly for MOP for UL-MIMO.

Proposal 3: Introduce MOP requirements based on following configuration:

Non-full power mode: 3 Layers, TPMI 0, 13100010001

Full power mode 0: 1 layer, TPMI 0,1,2, 13100,13010,13001

Proposal 4: Wait for further clarification by RAN1 on how to differentiate 3Tx and 4Tx in MOP configuration since legacy method of using “nrofSRS-Ports” cannot be used anymore.

Proposal 5: Discuss if some UE type restriction should be made on the UE type for 3Tx, especially for the case of full power mode 0.

Proposal 6: Do not consider RAN4 impact of 3T6R in MIMO Phase 5 WI since it already covered by another RAN4 WI.

Proposal 7: Keep track on possible up scoping of WI in RAN.

- References

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102

[2] R1-2403852, FL Summary Support for 3TX CB-based Uplink; Second Round	Moderator (InterDigital, Inc.), RAN1 #117

[3] R4-2405879, Summary of Offline Discussions on 3TX CB-based Uplink, Moderator (InterDigital, Inc.), RAN4 #118

- Annex Initial Text Proposals

Table 6.2D.1-2: UL MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission scheme | DCI format | Number of layers | TPMI index |
| --- | --- | --- | --- |
| Codebook based uplink | DCI format 0_1 | 2 | 0 |
| Codebook based uplink | DCI format 0_1 | 4 | 02 |
| Codebook based uplink | DCI format 0_1 | 3 | 0TBD |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |

Table 6.2D.1-3: PUSCH Configuration for uplink full power transmission (ULFPTx)

| ULFPTx Mode | Transmission scheme | DCI format | Modulation | Number of layers | Number of Tx Port | TPMI index |
| --- | --- | --- | --- | --- | --- | --- |
| Mode-1 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM NOTE3 | 1 | 2 NOTE1 | 2 |
|  |  |  |  |  | 4 NOTE4 | 13 |
|  |  |  | CP-OFDM NOTE3 | 2 | 4 | 6 |
| Mode-2 | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0 or 1NOTE2 |
|  |  |  |  |  | 4 NOTE4 | 4, 5, 6 ,7 or 4, 5, 6 ,7, 8, 9, 10, 11NOTE2 |
|  |  |  | CP-OFDM | 2 | 4 | 1 or 0, 1, 2, 3, 4 ,5NOTE2 |
| Mode-full power | Codebook based uplink | DCI format 0_1 | DFT-s-OFDM, CP-OFDM | 1 | 2 NOTE1 | 0,1 |
|  |  |  |  | 1 | 3 TBD | 0,1,2 |
| NOTE 1: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 2. NOTE 2: TPMI index selected shall be based upon the full power TPMI reported by the UE [8, TS 38.213]. NOTE 3: For PUSCH configured with ul-FullPowerTransmission set to fullpowerMode1, all the transmitter requirement for CP-OFDM based modulation does not need to be verified if the requirements for 2-layer or 4-layer UL MIMO according to Table 6.2D.1-2 has been verified. NOTE 4: The UE is configured with one SRS resource with the parameter nrofSRS-Ports set to 4. |  |  |  |  |  |  |