3GPP TSG-RAN WG4 Meeting # 112	R4-242099

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements

Agenda Item:	8.8.2.2

Document for:	Approval

Introduction

In RAN#103, a new WID [1] was approved on NR MIMO Phase 5. In this WI, several areas of scopes were incorporated, including:

- High power UE (HPUE) for NTN

- Less than 5MHz for NTN

- NTN testing for NGSO

The scope of HPUE for NTN part is as following:

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this paper, an initial feasibility analysis for HPUE NTN, and a discussion of the impact of HPUE to Tx requirements for NTN are provided.

Feasibility Analysis

General

In the Scope, there are following scope considering the feasibility::

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

…<omitted part>…

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands …<omitted part>…

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

It can be seen that there are following scenarios and cases need to be considered:

Table 1. Clarification of different cases

| Classification | Options |
| --- | --- |
| NR or Iot | NR NTN, Iot-NTN (NB1/2, M1) |
| Power class | PC2, PC1.5(NR-NTN only), PC1 |
| Handheld or not | Handheld and non-handheld, non-handheld only |

We would like to discuss the feasibility based on these cases. In addition, it can be seen that power class 2 (PC2, 26dBm) does not have an explicit feasibility study scope. However, we still would like to have some discussion since it is the very first meeting.

Observation 1: Feasibility study cases can be classified by combination of NR/Iot, Power classes and handheld or not.

NR-NTN

Baseline consideration

One of the basic consideration of NR-NTN is that it will basically reuse existing components from NR as much as possible, since it would also need to work in TN network. So the feasibility of NR-NTN for a particular power class is pretty much dependent on the feasibility of NR for that power class.

In this spirit, for NR NTN, the analysis feasibility of power classes and handheld or not, should be similar to NR TN scenario, and all the existing study and state of the art technology for NR TN can be considered. Particularly, FDD should be considered since all the current NTN bands in FR1 are FDD bands.

Observation 2: The feasibility of NR NTN is similar to FDD NR TN case.

The feasibility of different power classes are considered in the following:

Power Class 2

A brief summary of the current power class specification condition for NR TN is as following:

Table 2. Current of specification condition for NR TN Power Class 2

|  | Specification history | Implementation assumption | Handheld or not |
| --- | --- | --- | --- |
| TDD | Specified from Rel-15; | 1Tx | Handheld and non-handheld |
| FDD | Feasibility conclude in Rel-17; Specified in Rel-18. New bands in Rel-19; | 2Tx and 1Tx; Migration to 1Tx anticipated | Handheld and non-handheld |

From this summary, it can be seen that power class 2 is already well defined for NR TN for some time. Even for FDD, the feasibility is already concluded in Rel-17, and more bands already specified for Rel-18. This has sufficiently proved the feasibility.

For handheld/non-handheld, no differentiations are made for this in the specification.

One possible issue is some of the PC2 FDD may based on 2Tx, which might be proved to be not suitable for further extension to higher power class. However, this does not impact that 1Tx implementation would be more and more mature since a number of mertis.

Proposal 1-1: Power class 2 is feasible for NR NTN UE, for both handheld and non-handheld.

Power Class 1.5

A brief summary of the current power class specification condition for NR TN is as following:

Table 3. Current of specification condition for NR TN Power Class 1.5

|  | Specification history | Implementation assumption | Handheld or not |
| --- | --- | --- | --- |
| TDD | Specified from Rel-15; | 2Tx (Typical) 4Tx | 2Tx: Handheld and non-handheld; 4Tx: non-handheld only |
| FDD | Not Specified |  |  |

PC1.5 was introduced very early for TDD. Although the original scenario is more FWA related, gradually it was some accepted that this is also applicable to handheld. Actually, there is no restriction on the specification of the handheld or not for PC1.5. For implementation assumptions, there is note: “NOTE 5:Achieved via dual Tx” in MOP part for PC1.5, indicating that 2Tx is assumed for PC1.5.

For FDD, currently there is still no PC1.5 definition, partly may because the high power 26dBm Tx is still not as mature as TDD. It can be seen that NR FDD PC2 has just start to be specified in Rel-18, and still based on combining two 23dBm Tx to achieve PC2. However, we have following observation:

Observation 3. The feasibility of NR FDD for PC1.5 is comparable to NR FDD PC2 1Tx implementation.

For the PA and duplexer, as long as components are available for NR FDD PC2 1Tx for a band, there would be no problem to combine them to a PC1.5. And this feasibility is already confirmed before in NR FDD PC2 study.

Admittedly, some restrictions may still exist for the component’s availability for certain bands, especially by the duplexer’s heat dissipation capability and isolation. However, this would not prevent the feasibility of this case as a whole.

Proposal 1-2: Power class 1.5 is feasible for NR NTN UE, for both handheld and non-handheld, though some restrictions may still exist for the component’s availability for FDD.

More study could be made on components on the market for this case.

Power Class 1

A brief summary of the current power class specification condition for NR TN is as following:

Table 4. Current Specification condition for NR TN Power Class 1

|  | Specification history | Implementation consideration | Handheld or not |
| --- | --- | --- | --- |
| TDD | Specified from Rel-18; |  | Non-handheld (Generally, PC1 UE is not targeted for smartphone form factor.) |
| FDD | Specified from Rel-16 (n14); More study in Rel-17 (37.880, 37.828); More bands in Rel-18; (37.829) |  | Non-handheld (Generally, PC1 UE is not targeted for smartphone form factor.) |

PC1 is firstly introduced as public safety services. Based on the study history (such as documented in 37.880/ 37.828/ 37.829), this is considered as non-handheld only. Currently the usefulness of this power class is still quite limited compared to other power classes. Some components, e.g. the duplexer may be significantly different from other power classes.

Based on this, the feasibility is still need more study on this.

Proposal 1-3: Power class 1 feasibility for NR NTN can be FFS.

In all the following proposal can be made for NR NTN feasibility in current stage.

Proposal 1: For NR-NTN, including handheld and non-handheld:

    - Power Class 2 is feasible;

    - Power Class 1.5 is feasible [with possible restrictions on components]

    - Power Class 1 is FFS

Iot-NTN

Baseline consideration

For Iot TN case including NB-Iot and MTC, currently there is no HPUE defined yet, mostly from considerations of cost/scenarios etc. So the feasibility of Iot-NTN HPUE would have to consider more other aspects compared to direct reference to TN counterpart.

In addition, since 1Tx basic assumption is assumed for Iot-NTN, PC 1.5 was not considered.

NB-Iot Based

NB-IoT operates only in half-duplex mode, eliminating the need for duplexers, which avoids various limitations associated with duplexers and reduces front-end insertion loss. Additionally, the narrow bandwidth of NB-IoT and its low peak-to-average power ratio. Those features are very apt to achieving higher transmission power.

Observation 4: It is easier for NB-Iot to implement higher power with its half-duplex and very narrow bandwidth.

For power class 2, since it is already well-developed, achieving this power level for NB-IoT should not pose technical challenges. The existing technology provides a solid foundation for NB-IoT, making it feasible to achieve high transmission power.

For power class 1, it is suggested to have some reference in some commercial status. The industry has already commercialized high-orbit satellite communication services, such as China's Tiantong-1 satellite system. This system using 3 GEO satellites, proprietary baseband and RF to provide low speed voice service. Currently deployed in band n256 with a working bandwidth of 22kHz, these parameters are quite similar to the current NB-IoT based NTN. There were already several commercial UE on the market already.

It should be noted the UE transmit power of this systems have already reached the PC1 level according to implantation and preliminary requirements.

To better facilitate comparison, we are currently validating the capabilities of relevant devices at higher power levels according to IoT NTN configurations.

Proposal 2: For NB-Iot based Iot-NTN, including handheld and non-handheld:

    - Power Class 2 is feasible;

    - Power Class 1 is likely to be feasible.

eMTC Based

eMTC is actually quite similar to NR, including the RF requirements. It support both half duplex and full duplex. From technical point of view, the feasibility of those may be comparable. More study can be done in later stage.

Observation 5: eMTC based Iot-NTN feasibility is largely comparable to NR-NTN.

Proposal 3: For eMTC based Iot-NTN:

    - Power Class 2 is likely to be feasible for handheld and non-handheld;

    - Power Class 1 feasibility is FFS;

In all, more concrete data are still needed. More investigation and verification are needed, particularly for the cases that are not conclusive.

HPUE Impact for Tx Requirements

Guidelines

In this paper, a high level analysis was performed. Not too many details were listed, and the main concern of this paper is what should be studied and how.

Generally there are three main references for a NTN HPUE Tx requirement, to make them clear the following table is provided:

Table 5 Main considerations for NTN HPUE Tx requirements

|  | Description and Analysis |
| --- | --- |
| Co-existence need | Mainly impact UE ACLR for HPUE. Basic guideline for ensuring system performance. MPR also impacted since Evaluation will depend on ACLR etc. |
| UE Implementation Feasibility | A restraint from UE side. Depending on state of the art technology. |
| Existing HPUE requirements | NR-NTN can check NR HPUE requirements with corresponding power class; Iot-NTN do not have corresponding HPUE yet in LTE; |

The study of the requirements are generally a study and balance among those three references. In many cases, it is a balance between those references.

Observation 6: The study of NTN HPUE Tx requirements generally would have to consider co-existence need, UE implementation feasibility, and existing HPUE requirements.

In addition, it should be noted that 2Tx would only be considered in NR NTN, not Iot NTN.

The detailed analysis would be done in individual requirements.

Impact analysis

MOP and SAR solution

The maximum outpower table and corresponding tolerances are not controversial. The nominal MOP corresponding power class (PC2/1.5/1) are already quite stable.

However, there might be still some details such as tolerances and relaxations still need to be discussed, e.g. if not too much existing reference available for Iot cases. Most probably many parameters can be reused from various existing specs, though special implementation for HPUE might also be considered.

Proposal 4: Reuse nominal MOP and study the related parameters such as tolerances.

SAR solution is one of major items usually considered in NR TN WIs. Generally, there basic methods are:

- In NR, SAR solution include P-MPR and duty cycle based；

- In LTE, SAR solution include P-MPR and UL/DL configuration based.

In Rel-17 and REl-18, this P-MPR is already introduced into NTN technology, and the current situation is as following table:

Table 6. Current supporting situation for P-MPR

|  | Spec status |
| --- | --- |
| NR NTN | Supported. (Reference from NR) |
| eMTC based Iot-NTN | Supported. (Reference from LTE) |
| NB-Iot based Iot-NTN | Not Support yet(NB-Iot not support) |

Though mostly re-use the NB-Iot requirements, it might be still useful to discuss whether it is useful for NB-Iot based Iot NTN to support P-MPR, considering the deployment scenario and power is different.

Proposal 5-1: Discuss if P-MPR is needed for NB-Iot based Iot-NTN.

Duty cycle solution is currently considered in NR TDD related scenario. It is defined mainly on the MOP section after the MOP table. The basic thinking is to limit the UL duty cycle to a certain degree to enable High power transmission, while mandatorily reduce the MOP in case certain threshold is exceeded.

For NR FDD HPUE in TN, it was agreed that duty cycle solution would not be considered, partly because there is no duty cycle concept for FDD, and not so apt to this solution.

Considering that NR NTN is currently all FDD, duty cycle based solution probably doesn't need to be considered either;

Proposal 5-2: For NR NTN HPUE, currently do not consider duty cycle based SAR solution.

UL/DL configurations are applicable to TDD LTE TN networks, and it is based on network UL/DL configurations to decide whether UE operation could use higher power or not, in that a lower UL ratio would benefit the higher power.

For MTC, NB-Iot, currently it is limited to FDD as well. so there is no UL/DL configuration exist, thus no need to consider it for the time being as well.

对MTC, NB-Iot，当前其实也是限于FDD，在LTE中UL/DL configuraiotn都是适用于TDD，所以也暂时不用考虑。

Proposal 5-3: For Iot-NTN HPUE, do not consider UL/DL configuration based SAR solution.

MPR

MPR is one of the most complicate and studied Tx requirements. The evaluation was usually by means of simulation plus measurement results verification. For 2Tx case, usually measurements would be considered rather than simulation. For HPUE, usually a new set of MPR would be developed, based on the implementation of high power components.

MPR requirements for default power class had already been specified in NR-NTN and Iot NTN UE specifications. As for the requirements for NTN HPUE, the following general considerations are provided:

NR-NTN: Based on ensure NTN UE operation also in TN network, the components for TN should be re-used as much as possible and this guideline already works for default power class which already been specified. For HPUE, this principle and consideration still works, that the TN HPUE should be an important consideration.

For power class 2 and also possibly 1.5, which feasibility generally ok, reusing the components and basic emission requirements are baseline. In this condition, re-using MPR is a natural choice. It should be also noted that though PC1.5 is currently not applicable to any FDD NR TN band, there are already MPR requirements defined for TDD NR TN PC1.5, and these 2Tx requirements can also be a baseline for future work.

For power class 1, the feasibility is still FFS for NR-NTN, the discussion of the requirements is also suggested to be LS.

The co-existence study also need to be taken into consideration, to make sure ACLR can be re-used if same MPR would like to be reused.

Based on these situations, the proposal is:

Proposal 6: Considering the reuse of components and feasibility, the MPR requirements for NR NTN are as following, provided that the corresponding ACLR be reused from TN and not violate co-existence :

- Power class 2: 	Reuse the current TN PC2 MPR requirements.

- Power class 1.5: 	Use the current TN PC1.5 MPR requirements as starting point

- Power class 1: 	FFS

For Iot-NTN, it should be noted the corresponding NB-Iot and eMTC in TN do not have power classes higher than PC3 specified, thus do not have a baseline MPR for higher power class, the MPR requirements can FFS.

Observation 7: Higher power class not defined for NB-Iot and eMTC yet, and there is no similar baseline MPR requirements for Iot NTN HPUE.

Proposal 7: MPR for Iot-NTN HPUE is FFS.

ACLR

In the analysis of ACLR, the co-existence study and feasibility should be considered. In the first stage, co-existence is the main reference for ACLR.

For NR NTN, the HPUE requirements already defined. Based on similar analysis as MPR, if corresponding TN requirements for the power class 2/1.5 also satisfy co-existence need, then the TN requirements should be the baseline.

PC1 has special challenges and uncertainty on the reuse of components, thus should be FFS. the following is proposed;

Proposal 8: Considering the reuse of components and feasibility, the ACLR requirements for NR NTN are as following, provided that the condition that co-existence can be satisfied

- Power class 2/1.5: 	Reuse the current TN PC2/1.5 ACLR requirements.

- Power class 1: 	FFS

For Iot NTN, there is no HPUE in NB-Iot or eMTC, thus no clear baseline for requirements. The study should joint consider co-existence, feasibility, and existing requirements for default power class.

One example could be: If co-existence requires a much lower ACLR compared to TN, then this means a large margin would be ready for HPUE ACLR, if implementation do need this margin.

Proposal 9: For Iot NTN, the ACLR for HPUE is FFS.

Others

Some discussion is needed for A-MPR on how to proceed. A-MPR would be impacted by higher power class, but the requirements would be band specific.

Observation 8: A-MPR may also be impacted by higher power class.

Other requirements are generally not impacted by HPUE.

Conclusion

In this paper, an initial feasibility analysis for HPUE NTN, and a discussion of the high level impact of HPUE to Tx requirements for NTN are provided.

Feasibility

Observation 1: Feasibility study cases can be classified by combination of NR/Iot, Power classes and handheld or not.

Observation 2: The feasibility of NR NTN is similar to FDD NR TN case.

Observation 3. The feasibility of NR FDD for PC1.5 is comparable to NR FDD PC2 1Tx implementation.

Proposal 1: For NR-NTN, including handheld and non-handheld:

    - Power Class 2 is feasible;

    - Power Class 1.5 is feasible [with possible restrictions on components]

    - Power Class 1 is FFS

Observation 4: It is easier for NB-Iot to implement higher power with its half-duplex and very narrow bandwidth.

Proposal 2: For NB-Iot based Iot-NTN, including handheld and non-handheld:

    - Power Class 2 is feasible;

    - Power Class 1 is likely to be feasible.

Observation 5: eMTC based Iot-NTN feasibility is largely comparable to NR-NTN.

Proposal 3: For eMTC based Iot-NTN:

    - Power Class 2 is likely to be feasible for handheld and non-handheld;

    - Power Class 1 feasibility is FFS;

HPUE Impact guidelines

Observation 6: The study of NTN HPUE Tx requirements generally would have to consider co-existence need, UE implementation feasibility, and existing HPUE requirements.

Tx Impact

Proposal 4: Reuse nominal MOP and study the related parameters such as tolerances.

Proposal 5: SAR solution:

    - Discuss if P-MPR is needed for NB-Iot based Iot-NTN.

    - For NR NTN HPUE, currently do not consider duty cycle based SAR solution.

    - For NR NTN HPUE, currently do not consider duty cycle based SAR solution.

Proposal 6: Considering the reuse of components and feasibility, the MPR requirements for NR NTN are as following, provided that the corresponding ACLR be reused from TN and not violate co-existence :

- Power class 2: 	Reuse the current TN PC2 MPR requirements.

- Power class 1.5: 	Use the current TN PC1.5 MPR requirements as starting point

- Power class 1: 	FFS

Observation 7: Higher power class not defined for NB-Iot and eMTC yet, and there is no similar baseline MPR requirements for Iot-NTN HPUE.

Proposal 7: MPR for Iot-NTN HPUE is FFS.

Proposal 8: Considering the reuse of components and feasibility, the ACLR requirements for NR NTN are as following, provided that the condition that co-existence can be satisfied

- Power class 2/1.5: 	Reuse the current TN PC2/1.5 ACLR requirements.

- Power class 1: 	FFS

Proposal 9: For Iot NTN, the ACLR for HPUE is FFS.

Observation 8: A-MPR may also be impacted by higher power class.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] TS 38.101-5

[3] TS 36.102

[4] TS 38.101-1

[5] TS 36.101