3GPP TSG-RAN WG4 Meeting # 112	R4-2412100

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Initial analysis for NTN HPUE RF Rx Requirements

Agenda Item:	8.8.2.3

Document for:	Approval

Introduction

In RAN#103, a new WID [1] was approved on NR MIMO Phase 5. In this WI, several areas of scopes were incorporated, including:

- High power UE (HPUE) for NTN

- Less than 5MHz for NTN

- NTN testing for NGSO

The scope of HPUE for NTN part is as following:

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this paper, a discussion of the impact of HPUE to Rx requirements for NTN are provided.

HPUE Impact for Rx

General

Firstly, the scope of this WI is for single carrier only, that means that the bunch of requirements which related to CA/DC would not be considered.

Observation 1: Scope does not include any requirements related to CA/EN-DC etc, such as MSD.

Similar to feasibility part, study can be classified by combination of NR/Iot, Power classes and handheld or not as the table below:

Table 1. Clarification of different cases

| Classification | Options |
| --- | --- |
| NR or Iot | NR NTN, Iot-NTN (NB1/2, M1) |
| Power class | PC2, PC1.5(NR-NTN only), PC1 |
| Handheld or not | Handheld and non-handheld, non-handheld only |

It is noted that though UE type was not considered frequently, actually handheld was usually considered in the configuration of 10dB Antenna isolation parameter.

Observation 2: The study can be classified by combination of NR/Iot, Power classes and handheld or not.

Based on legacy study in TN, almost all the Rx requirements impact by HPUE are sensitivity related. Specifically, currently only NR NTN currently have HPUE definition, and the newly defined requirement is Reference Sensitivity Degradation (RSD) compared default power class. This is because the receiver sensitivity may be affected by the leakage from its own transmitter, and higher power class can involve larger PA wideband noise and/or CIM5 if the duplex distance is too small or if the CBW is very large.

Observation 3: HPUE may involve larger impact to its own Rx and bring sensitivity degradation. In FDD NR TN HPUE, the concept and requirements of Reference Sensitivity Degradation (RSD) were introduced.

Based on this situation, we would continue to use Reference Sensitivity Degradation (RSD) for the NTN HPUE Rx requirements.

Proposal 1. Use Reference Sensitivity Degradation (RSD) for the NTN HPUE Rx requirements.

NR-NTN RSD

Background for FDD NR TN PC2 impact

When power class 2 is introduced in FDD NR TN HPUE, the following tables of Reference Sensitivity Degradation (RSD) were introduced:

In these two tables, the degradation of sensitivity compared to default power class were defined. The tables were related to Tx architecture, and classified by 1Tx/2Tx in which 2Tx have more interference and larger degradation.

The study and derivation of these requirements can reference to [3]. The basic idea is by accumulation the interference and one example is also shown below which is referenced from [4]. Compared earlier SI report in [2], those WI stage work is closer to the final requirements.

As commented in previous part, the UE type may impact the parameters here. In this example, the antenna isolation of 10dB is for handheld. Separate requirements for handheld and non-handheld in the Rx requirements are very rare.

Observation 4: A complete method exists for RSD derivation for FDD NR HPUE.

NR NTN Considerations

Generally, NR NTN is quite similar to NR, and the RSD definition and study method can use TN as reference.

Proposal 2: The study method of NR NTN Rx requirements can be similar to FDD HPUE Wis.

In addition, we can even try to reuse requirements based on similar cases.

For example, band n1 does not suffer from own Tx leakage, thanks to the relatively large duplex distance. The RSD values are all 0dB for n1 for power class 2. It can be seen that frequency separation of n1 is 190MHz.

By comparison, n256/n255/n254 also have large frequency separation as following:

The basic situations are:

- N256: separation same to n1, also in 2MHz area;

- N255: similar to n24, smaller gap but also lower channel;

- N254: very large separation

At least for n256 and n254, it can be safe to say that the RSD situation would either similar or better compared to n1, thus RSD =0 can be foreseen. N255 may need more study, but still optimistic.

Proposal 3: For power class 2, at least n256 and n254 can have 0dB RSD for NR-NTN.

For Power class 1.5, the uplink must be composed of two PC2 RF chains, and the interference profile for this case need more study.

Proposal 4: For power class 1.5, more study on RSD is needed for NR-NTN..

For even higher power classes, the feasibility is still problematic, and analysis can be postponed.

Proposal 5: Power class 1 is FFS for NR NTN.

Iot-NTN RSD

NB-Iot Based

Since NB-Iot is half-duplex, the Tx leakage would not impact Rx, so the analysis would be simpler. As in TDD systems, HPUE would not impact Refersens. So it is proposed:

Proposal 6: For NB-Iot based Iot-NTN, the sensitivity would not be impacted by HPUE.

MTC Based

MTC is more complex than NB-Iot in that it has both full duplex and half duplex. For half duplex, situation is similar to NB-Iot.

Proposal 7: For eMTC based Iot-NTN and half duplex, the sensitivity would not be impacted by HPUE.

For full duplex, it is more similar to NR, and the method can reference NR NTN to define RSD.In addition, MTC only consider 1Tx.

Proposal 8: For eMTC based Iot-NTN and full duplex, reference NR NTN method, but only consider 1TX.

Conclusion

In this paper, an initial feasibility analysis for HPUE NTN, and a discussion of the impact of HPUE to Tx requirements for NTN are provided.

Observation 1: Scope does not include any requirements related to CA/EN-DC etc, such as MSD.

Observation 2: The study can be classified by combination of NR/Iot, Power classes and handheld or not.

Observation 3: HPUE may involve larger impact to its own Rx and bring sensitivity degradation. In FDD NR TN HPUE, the concept and requirements of Reference Sensitivity Degradation (RSD) were introduced.

Observation 4: A complete method exists for RSD derivation for FDD NR HPUE.

Proposal 1. Use Reference Sensitivity Degradation (RSD) for the NTN HPUE Rx requirements.

Proposal 2: The study method of NR NTN Rx requirements can be similar to FDD HPUE Wis.

Proposal 3: For power class 2, at least n256 and n254 can have 0dB RSD for NR-NTN.

Proposal 4: For power class 1.5, more study on RSD is needed for NR-NTN..

Proposal 5: Power class 1 is FFS for NR NTN.

Proposal 6: For NB-Iot based Iot-NTN, the sensitivity would not be impacted by HPUE.

Proposal 7: For eMTC based Iot-NTN and half duplex, the sensitivity would not be impacted by HPUE.

Proposal 8: For eMTC based Iot-NTN and full duplex, reference NR NTN method, but only consider 1TX.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] TR 38.861, Study on high power UE (power class 2) for one NR FDD band

[3] TR 38.896, High power UE (power class 2) for NR FR1 FDD single band

[4] R4-2307239, n8 PC2 RSD for 25MHz DL CBW, Skyworks, RAN4#109