3GPP TSG-RAN WG4 Meeting # 112	R4-2412101

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	vivo

Title: 	Some remaining issues of Multi-carrier enhancements

Agenda Item:	5.20.1

Document for:	Approval

Introduction

Currently two issues were found:

- In RAN4#109, an LS [1] has been sent out to RAN2 to solve the issue of ambiguity in fall back operations, which the background could be reference to [2]. After that, RAN2 have updated 38.331 to add RRC configuration to consider this. In the last RAN4 meeting, RAN4 have conclude the feature list [3] to also considering this. However, we found that RAN2’s newly introduced RRC did not consider the capability defined in RAN4, thus greatly enlarged the applied scenario, and greatly increased the possibilities of contradiction.

- The current spec for ON/OFF mask has defined 3 band cases, but some 4 band cases are missing. An example could be some cases in [6] are not covered by any spec.

This paper discussed these two issues, and check what action should be taken.

Discussion

Fallback requirements

Background & current RAN4/2 solutions

In RAN4#108bis, an ambiguity problem is found in [2]:

After some discussion, an agreement has been reached in [1]:

Based on this spirit, RAN2 and RAN4 have developed some related schemes basically in parallel.

RAN4 solution and analysis

In the feature list [3] that RAN4 agreed, there is the following capability:

The basic thinking is UE to provide a capability, that absent means fallback value is applied, while not absent means 210us supported for all fallback BCs.

RAN2 solution and analysis

In RAN2, the following RRC configuration has been added:

UplinkTxSwitchingBandPairConfig-r18::=    SEQUENCE {

bandInfoUL1-r18                           UplinkTxSwitchingBandIndex-r18,

bandInfoUL2-r18                           UplinkTxSwitchingBandIndex-r18,

switchingOptionConfigForBandPair-r18      ENUMERATED {switchedUL, dualUL},

switching2T-Mode-r18                      ENUMERATED {enabled}                                             OPTIONAL,   -- Need S

switchingPeriodConfigForBandPair-r18      ENUMERATED {n35us, n140us}                                       OPTIONAL,   -- Need S

...

}

The description is as following:

UplinkTxSwitchingBandPairConfig field descriptions

bandInfoUL1, bandInfoUL2 Indicates the band index for a band pair. The value n indicates the band included at the n-th entry of uplinkTxSwitchingBandList.

switching2T-Mode Indicates 2Tx-2Tx switching mode is configured to the band pair. If this field is absent when uplink Tx switching is configured, it is interpreted that 1Tx-2Tx/1Tx-1Tx UL Tx switching is configured as specified in TS 38.214 [19].

switchingOptionConfigForBandPair Indicates the switching option for the band pair as specified in TS 38.214 [19], clause 6.1.6.

switchingPeriodConfigForBandPair Indicates the value of switching period for the band pair as specified in TS 38.214 [19], clause 6.1.6. Value n35us represents 35 us, n140us represents 140us. If the field is absent, 210 us is applied.

This means that RAN2 has added a new RRC configuration for a band pair, that has only 35/140us, and absent value means that 210us should be used.

Problems and Possible contradictions

Initial consideration was that the RRC configuration (switchingPeriodConfigForBandPair) should only work in ambiguous scenarios previously defined only for the fallback combinations, and should also be related to the terminal capability.

However, there is no such restriction in the current RAN2 scheme, which also may because the 38-6 feature came out later than RAN2 discussion. As a result, the newly defined configuration may completely overwrite the original ABCD band fallback switching period in any case.

Even when this RRC configuration does not exist, it implies that 210us is applied. This greatly extends the scope of use cases of this RRC configuration;

Observation 1: Currently new RRC signalling switchingPeriodConfigForBandPair did not consider new capability 38-6 and applied to more cases than fallback.

Take an example, in case there is no ambiguity and no fallback, in a combination ABCD, what should be applied for switching period should be based on UE capability switchingPeriodFor1T or switchingPeriodFor2T and the description is copied below from 38.306.

However, there is a danger that all those basic values would be overwritten by the new configuration.  Furthermore, for more cases such as unaffected band, RAN4 also has dedicated new capability for switching period, their applicability also may be impacted by the new RRC.

SwitchingPeriodUnaffectedBandDualUL-r18::=                SEQUENCE {

bandIndexUnaffected-r18                                   INTEGER(1..maxSimultaneousBands),

periodUnaffectedBandDualUL-r18                            CHOICE {

maintainedUL-Trans-r18                                    NULL,

periodOnULBands-r18                                       ENUMERATED {n35us, n140us, n210us}

}

}

This situation is apparently not that consistent with common understanding. And may making switching period even more ambiguous when fallback and the advance capabilities are provided.

Admittedly, there is an argument that redundancy and contradiction in configurations might be considered as a false configuration and thus not necessarily have to be ruled out explicitly. However, given the very wide impact, it seems that network has to constantly maintain this new IE to make it consistent with other capabilities is still a strange behavior, and may cause more trouble.

Observation 2: More problems for consistency and overwriting may happen in current scheme.

Based on this situation, we propose to discuss first in RAN4 the current situation, and see if some actions in RAN4 and/or feedback is needed for RAN2 via LS.

Proposal 1: Discuss this issue, and see if some actions in RAN4 and/or feedback is needed for RAN2 via LS.

4-band Mask

Currently in 38.101-1, although 3band and 4band requirements are defined in the same clause, actually for many cases, only 3 band case are defined specifically and 4 band cases are not. One example could be as in [6] as following:

This is a conclusion and the usual procedure is some how introduce into spec. However, this is the closest one in the spec:

It can be seen that this is 3 band case with clear X/Y/Z defined. There is no 4 band description and no general descriptions to extend the requirements to 4 band case.

Observation 3: Currently the requirements for some 4 band cases are not explicitly defined.

Based on this situation, the following proposal is provided:

Proposal 2: Discuss whether and how to cope with 4 band requirements.

Conclusion

In this paper, two identified issues are introduced, and propose to discuss solutions.

Observation 1: Currently new RRC signalling switchingPeriodConfigForBandPair did not consider new capability 38-6 and applied to more cases than fallback.

Observation 2: More problems for consistency and overwriting may happen in current scheme.

Proposal 1: Discuss this issue, and see if some actions in RAN4 and/or feedback is needed for RAN2 via LS.

Observation 3: Currently the requirements for some 4 band cases are not explicitly defined.

Proposal 2: Discuss whether and how to cope with 4 band requirements.

- References

[1] R4-2321986, LS on Rel-18 Tx switching enhancement, Huawei, RAN4#109

[2] R4-2317774, LS on Rel-18 Tx switching enhancement, Huawei, RAN4#10bis

[3] R4-2410747, Rel-18 RAN4 UE feature list for NR (version 5), CMCC, RAN4#111

[4] 38.331 V18.2.0;

[5] 38.101-1;

[6] R4-2220548, LS on Rel-18 UL Tx switching, RAN4#105