3GPP TSG-RAN WG4 Meeting # 112		R4-2412110

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:		8.18.3

Source:		Samsung

Title:	Discussion on RRM impacts on Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest WID [1] has been approved in the RAN plenary RAN#103 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In this contribution, we analyze the overall impacts to RRM requirements related to the above objectives.

In addition, from rapporteur perspective, the initial work plan for Rel-19 MIMO Phase 5 RRM core part is provided in [2].

- Discussion

Although RAN1/RAN2 process of R19 MIMO Phase 5 is still ongoing, we can start high level analysis of the potential impact to RRM requirements as well as other WGs progresses are ongoing.

2.1 UE-initiated/event-driven beam management

For objective#1 in WID, RAN1 is introducing a new procedure for beam management.

Before Rel-19, the legacy beam management procedure is based on CSI resource and reporting mechanism.

In this mechanism, the network configured UE for the CSI resources and report setting. In legacy RAN4 requirements, we defined such requirements like: L1-RSRP/L1-SINR, TCI state switching etc.

In Rel-19, RAN1 is working on specifying a UL-initiated reporting mechanism.

RAN1: Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources - Other procedure(s) as required

According to RAN1 agreements, UL-initiated reporting is based on trigger-event detection for beam reporting. The beam management procedure is different as legacy CSI based. The legacy RAN4 requirements for beam management cannot be used directly.

Proposal 1: RAN4 should specify RRM requirements for UE-initiated/event-driven beam management.

In RAN1 discussion, there are multiple types of trigger events. At least Event-2 is agreed in RAN1. RAN4 can start the discussion on Event-2 firstly. For other trigger events, wait further RAN1 progress.

RAN1: Agreements: Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

Proposal 2: RAN4 can start the discussion based on Event-2 or UE-initiated beam management. For other trigger events, need further RAN1 progress.

Observation 1: For Event-2, it can be observed that current beam should be measured and new beam(s) should be measured. L1-RSRP is used as quality for comparison.

RAN1: Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

In the WID, the scenarios are targeting FR2 and sTRP with intra-and inter-cell beam management.

Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management

Observation 2: For the scenario, the target is FR2 for sTRP with intra- and inter-cell beam management.

Observation 3: The measurement quantity on SSB for intra-cell and inter-cell. and CSI-RS.

RAN1: Agreement (updated as shown in red in Thursday session) On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the current beam for Event-2, down-select one or more of the following: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - FFS: Whether/how to specify the relationship between above RS measurement options for new and current beam(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable.

RAN1: Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Note: SSB or CSI-RS can be configured

RAN1: Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state. In last RAN1#117: Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded. Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list In last RAN1#117: Agreement Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

For the measurement RS for current beam, it is implicitly derived from QCL RS by indicated TCI state. In RAN1 conclusion, both SSB or CSI-RS can be configured.

For the measurement RS for new beams, the explicit configuration is supported. RAN1 has no agreements on how to configure.

Proposal 3: For the measurement RS:

    - Current beam, both SSB or CSI-RS can be configured. Derived from QCL RS by indicated TCI state. The details are still FFS.

    - New beams, the explicit configuration is supported, need further RAN1 progress.

Observation 4: However, in Rel-18 MIMO, only SSB based L1-RSRP is supported for inter-cell. There is no CSI-RS based L1-RSRP is supported.

If CSI-RS based measurement for inter-cell is used, there is no reference. The only similar measurement will be specified in Rel-19 mobility enhancement. However, the scenarios and procedure might be different. It is not clear at this first stage what will be difference between multiple WIs.

Proposal 4: RAN4 can start the discussion based on SSB firstly. Send LS to ask RAN1 confirmation for CSI-RS based measurement for inter-cell if necessary.

For the report format(s), RAN1 agreed:

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

According to RAN1 agreement, the beam report comprising N beams: N includes at least one new beam which is satisfies the event-2 condition. It can be configured whether current beam is reported or not.

Observation 5: For the beam reporting, the beam report can include the report for current beam or not depends on the configuration by RRC. The beam report should comprise N beams for new beams: N includes at least one new beam which satisfies the event-2 trigger condition.

Proposal 5: RAN4 can discuss UE-initiated/event-driven beam management requirements for above contents, use L1-RSRP as baseline, more RAN1 progress is needed.

2.2 CSI enhancement

In objective#2 and objective#3, CSI enhancement are related to four parts as shown:

| Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | RAN1 introduced: - Type-I single-panel codebook for 48, 64, 128 ports. - Type-I multi-panel codebook for 48, 64, 128 CSI-RS ports For codebook refinement, it will impact to UE performance requirements such as in Demod but no impact to RRM requirements. |
| --- | --- |
| Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | RAN1 introduced: Type-II codebook for 48, 64, and 128 ports For codebook refinement, it will impact to UE performance requirements such as in Demod but no impact to RRM requirements. |
| CRI-based CSI refinement for up to 128 CSI-RS ports | RAN1 introduced: CRI-based CSI for up to 128 ports For CRI-based CSI refinement, it will impact to UE performance but no impact to RRM requirements. |
| Aperiodic standalone CJT calibration reporting | RAN1 introduced: - CJTC delay offset reporting - CJTC frequency offset reporting - CJTC phase offset reporting |

Proposal 6: No impact to RRM requirements for objective#2: CSI support for up to 128 CSI-RS ports.

In Aperiodic standalone CJT calibration reporting, the scenario is multi-TRP CJT deployments. Before Rel-19, ideal synchronization and calibration are assumed for the coherent joint transmission (CJT). When the different TRPs are under non-ideal synchronization and backhaul, the issue is the timing delay between multi-TRPs can be larger. E.g. larger than CP. In addition, the multi-TRPs do not share the same system clock which will caused frequency/phase offset between multi-TRPs. The non-ideal characteristics will degrade the performance of CJT. Thus, RAN1 introduced the three types reporting for it which includes:

- CJTC delay offset reporting

- CJTC frequency offset reporting

- CJTC phase offset reporting

In last RAN1#117 meeting, RAN1 agreed:

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.75CP, 1.5CP, 14∆f, 112∆f,124∆f} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.025ppm, 0.05ppm, 1/(8t), 1/(16t), 1/(32t)} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), =1 only (agreed in RAN1#116bis) implies that the measured/reported phase offsets {n,, n=0, 1, …, NTRP – 1, n≠nref} are associated with the entire configured CSI reporting band (i.e. ‘wideband’).

For all three types of reporting, they are based on aperiodic reporting. No RRM core requirements.

Proposal 7: For objective#3, CJT calibration reporting of delay offset/frequency offset/phase offset, no RRM core requirements.

2.3 3Tx: 3-antenna-port codebook-based UL transmission

RAN1 has some agreements on 3TX, including:

- The precoders for single-layer, two-layer, three-layer

- Support 3 ports SRS transmission

- Codebook-based UL transmission such as PUSCH for 3Tx, PT-RS-DMRS association

- Etc.

From RRM perspective, the only potential RRM impact is for the requirements of Interruptions at NR SRS antenna port switching. In legacy RAN4 requirements for it, the common understanding is the components of the interruption time of SRS antenna port switching are: antenna switching time before and after SRS transmission; SRS transmission time of X symbols. And in the requirements, it is separated to 1 SRS symbol is configured or others (X=6 SRS symbols).

However, these components are not special for MIMO no matter whether to use codebook-based UL transmission or non-codebook-based UL transmission.

If 3TX is supported, the 3 ports SRS transmission will be specified in general.

In last RAN#104 meeting, in the discussion of another RAN4 WI (NR_ENDC_RF_Ph4), it is discussed whether SRS antenna switching for 3t6r is in the scope or not. In RAN#104 meeting, the conclusion is:

RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. Note 1: SRS antenna switching to support 4T6R is precluded. Note 2: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

- Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024)

By our understanding, as mentioned above, if 3t6r is supported, no additional enhancement for MIMO 3-antenna-port codebook-based transmissions, we can reuse the same requirement in 3t6r SRS antenna switching in NR_ENDC_RF_Ph4.

Proposal 8: If 3t6r is supported in NR_ENDC_RF_Ph4, it is supposed no additional enhancement for MIMO 3-antenna-port codebook-based transmissions, reuse the same requirement in 3t6r SRS antenna switching as the assumption if no further difference is observed.

2.4 Asymmetric DL sTRP/UL mTRP

In this objective, the enhancement is for the asymmetric DL sTRP/UL mTRP deployment scenarios.

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 a. Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

The sicario is like: UE can receive DL transmission and may transmit UL transmission from the TPR#1 while only transmit UL transmission from multiple UL TRPs (UL only TRPs for other N-TRPs.). In the both DL/UL capable TRP#1, it is a regular TRP. No further enhancement for pathloss RS is needed.

For other UL only TRPs, there is no DL transmission which means no DL RS including pathloss RS. The UE only received the PL-RS from TRP#1. And the real pathloss between UL only TRPs and the UE are different from the PL-RS measurement from TRP#1. That is why RAN1 introduce a PL-offset to reflect this difference.

RAN1: Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, support to associate a UL TCI state with a PL offset:

Observation 6: RAN1 introduced the associate a UL TCI state with a PL offset.

In RAN1#117 meeting,

To fulfil the asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP. - Reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured. Companies are encouraged to consider above for further discussion in RAN1#118.

In Rel-18, RAN1 introduced two TA mechanism for multi-DCI multi-TRP scenario. RAN4 define related timing requirements for two TA in multi-DCI multi-TRP.  By the conclusion in RAN1#117 meeting, the two TAs mechanism will be extended to single DCI multi-TRP scenario. If so, RAN4 need to update the timing requirements for two TA for single DCI multi-TRP. However, in last RAN#104 meeting, there were discussion whether it is in scope or not. The conclusion in RAN plenary is shown as:

Postpone discussions on the following topics until RAN#105. These topics will not be discussed in RAN1 before the decision in RAN#105.  Support of two TAs as part of enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios  SRS port grouping as part of CSI enhancement  Partial-coherent UL codebook for 3Tx multi-panel  Open loop based UL precoder cycling

Observation 7: RAN1 intended to extend two TA in single-DCI multi-TRP. The discussion is postponed until RAN#105. RAN4 define timing requirements for two TA in multi-DCI multi-TRP in Rel-18.

Proposal 9: If the extension to single-DCI multi-TRP in Rel-19, RAN4 should update the timing requirements to single-DCI multi-TRP. Wait further RAN/RAN1 conclusion.

Because the PL-RS cannot be derived directly from UL-TRP, the legacy UL TCI states switching delay or joint TCI states impacted by PL-RS measurement might be updated.

In latest RAN1 conclusion,

RAN1#117: Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Observation 8: For FR2, joint TCI state cannot be supported in asymmetric DL sTRP/UL mTRP scenario. DL-RS as source RS cannot be used. The UL states can only be associated to SRS in FR2.

Observation 9: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement.

Proposal 10: In asymmetric DL sTRP/UL mTRP scenario:

      - For FR1, FFS on UL TCI state switching delay requirement for unified TCI states.

      - For FR2, no RRM impacts on UL TCI state switching delay requirement for unified TCI states. Do not define such requirements if the UL TCI state is associated to SRS.

In addition, RAN1 is still discussing other procedure such as how to update PL-offset, PL-offset for PRACH and so on.

Proposal 11: RAN4 should wait further RAN1 progress to discuss whether other new RRM requirements are needed or not.

Proposal 12: In summary, the potential RRM impact is as below:

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | NO RRM core |
| 3TX | 3-antenna-port codebook-based UL transmission | NO, if no further special difference is identified |
| Asymmetric DL sTRP/UL mTRP | PL-offset FFS on two TA in s-DCI | [YES] |

- Conclusion

In this contribution, we provide our proposals for overall RRM impacts of Rel-19 MIMO Phase 5:

Proposal 1: RAN4 should specify RRM requirements for UE-initiated/event-driven beam management.

Proposal 2: RAN4 can start the discussion based on Event-2 for UE-initiated beam management. For other trigger events, need further RAN1 progress.

Observation 1: For Event-2, it can be observed that current beam should be measured and new beam(s) should be measured. L1-RSRP is used as quality for comparison.

Observation 2: For the scenario, the target is FR2 for sTRP with intra- and inter-cell beam management.

Observation 3: The measurement quantity on SSB for intra-cell and inter-cell. and CSI-RS.

Proposal 3: For the measurement RS:

    - Current beam, both SSB or CSI-RS can be configured. Derived from QCL RS by indicated TCI state. The details are still FFS.

    - New beams, the explicit configuration is supported, need further RAN1 progress.

Observation 4: However, in Rel-18 MIMO, only SSB based L1-RSRP is supported for inter-cell. There is no CSI-RS based L1-RSRP is supported.

Proposal 4: RAN4 can start the discussion based on SSB firstly. Send LS to ask RAN1 confirmation for CSI-RS based measurement for inter-cell if necessary.

Observation 5: For the beam reporting, the beam report can include the report for current beam or not depends on the configuration by RRC. The beam report should comprise N beams for new beams: N includes at least one new beam which satisfies the event-2 trigger condition.

Proposal 5: RAN4 can discuss UE-initiated/event-driven beam management requirements for above contents, use L1-RSRP as baseline, more RAN1 progress is needed.

Proposal 6: No impact to RRM requirements for objective#2: CSI support for up to 128 CSI-RS ports.

Proposal 7: For objective#3, CJT calibration reporting of delay offset/frequency offset/phase offset, no RRM core requirements.

Proposal 8: If 3t6r is supported in NR_ENDC_RF_Ph4, it is supposed no additional enhancement for MIMO 3-antenna-port codebook-based transmissions, reuse the same requirement in 3t6r SRS antenna switching as the assumption if no further difference is observed.

Observation 6: RAN1 introduced the associate a UL TCI state with a PL offset.

Observation 7: RAN1 intended to extend two TA in single-DCI multi-TRP. The discussion is postponed until RAN#105. RAN4 define timing requirements for two TA in multi-DCI multi-TRP in Rel-18.

Proposal 9: If the extension to single-DCI multi-TRP in Rel-19, RAN4 should update the timing requirements to single-DCI multi-TRP. Wait further RAN/RAN1 conclusion.

Observation 8: For FR2, joint TCI state cannot be supported in asymmetric DL sTRP/UL mTRP scenario. DL-RS as source RS cannot be used. The UL states can only be associated to SRS in FR2.

Observation 9: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement.

Proposal 10: In asymmetric DL sTRP/UL mTRP scenario:

      - For FR1, FFS on UL TCI state switching delay requirement for unified TCI states.

      - For FR2, no RRM impacts on UL TCI state switching delay requirement for unified TCI states. Do not define such requirements if the UL TCI state is associated to SRS.

Proposal 11: RAN4 should wait further RAN1 progress to discuss whether other new RRM requirements are needed or not.

Proposal 12: In summary, the potential RRM impact is as below:

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | NO RRM core |
| 3TX | 3-antenna-port codebook-based UL transmission | NO, if no further special difference is identified |
| Asymmetric DL sTRP/UL mTRP | PL-offset FFS on two TA in s-DCI | [YES] |

- Reference

[1] RP-240087, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2412134, Work plan for Rel-19 NR MIMO Phase 5, Samsung