3GPP TSG-RAN WG4 Meeting # 112		R4-2412111

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:		8.8.3.4

Source:		Samsung

Title:	Discussion on RRM impacts on Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

The revised WID [1] has been approved in the RAN plenary RAN#104 meeting. The objectives of Rel-19 “Enhanced for NR NTN and IoT NTN” are described in [1] including:

The objectives of core part for enhanced requirements and test methodology for NR and IoT NTN include: High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this contribution, we analyze the potential impacts to RRM requirements related to the above objectives.

- Discussion

Although there is no conclusion of system parameters for channel bandwidth of less than 5 MHz for NR-NTN, we can start the analysis on the potential RRM impacts based on WID.

In WID, the 3MHz designed in Rel-18 will be used in NTN FR1 band.

- Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18

In Rel-18 3GPP design of 3MHz, the number of transmission bandwidth is 15RBs which PBCH transmission bandwidth is 12PRBs.

Only SCS = 15kHz is supported in 3MHz bandwidth. SCS (kHz); 3 MHz; 5 MHz. NRB; NRB. 15; 15; 25. 30; N/A; 11. 60; N/A; N/A.

Observation 1: In Rel-18, in 3MHz bandwidth system, only 15kHz SCS is supported.

In RAN1 design of 3MHz, the PSS/SSS haven’t been impacted. However, the PBCH bandwidth is reduced to 12PRBs.

For cell search on a carrier with a channel bandwidth of 3 MHz, the UE is not expected to receive subcarriers 0 to 47 and 192 to 239 in any of the 4 OFDM symbols of the SS/PBCH block.

Observation 2: In Rel-18, in 3MHz bandwidth system, number of PBCH is punctured to 12 PRBs.

Proposal 1: RAN4 to discuss the RRM requirements based on Rel-18 3MHz design: SCS is only for 15kHz. PBC is punctured to 12 PRBs.

When SSB is punctured to 12PRBs, the performance related to PBCH DMRS detection/evaluation will be degraded. In Rel-18 TN less than 5MHz WI, there were analysis and consensus on which RRM requirements will be defined due to reduced 3MHz bandwidth. In TN 3MHz system, all LLS evaluation are based on the propagation condition considered 3km/h and 500km/h. In NTN system, the only difference is the propagation condition is: the speed of satellite such LEO is so fast (7.56km/s) which caused maximum doppler is different.

If we take the TN 3MHz and conclusion as baseline, we analyze the potential RRM impacts by introducing 3MHz in NTN system:

- RRC_Idle mode mobility and RRC_Inactive mode mobility

In Idle/Inactive mode, cell selection and cell re-selection are the main procedures. In NTN system, the enhanced cell selection and cell re-selection is time-based and/or location-based cell reselection. It is not related to PBCH. Therefore, we think there should be no impacts.

In EMR measurement, the SSB detection may have impacts because of SSB index reading. However, in NTN, it only supported SA mode. It hasn’t been supported for CA/DC scenarios. No impacts.

Proposal 2: No RRM impacts for RRM requirements in RRC_IDLE/RRC_INACTIVE modes by introducing 3MHz in FR1-NTN.

- Handover

The interruption time includes Tsearch or time for fine tracking depends on SSB index reading. RAN4 need to define new RRM requirements for 3MHz in FR1-NTN. In TN system, the evaluation is based on the propagation condition which considered 3km/h and 500km/h.

In addition, in NTN system, RACH-less handover has been introduced which depends on Tsearch or T∆ (time for fine time tracking and acquiring full timing information). When RAN4 define the new requirements for RACH-based handover, the same enhancement can be reused in RACH-less handover.

Proposal 3: RAN4 to evaluate and define new requirements for HO (RACH-based and RACH-less HO) to consider the update of Tsearch or T∆ (time for fine time tracking and acquiring full timing information) by introducing 3MHz in FR1-NTN.

Compared to TN system, the other difference is: in NTN system, the conditional handover is supported including:

- Time/location-based CHO with L3 measurement

- Time/location-based CHO without L3 measurement.

In time/location-based CHO with L3 measurement, the legacy CHO handover delay is as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

In those components, TRRC, TEvent_DU, TCHO_execution should be the same as legacy. Only Tmeasure or Tinterrupt including time for fine time tracking depends on PBCH detection such as L3 SSB index reading. However, in the requirements, they are referred to L3 measurements. If RAN4 define new L3 requirements, it can be used directly for Tmeasure. RAN4 to discuss the impacts on Tmeasure and T∆ (time for fine time tracking and acquiring full timing information) in Tinterrupt.

For time/location-based CHO without L3 measurements, the legacy CHO handover delay is as:

DCHO = TRRC + TEvent_DU + Tinterrupt + TCHO_execution

In those components, TRRC, TEvent_DU, TCHO_execution should be the same as legacy. Only Tinterrupt depends on Tsearch which has impacts.

Proposal 4: In 3MHz for FR1-NTN, for Time/location-based CHO with L3 measurement handover delay, the impacts by affected by punctured PBCH can be discussed for Tmeasure and T∆.

Proposal 5: In 3MHz for FR1-NTN, for Time/location-based CHO without L3 handover delay, the impacts by affected by punctured PBCH can be defined if Tsearch and T∆ is concluded to be updated in P3.

In addition, in NTN system, satellite switching with re-synchronization including hard switch and soft switch is specified. RAN4 should discuss the satellite switch time including Tmeasure and T∆ should be affected or not. It can also be discussed on whether the same or different principle can be used in handover procedure or not.

Proposal 6: In 3MHz for FR1-NTN, for satellite switching with re-synchronization including hard switch and soft switch, RAN4 should discuss the satellite switch time including Tmeasure and T∆ should be impacted or not. In addition, RAN4 should discuss whether to use the same update in handover or not if any.

- RRC Re-establishment/RRC connection Release with re-direction

In such delay requirements, if the SSB side condition is not changed, the delay depends on the time to identify the target cell such as cell search time and measurement time which there is no enhancement.

Proposal 7: In 3MHz for FR1-NTN, For RRC Re-establishment/RRC connection Release with re-direction, no RRM impacts.

- Timing.

Proposal 8: In 3MHz for FR1-NTN, for timing requirements, no RRM impacts.

- RLM/BFD/CBD

Due to the reduced number of PRBs in 3MHz, the hypothetical PDCCH transmission parameter should be defined for 3MHz in FR1-NTN. RAN4 can reuse the parameters in TN 3MHz for SSB-based RLM/BFD/CBD requirements.

However, in Rel-18 TN system, CSI-RS based RLM/BFD/CBD requirements haven’t been defined due to workload in Rel-18 although there is impact of the similar root of hypothetical PDCCH transmission parameter. SSB-related requirement can be prioritized. FFS on whether to define RLM/BFD/CBD based on CSI-RS if time allowed.

Proposal 9: For RLM/BFD/CBD requirements, in 3MHz for FR1-NTN:

- RAN4 to define the new requirements for SSB based. TN 3MHz can be used as baseline.

- SSB based RLM/BFD/CBD can be prioritized. FFS on whether to define RLM/BFD/CBD based on CSI-RS because the number of PRB is 48 which is larger than 3MHz.

- SCell/PSCell/Interruption requirements.

Proposal 10: For Scell/PSCell/Interruption, no RRM impacts because NTN only supports single carrier.

- Measurement procedure

The SSB based measurement should be impacted and RAN4 need to evaluate and define the intra-/inter-frequency measurement in 3MHz NTN system.

Proposal 11: RAN4 to evaluate and define SSB based measurement including intra-/inter frequency measurement in 3MHz FR1-NTN.

For CSI-RS based L3 measurements, in NTN system, it doesn’t support it.

Proposal 12: For CSI-RS based L3 measurement, in 3MHz for FR1-NTN, no RRM impacts.

- L1-RSRP

In SSB based L1-RSRP, no impacts because no PBCH-DMRS is related.

In legacy NTN CSI-RS RSRP, the bandwidth of CSI-RS is 48PRBs. In TN 3MHz system, it is deprioritized due to workload in Rel-18.

Proposal 13: For L1-RSRP measurement, in 3MHz for FR1-NTN:

- For SSB-based, no RRM impacts.

- For CSI-RS based, L1-RSRP based on CSI-RS has impacts by reducing bandwidth to 3MHz because the side condition of CSI-RS is 48PRBs which is larger than 3MHz. FFS on whether to define L1-RSRP based on CSI-RS if time allowed.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for less than 5MHz in NTN:

Observation 1: In Rel-18, in 3MHz bandwidth system, only 15kHz SCS is supported.

Observation 2: In Rel-18, in 3MHz bandwidth system, number of PBCH is punctured to 12 PRBs.

Proposal 1: RAN4 to discuss the RRM requirements based on Rel-18 3MHz design: SCS is only for 15kHz. PBC is punctured to 12 PRBs.

Proposal 2: No RRM impacts for RRM requirements in RRC_IDLE/RRC_INACTIVE modes by introducing 3MHz in FR1-NTN.

Proposal 3: RAN4 to evaluate and define new requirements for HO (RACH-based and RACH-less HO) to consider the update of Tsearch or T∆ (time for fine time tracking and acquiring full timing information) by introducing 3MHz in FR1-NTN.

Proposal 4: In 3MHz for FR1-NTN, for Time/location-based CHO with L3 measurement handover delay, the impacts by affected by punctured PBCH can be discussed for Tmeasure and T∆.

Proposal 5: In 3MHz for FR1-NTN, for Time/location-based CHO without L3 handover delay, the impacts by affected by punctured PBCH can be defined if Tsearch and T∆ is concluded to be updated in P3.

Proposal 6: In 3MHz for FR1-NTN, for satellite switching with re-synchronization including hard switch and soft switch, RAN4 should discuss the satellite switch time including Tmeasure and T∆ should be impacted or not. In addition, RAN4 should discuss whether to use the same update in handover or not if any.

Proposal 7: In 3MHz for FR1-NTN, For RRC Re-establishment/RRC connection Release with re-direction, no RRM impacts.

Proposal 8: In 3MHz for FR1-NTN, for timing requirements, no RRM impacts.

Proposal 9: For RLM/BFD/CBD requirements, in 3MHz for FR1-NTN:

- RAN4 to define the new requirements for SSB based. TN 3MHz can be used as baseline.

- SSB based RLM/BFD/CBD can be prioritized. FFS on whether to define RLM/BFD/CBD based on CSI-RS because the number of PRB is 48 which is larger than 3MHz.

Proposal 10: For Scell/PSCell/Interruption, no RRM impacts because NTN only supports single carrier.

Proposal 11: RAN4 to evaluate and define SSB based measurement including intra-/inter frequency measurement in 3MHz FR1-NTN.

Proposal 12: For CSI-RS based L3 measurement, in 3MHz for FR1-NTN, no RRM impacts.

Proposal 13: For L1-RSRP measurement, in 3MHz for FR1-NTN:

- For SSB-based, no RRM impacts.

- For CSI-RS based, L1-RSRP based on CSI-RS has impacts by reducing bandwidth to 3MHz because the side condition of CSI-RS is 48PRBs which is larger than 3MHz. FFS on whether to define L1-RSRP based on CSI-RS if time allowed.

- Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung