3GPP TSG-RAN WG4 Meeting # 112		R4-2412112

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:		8.25.4

Source:		Samsung

Title:	Discussion on RRM impacts on Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

The revised WID [1] has been approved in the RAN plenary RAN#104 meeting. The objectives of Rel-19 NTN Phase 3 are described in [1] including:

The work item aims at specifying further enhancements for NG-RAN based NTN (Non-Terrestrial Networks) with the following assumptions: - GSO (Geo Synchronous Orbit) and NGSO (Non-Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO). - Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO - FDD mode - UEs with GNSS (Global Navigation Satellite Systems) capabilities - In frequency band above 10 GHz, both Terminal Type 1 (Electronic steering antenna) and Type 2 (Mechanical steering antenna) to be considered for GSO and NGSO - Implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios, where relevant Note 1: In Rel-19 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion). The objectives of the work item are the following: - Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In this contribution, we analyze the potential impacts to RRM requirements related to the above objectives.

- Discussion

Although RAN1/RAN2 process of R19 NTN Phase 3 is still ongoing, we can start high level analysis of the potential impact to RRM requirements as well as other WGs progresses are ongoing.

2.1 Downlink coverage enhancements

For objective#1 in WID, RAN1 started the work for LLS and SLS evaluation for one satellite comprised many beams. Although there is no such agreement on how to define the power sharing among satellite beams or different satellite beam patterns/size across the satellite footprint, we can observe from:

RAN1#116 Agreement For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1: LEO600km Set1-1 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size(Note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 31.24. Aggregated EIRP (Total) (dBW); 61.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints***; 1058. Total number of simultaneously active beams **; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set1-2 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 34. Payload Total DL power level (dBW); 23. Aggregated EIRP (Total) (dBW); 53*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 41. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 16. % simultaneously active beams**; 1.5 %. *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. LEO600km Set 1-3 FR1 (i.e., S-band). Maximum Bandwidth per beam; 5 MHz. SCS; 15 kHz. Beam size (note 1); 50km. Satellite EIRP density /beam (dBW/MHz); 26. Payload Total DL power level (dBW); 23.24. Aggregated EIRP (Total) (dBW); 53.24*. Satellite Tx max Gain; 30 dBi. Maximum EIRP per Satellite beam (dBW); 33. Total number of beam footprints; 1058. Total number of simultaneously active beams**; 106. % simultaneously active beams**; 10.02 %. *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. Agreement For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band): LEO600km Set1-1 FR2 (i.e., Ka-band). Maximum Bandwidth per beam; 400 MHz. SCS; 120 kHz. Beam size; TBD in next meeting. Satellite EIRP density /beam (dBW/MHz). Payload Total DL power level (dBW). Aggregated EIRP (Total) (dBW). Satellite Tx max Gain. EIRP per Satellite beam (dBW). Total number of beam footprints; 800 (note 1). Total number of simultaneously active beams; 12. % simultaneously active beams; 1.5 %. Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF).

In RAN1 agreed parameters, one satellite has many (more than one thousand) beams totally which only some of them are active beams simultaneously. The number of active beams is much less than the total number of beams.

Observation 1: The number of active beams is much less than the total number of beams.

RAN1#116: For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access - N1 + N2 + N3 = “Total number of beam footprints “ - N1, N2, N3, X are to be reported by companies. - Resource utilization obtained under the assumptions above is to be reported by companies. - Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Observation 2: There is “off” state which means no any signal which means UE cannot receive DL signal including DL RS during “off” state.

Observation 3: The legacy RAN4 RRM requirements are mainly based on DL RS including SSB or CSI-RS. If there is no SSB or CSI-RS as the measurement RS, it cannot perform the measurements.

In RAN1 discussion, SSB periodicity might be extended to the values which are larger than 160ms. According to last RAN#104 conclusion shown as below:

- SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

SSB periodicity extension can be further discussed. If so, the SSB periodicity might be extended to the values which are larger than 160ms. Many RRM requirements are related to SSB and SSB periodicity.

Proposal 1: If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed.

Because RAN1 introduced “off” state for some beams, UE cannot receive DL-RS especially SSB both in idle/inactive mode and connected mode. Therefore, the measurements related to DL-RS should be impacts. RAN4 to discuss and define new RRM requirements. Need further progress from other WGs.

Proposal 2: All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs.

2.2 Uplink Capacity/Cell Throughput Enhancement for FR1-NTN

For objective#2 in WID, RAN1 support OCC for PUSCH in Rel-19 NR NTN. It is related demodulator but no RRM impacts.

Proposal 3: For uplink capacity/cell throughput enhancement for FR1-NTN, no RRM impacts.

2.3 Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN

For objective#3 in WID, it is RAN2/RAN3 signal design, no RRM impacts.

Proposal 3: For objective#3, no RRM impacts.

2.4 Support of regenerative payload

In TR 38.821:

The typical scenario of a non-terrestrial network providing access to user equipment is depicted below:

Figure 4.1-1: Non-terrestrial network typical scenario based on transparent payload

Figure 4.1-2: Non-terrestrial network typical scenario based on regenerative payload

In NTN based on regenerative payload, the satellite consists of most of the gNB functions including payload, RF filtering, frequency conversion, amplification, encoding/decoding, modulation/demodulation and so on. The reference point of uplink timing requirements can be different.

In Rel-17 the TA mechanism in NTN is introduced as:

The UE shall have capability to follow the frame timing change of the reference cell in connected state. The uplink frame transmission takes place NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

Observation 4: The component of common TA is for the timing component of feeder link delay. If satellite supports regenerative payload which means the gNB function of TA should be from the satellite but not includes feeder link delay.

Observation 5: It is not clear whether RAN will remove common TA or not. Or just set it to zero in this scenario.

For other legacy RRM requirements which involved the feed link impacts, such as re-establishment, satellite switch, RACH-less HO. It needs further RAN2 progress and conclusion. If additional procedure is enhanced in regenerative payload, RAN4 can further discuss potential RRM requirements.

Proposal 4: For support of regenerative payload, FFS on any update of timing requirements.

Proposal 5: For support of regenerative payload, FFS on other RRM requirements such as re-establishment, satellite switch, RACH-less HO etc, further RAN2 progress/conclusion is needed.

2.5 Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands

For objective#5 in WID,

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4]

In TN system, in Rel-17 RedCap UE and RAN4 requirements are introduced. In Rel-18, eRedCap UE and RAN4 requirements are introduced.

Proposal 6: For objective#5 to support RedCap and eRedCap UEs in FR1-NTN. RAN4 should specify new RRM requirements to consider the impact by using the similar assumption in RedCap and eRedCap.

In Rel-17 RedCap WI, it supports:

    - Full-duplex FDD; Half-duplex FDD; TDD

    - Both 1Rx and 2Rx

In Rel-17 NTN FR1, it doesn’t support 1Rx.

Proposal 7: RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx UE.

If we take the TN RedCap/eRedCap/FR1-NTN and conclusion as baseline, we analyze the potential RRM impacts by introducing RedCap/eRedCap support in FR1-NTN system:

- RRC_Idle mode mobility and RRC_Inactive mode mobility

In Idle/Inactive mode, cell selection and cell re-selection are the main procedures. In NTN system, the enhanced cell selection and cell re-selection is time-based and/or location-based cell reselection. In Rel-17 RedCap, eDRX has been introduced. In Rel-18, it is further enhanced.

Proposal 8: To support RedCap/eRedCap in FR1-NTN, RAN4 should to add support eDRX in RRC_Idle/RRC_Inactive mode.

In Rel-17 NTN system, both GSO and NGSO (LEO) are supported.

Observation 6: In eDRX_IDLE cycle, the values are extended to more than 10.24s or even larger one 10485.76s.

Observation 7: In typical LEO scenario such as R=50km and 7.56km/s for satellite speed, not all eDRX can be applicable for NGSO (LEO) scenario.

Proposal 9: RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario.

- Handover

In RedCap, NCD-SSB is introduced.

In addition, in NTN system, RACH-less handover has been introduced which depends on Tsearch or T∆ (time for fine time tracking and acquiring full timing information).

Compared to TN system, the other difference is: in NTN system, the conditional handover is supported including:

- Time/location-based CHO with L3 measurement

- Time/location-based CHO without L3 measurement.

In time/location-based CHO with L3 measurement, the legacy CHO handover delay is as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

For time/location-based CHO without L3 measurements, the legacy CHO handover delay is as:

DCHO = TRRC + TEvent_DU + Tinterrupt + TCHO_execution

In addition, in NTN system, satellite switching with re-synchronization including hard switch and soft switch is specified. RAN4 should discuss the satellite switch time It can also be discussed on whether the same or different principle can be used in handover procedure or not.

Proposal 10:

RAN4 should discuss and specify requirements for RedCap/eRedCap for FR1-NTN including:

- HO based RACH

- RACH-less HO

- Time/location-based CHO with/without L3 measurement

- Satellite switching with re-sync

to consider 1Rx and NCD-SSB

- RRC Re-establishment/RRC connection Release with re-direction

In such delay requirements, if 1RX is supported, define new requirements to support RedCap/eRedCap in FR1-NTN.

Proposal 11: To support RedCap/eRedCap in FR1-NTN, for RRC Re-establishment/RRC connection Release with re-direction, new requirements should be introduced. For 2Rx, legacy NTN FR1 requirements can be reused.

- Timing.

Proposal 12: To support RedCap/eRedCap in FR1-NTN, decide whether to support NCD-SSB. For timing requirements, no big RRM impacts.

- RLM/BFD/CBD

The hypothetical PDCCH transmission parameter should be studied and defined for 1Rx UE. RAN4 can use the parameters in TN RedCap as baseline.

The other enhancement is to add support for half-duplex.

Proposal 13: For RLM/BFD/CBD requirements, to support RedCap/eRedCap in FR1-NTN:

- RAN4 to define the new requirements to consider 1RX. TN RedCap 1Rx can be used as baseline.

- RAN4 to add support for half-duplex.

- SCell/PSCell/Interruption requirements.

Proposal 14: For Scell/PSCell/Interruption etc, no RRM impacts because NTN only supports single carrier.

- Measurement procedure

Proposal 15: For measurement procedure requirements, to support RedCap/eRedCap in FR1-NTN:

- RAN4 to define the new requirements to consider 1RX.

- RAN4 to add support for half-duplex.

For CSI-RS based L3 measurements, in NTN system, it doesn’t support it.

Proposal 16: For CSI-RS based L3 measurement, to support RedCap/eRedCap in FR1-NTN, no RRM impacts.

- L1-RSRP

Proposal 17: For L1-RSRP requirements, to support RedCap/eRedCap in FR1-NTN:

- RAN4 to define the new requirements to consider 1RX.

- RAN4 to add support for half-duplex.

Proposal 18: In summary, the potential RRM impact is as below:

| Topic | Description | RRM impact |
| --- | --- | --- |
| Downlink coverage enhancements | In “off” state, no DL-RS transmission Wait for conclusion of SSB periodicity extension | YES |
| Uplink Capacity/Cell Throughput Enhancement for FR1-NTN | OCC support for PUSCH | NO |
| Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN | Signaling support for broadcast | NO |
| Regenerative payload | Support of regenerative payload | FFS |
| Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands | Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands | YES |

- Conclusion

In this contribution, we provide our proposals for RRM impacts analysis of Rel-19 NTN Phase 3:

Observation 1: The number of active beams is much less than the total number of beams.

Observation 2: There is “off” state which means no any signal which means UE cannot receive DL signal including DL RS during “off” state.

Observation 3: The legacy RAN4 RRM requirements are mainly based on DL RS including SSB or CSI-RS. If there is no SSB or CSI-RS as the measurement RS, it cannot perform the measurements.

Proposal 1: If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed.

Proposal 2: All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs.

Proposal 3: For uplink capacity/cell throughput enhancement for FR1-NTN, no RRM impacts.

Observation 4: The component of common TA is for the timing component of feeder link delay. If satellite supports regenerative payload which means the gNB function of TA should be from the satellite but not includes feeder link delay.

Observation 5: It is not clear whether RAN will remove common TA or not. Or just set it to zero in this scenario.

Proposal 4: For support of regenerative payload, FFS on any update of timing requirements.

Proposal 5: For support of regenerative payload, FFS on other RRM requirements such as re-establishment, satellite switch, RACH-less HO etc, further RAN2 progress/conclusion is needed.

Proposal 6: For objective#5 to support RedCap and eRedCap UEs in FR1-NTN. RAN4 should specify new RRM requirements to consider the impact by using the similar assumption in RedCap and eRedCap.

Proposal 7: RAN4 should define separate sets of RRM requirements for 1Rx and 2 Rx UE.

Proposal 8: To support RedCap/eRedCap in FR1-NTN, RAN4 should to add support eDRX in RRC_Idle/RRC_Inactive mode.

Observation 6: In eDRX_IDLE cycle, the values are extended to more than 10.24s or even larger one 10485.76s.

Observation 7: In typical LEO scenario such as R=50km and 7.56km/s for satellite speed, not all eDRX can be applicable for NGSO (LEO) scenario.

Proposal 9: RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario.

Proposal 10:

RAN4 should discuss and specify requirements for RedCap/eRedCap for FR1-NTN including:

- HO based RACH

- RACH-less HO

- Time/location-based CHO with/without L3 measurement

- Satellite switching with re-sync

to consider 1Rx and NCD-SSB

Proposal 11: To support RedCap/eRedCap in FR1-NTN, for RRC Re-establishment/RRC connection Release with re-direction, new requirements should be introduced. For 2Rx, legacy NTN FR1 requirements can be reused.

Proposal 12: To support RedCap/eRedCap in FR1-NTN, decide whether to support NCD-SSB. For timing requirements, no big RRM impacts.

Proposal 13: For RLM/BFD/CBD requirements, to support RedCap/eRedCap in FR1-NTN:

- RAN4 to define the new requirements to consider 1RX. TN RedCap 1Rx can be used as baseline.

- RAN4 to add support for half-duplex.

Proposal 14: For Scell/PSCell/Interruption etc, no RRM impacts because NTN only supports single carrier.

Proposal 15: For measurement procedure requirements, to support RedCap/eRedCap in FR1-NTN:

- RAN4 to define the new requirements to consider 1RX.

- RAN4 to add support for half-duplex.

Proposal 16: For CSI-RS based L3 measurement, to support RedCap/eRedCap in FR1-NTN, no RRM impacts.

Proposal 17: For L1-RSRP requirements, to support RedCap/eRedCap in FR1-NTN:

- RAN4 to define the new requirements to consider 1RX.

- RAN4 to add support for half-duplex.

Proposal 18: In summary, the potential RRM impact is as below:

| Topic | Description | RRM impact |
| --- | --- | --- |
| Downlink coverage enhancements | In “off” state, no DL-RS transmission Wait for conclusion of SSB periodicity extension | YES |
| Uplink Capacity/Cell Throughput Enhancement for FR1-NTN | OCC support for PUSCH | NO |
| Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN | Signaling support for broadcast | NO |
| Regenerative payload | Support of regenerative payload | FFS |
| Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands | Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands | YES |

- Reference

[1] RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT