3GPP TSG-RAN WG4 Meeting # 112		R4-2412113

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:		8.23.2

Source:		Samsung

Title:	Discussion on RRM impacts on Rel-19 mobility enhancement Phase 4

Document for:		Discussion

- Introduction

The latest WID [1] has been approved in the RAN plenary RAN#104 meeting. The objectives of Rel-19 mobility enhancement Phase 4 are described in [1] including:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we analyze the potential impacts to RRM requirements related to the above objectives.

- Discussion

2.1 Inter-CU Layer1/Layer 2 Triggered Mobility

For first bullet of objectives in WID, RAN2/RAN3 started the discussion on inter-CU LTM.

In Rel-18 mobility enhancement WI, the intra-CU LTM has been discussed and specified. RAN4 specified such requirements such as LTM PCell Cell switch requirements.

In RAN2#125bis meeting, RAN2 agreed:

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

From the measurement and reporting procedure, inter-CU will use the same as that in intra-CU. Therefore, there is no special enhancement for inter-CU LTM. From the RRM requirements, they can use the same requirements for intra-CU scenario.

Proposal 1: For Inter-CU LTM, no enhancement for RRM requirements. The requirements in Rel-18 can be applied to both intra-CU and inter-CU scenarios.

2.2 Measurements related enhancements for purpose of supporting LTM

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

For second bullet of objectives in WID, RAN2 started the discussion on the procedures.

From the definition in WID, RAN2 and RAN1 will specify at least including: event triggered L1 measurement report and CSI-RS measurements for LTM and CSI-RS beam management.

For the CSI-RS measurement and based beam management for LTM, RAN1 hasn’t started the discussion. We need further RAN1 progress. However, it will be defined, RAN4 should define new RRM requirements for CSI-RS measurement and based beam management for LTM, definitely.

Proposal 2: For CSI-RS measurement and based beam management for LTM, RAN4 should define new RRM requirements. Need further RAN1 progress.

For event triggered L1 measurement reporting, in last RAN2#126 meeting, RAN2 agreed as below:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

From the agreements, it can be understood, the event triggered L1 measurement is introduced. At least, four Events are considered. The beam level measurement will be used at least.

For CSI-RS based, RAN1 hasn’t started the discussion. RAN4 can start the discussion based on SSB firstly.

Proposal 3: RAN4 should discuss and specify new RRM requirements of delay/measurement period requirements based on SSB and CSI-RS resource. RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed.

In current spec, L3 measurement in clause 9 is based on event-trigger. In the delay requirements based on SSB, current LTM L1-RSRP measurement delay can be reused.

Proposal 4: For the delay of event triggered L1 measurement reporting requirements based on SSB, current LTM L1-RSRP measurement delay can be reused.

2.3 Specify support of conditional LTM

- Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

In Rel-18, it cannot support the UE based LTM. RAN4 can enhance the delay for conditional LTM. It also needs progress from RAN1/RAN2. We can come back to this discussion after the checkpoint.

Proposal 5: If conditional LTM is introduced, RAN4 need to define the requirements as least for delay requirements. Wait further progress after the checkpoint.

- Conclusion

In this contribution, we provide our proposals for RRM impacts analysis of Rel-19 mobility enhancement Phase 4:

Proposal 1: For Inter-CU LTM, no enhancement for RRM requirements. The requirements in Rel-18 can be applied to both intra-CU and inter-CU scenarios.

Proposal 2: For CSI-RS measurement and based beam management for LTM, RAN4 should define new RRM requirements. Need further RAN1 progress.

Proposal 3: RAN4 should discuss and specify new RRM requirements of delay/measurement period requirements based on SSB and CSI-RS resource. RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed.

Proposal 4: For the delay of event triggered L1 measurement reporting requirements based on SSB, current LTM L1-RSRP measurement delay can be used as baseline.

Proposal 5: If conditional LTM is introduced, RAN4 need to define the requirements as least for delay requirements. Wait further progress after the checkpoint.

- Reference

[1] RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom