3GPP TSG-RAN WG4 Meeting # 112		R4-2412114

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:		5.27.1

Source:		Samsung

Title:	Discussion on RRM core maintenance for MIMO evolution for downlink and uplink

Document for:		Discussion

- Introduction

In this contribution, we discuss RRM core maintenance for Rel-18 MIMO.

- Discussion

In last RAN4#111 meeting, the WF is approved in [1].

We will discuss the two issues in the WF.

Issue 1-1-3: For PDCCH order RACH, uplink timing and DL timings association. < Agreement> - For intra-cell, confirm the association in RAN1 LS R4-2407007. - For inter-cell, FFS on: - If the PRACH is triggered towards serving cell PCI, the uplink transmission takes place NTA offset×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the reference cell associated with the coresetPoolIndex having the same TAG as that serving cell, where NTA offset is the first n-TimingAdvanceOffset value, - If the PRACH is triggered towards active additional cell PCI, the uplink transmission takes place NTA offset×Tc before the reception of the first detected path (in time) of [one of] the corresponding downlink reference signal(s) of the reference cell associated with the coresetPoolIndex having the same TAG as that active additional PCI, where NTA offset is the second n-TimingAdvanceOffset value.

For PDCCH order RACH in inter-cell, in last meeting, we showed that RAN1 also supported inter-cell scenario. RAN4 should define such uplink timing requirements. But due to limited time for further discussion, some companies want to further check it.

In RAN1 spec,

DCI Format 1_0 PRACH association indicator - 0 or 1 bit - 1bit if the UE is provided with tag-Id2, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. - This field indicates the PCI associated with the PRACH transmission if the UE is provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the PCI of the serving cell, and the bit field index 1 of this field is mapped to the additional PCI associated with active TCI states. - This field indicates the PL-RS for the PRACH transmission if the UE is not provided SSB-MTC-AddtionalPCI. The bit field index 0 of this field is mapped to the DL RS that the DM-RS of the PDCCH order is quasi-collocated with, and the bit field index 1 of this field is mapped to the SS/PBCH indicated by the SS/PBCH index field in this DCI format. - 0 bit otherwise.

In RAN1 spec, it only defined the association between TCI states and serving cell/additional PCI cell.

In RAN4 spec, the reference timing of uplink channels/signals are defined. If only keep the RAN1 spec, there is no clear definition of reference timing of PRACH transmission.

Proposal 1: Confirm the association for inter-cell scenario for PRACH. RAN4 should specify the reference timing/point of PRACH transmission. The wording is in CR stage.

Issue 1-2-2: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs (RTD<CP and RTD>CP)? < Way forward > - Option 1 (Apple, Samsung) - For joint TCI state, no additional DL RS tracking time for UL TCI state switching. - For separate UL TCI state, If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as: - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms) - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS + 4*Ttarget_PL-RS + 2ms - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex - For other cases of separate UL TCI state, no additional DL tracking is needed. - Option 2: (Huawei, MediaTek, Ericsson) - No additional DL RS tracking time for UL TCI state switching

Compare Option 1 and Option 2, the only difference is when TOk-ref = 1. For other condition, option 1 and option 2 are the same.

If it is joint TCI state, the UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch. In such procedure, the DL timing can always be achieved by DL TCI. Option 2 can be used.

If it is separate UL TCI state, if the DL beams are changed at the same time. The UE can be configured with the list of up to 128 TCI-State configurations. After DL beams changed, if DL TCI state is not in active list, the previous DL timing cannot be used for accuracy. The update of DL timing tracking is needed.

Proposal 2: For joint TCI state, the UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch. The DL timing can always be achieved by DL TCI. No additional DL RS tracking time for UL TCI state switching.

For separate UL TCI state switch:

- If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as:

  - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

  - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex

- For other cases, no additional DL tracking is needed.

- Conclusion

In this contribution, we provide our proposals for RRM core maintenance of Rel-18 MIMO evolution:

Proposal 1: Confirm the association for inter-cell scenario for PRACH. RAN4 should specify the reference timing/point of PRACH transmission. The wording is in CR stage.

Proposal 2: For joint TCI state, the UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch. The DL timing can always be achieved by DL TCI. No additional DL RS tracking time for UL TCI state switching.

For separate UL TCI state switch:

- If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as:

  - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

  - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex

- For other cases, no additional DL tracking is needed.

- Reference

[1] R4-2410317, WF on RRM requirements for NR_MIMO_evo_DL_UL, Samsung