3GPP TSG-RAN WG4 Meeting # 112		R4-2412115

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:		5.23.7

Source:		Samsung

Title:	Discussion on RRM maintenance for Rel-18 NTN enhancement

Document for:		Discussion

- Introduction

In this contribution, we discuss RRM maintenance for Rel-18 NTN enhancement.

- Discussion

In last RAN4#111 meeting, the WF was approved in [1].

There are still some FFS in the performance part. We continue discussion on those open issues.

### Issue 6-2-2: (FR2-NTN) Rx beam gain Agreement: (ad-hoc + online) - The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with [1]dB relaxation. - Note: Companies are encouraged to further evaluate the performance loss due to single polarization assumption on FR2 Ka band VSAT UE. It’s not precluded to further update the tentative relaxation value in future RAN4 meeting. - Remove an RF margin for different RX beams in the relative accuracy. - Define the minimum SSB_RP condition for accuracy requirement of five UE types specified in table 9.2.1.0-1 of TS38.101-5. Minimum SSB_RP_NTN_FR2 for Rx Beam Peak angle of arrival = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n, where, - Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5. - PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32; - SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB; - SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise; - ∆MBP,n is 0. - For the minimum SSB_RP condition, - The gain difference between fine and rough beams is YdB: - For mechanical steering antenna, Y=0 - For electronic steering antenna, FFS Y - Do not define separate spherical coverage unless spherical coverage is introduced in RF session. - FFS: Gmin and Gmax - Note: If anything above inconsistent with RF requirement is identified, RAN4 to make updates to those aspects accordingly.

In last meeting, Y is still open for electronic steering antenna.

For the RF margin for different RX beams, in legacy Rel-15 requirements, the UE RX beam types are defined in assumption as:

- Fine UE RX beams, used to define UE RF EIS spherical coverage and so on.

- Rough UE beams, used for RRM measurement such as SSB measurement.

- UE RX beam peak is used for fine beam for UE RF beam peak requirements.

In addition, according to current conclusion in RF session, RF requirements are only based on RX Beam Peak direction. SSB_RP can be updated for Rx Beam Peak for fixed UE and mobile UE separately.

### 10.3.1 General The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel. ### 10.3.2 Minimum requirement The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as [specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT]. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle). The EIS of Rx beam peak direction should be verified within the declared minimum elevation angle supported for receiving. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type. Where the supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 10.3.2-1 below is taken as an example.

RAN4 has already agreed Y = 0 for mechanical steering antenna.

For the electronic steering antenna or called phase array antenna, the phase array and antenna elements are similar as that in TN UEs. The RF margin exists for different RX beams. Although the principle might be similar as UE in TN. In RF session, EIS spherical coverage is discussed at the beginning. However, according to current RF conclusion, there is no such definition. So, it cannot be assumed as legacy TN UE. For electronic steering antenna, Y is 0 as well.

Proposal 1: For electronic steering antenna, Y is 0.

For the discussion of Gmin and Gmax, in RAN4#111 meeting, the Gmin and Gmax of PC1/5/6 were discussed and WF in [2] has been approved.

In [2], Gmin is agreed as: the value derived based on minimum EIS delta compared to PC3

### Issue 1-1-2 What is the UE gain G for PC 1/5/6? (Table B.2.1.5.1-1: UE gain G, Rx beam peak direction) - Agreement from ad-hoc minutes - For minimum dBi: value from proposal 1 agreed with [ ]. - Note: Value derived based on minimum EIS delta compared to PC3) - For maximum dBi: - Equation: - Option 1: 20log (# antenna element) +5dBi +3 - FFS whether need to update the value for PC3 to align the equation - Option 2: 10log (# antenna element) +5dBi +3 (PC3 assumption) - Number of antenna elements - Option 1: 144 for PC1, 64 for PC5/6? - Option 2: aligned with the assumption from Y and Z - 64 for PC1, 36 for PC5/6 - Note: Test ability issue also need to be considered for feasible values of Gmin and Gmax.

The Gmin is derived from the difference in reference sensitivity compared to the PC3.

In NTN RF spec, the reference sensitivity is -122dBm per BW for NTN VSAT type 1, 2, 4, 5 and -115.6dBM for NTN VSAT type 3.

Therefore, Gmin can be 33.7dBi for NTN VSAT type 1, 2, 4, 5 while 27.3dBi for NTN VSAT 3.

Proposal 2: Gmin can be 33.7dBi for NTN VSAT type 1, 2, 4, 5 while 27.3dBi for NTN VSAT type 3.

For Gmax, it depends on the number of antenna element according to the agreement above. However, in the RF discussion, there was no common assumption. This value depends on the typical products of implementation.

Proposal 3: Gmax depends on typical implementation of antennas.

- Conclusion

In this contribution, we provide our proposals for RRM maintenance of Rel-18 NTN enhancement:

Proposal 1: For electronic steering antenna, Y is 0.

Proposal 2: Gmin can be 33.7dBi for NTN VSAT type 1, 2, 4, 5 while 27.3dBi for NTN VSAT type 3.

Proposal 3: Gmax depends on typical implementation of antennas.

- Reference

[1] R4-2410380, WF on RRM requirements for NR_NTN_enh, Qualcomm

[2] R4-2410294, WF on defining the missing testing parameter for PC1/5/6, Samsung