# 3GPP TSG-RAN WG4 Meeting #112	R4-2412117

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.2.1

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In RAN4 #111 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 2-1-1: Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF Applicability requirement: Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. UE Power class: FFS the following options and moderator option: - Option 1 (Apple, OPPO, LGE, vivo, HW, Samsung): - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Option 1a (LGE): - but RAN4 should consider if other power classes could apply the outcome of the WI discussion - Option 2 (CATT): - It is proposed not to restrict the number of configured carriers and applied power class. - Option 3 (Ericsson): - We can have generic requirements for all power classes, maybe PC6 can be precluded, if no specific use cases for some power classes are pursued. - Option 4 (Moderator) - Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. Other clarification on WID: - FFS: - “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

For UE power class, it’s proposed to consider UE supporting FR2-1 power class 3 as first priority.

Proposal 1: It’s proposed to consider UE supporting FR2-1 power class 3 as first priority.

For UE supporting multiple Rx simultaneous reception for L3 delay enhancement, we also think it means UE supporting simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window, but it does not mean that UE can process multiple SSBs from different directions of the target frequency in parallel, because multiple Rx simultaneous reception is the simultaneous reception capability, not the simultaneous process capability.

Proposal 2: “For UE supporting multiple Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel”.

Status in the WF [1]:

Issue 2-1-2: Conditions to apply L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The condition here means in which case/condition/use-case/mode UE can apply the L3 measurement delay reduction by optimizing Rx BSF. Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF: - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - Option 1 (Apple, CATT, ZTE, vivo, CTC, Intel): multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature. - Option 1a (ZTE): The UE is in multi-Rx operation if following condition is met: UE is configured with group-based beam reporting (GBBR) report - Option 1b (Intel): The UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with group-based beam reporting. The UE is considered activated for L3 reporting when the GBBR is configured not long prior to the expected L3 reporting. - Option 2 (LGE): The L3 measurement delay enhancement requirements for UE supporting multi-Rx simultaneous reception can apply if multiple panels are activate and SSBs in a SMTC window can be measured with multiple beams. - Option 3 (CMCC): the conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode, as for whether the condition is same as that for Rel-18 multi-Rx simultaneous reception can be further discussed. - Option 4 (HW, Nokia, Ericsson): Do not reuse the same applicable conditions specified in Rel-18 multi-Rx - Option 4a (QC) : R19 Rx BSF is independent to R18 multi-Rx simultaneous reception feature includes conditions and requirement. - Option 5 (MTK): The conditions in R18 multi-Rx WI do not limit the discussion on having further conditions in R19 for UE supporting multi-Rx to enhance FR2-1 SSB based L3 measurement delay - FFS: UE’s mobility status, e.g., whether HST is precluded or not - Option 1 (Apple, ZTE(preclude HST), vivo, CTC, Samsung(preclude HST)): low mobility status, i.e., preclude HST - Option 2 (CATT, CMCC): enhanced BSF can also be used for HST - Option 3 (Ericsson): no scenario restriction (e.g., low mobility) is needed - Option 4 (MTK): For UE with high mobility - FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - Option 1 (Apple): the legacy searcher assumption and legacy CSSF shall still be applied for L3 RRM measurement with two panels activated - Option 1a (Ericsson): - The WI shall prioritize the use case of the same receiver for search and measurement processing on one carrier simultaneously received from multiple panels, e.g. a single searcher receives and processes the same carrier on multiple panels. - Option 2 (vivo, HW): RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - Option 1 (vivo, QC, Intel, Apple): needs SSB processing delay/time for processing multiple beams received in a SMTC - FFS: Power consumption issue - Option1 (Apple): considering power consumption, BSF reduction of L3 measurement will not trigger UE to activate multi-Rx. - Option 2 (LGE): For power consumption of multi-Rx operation, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 3 (Ericsson): To avoid unnecessary power consumption and computation load, enabling/disabling the parallel L3 measurement on multiple panels (if it is one of solutions addressing L3 measurement delay) may be determined by at the least one of the below options: - Option 3-1: NW indicates UE enabling parallel L3 measurement on multiple panels for serving L3 measurement delay reduction through L3 or lower layers signalings. - Option 3-2: UE determines to apply parallel L3 measurement on multiple panels for serving L3 measurement delay reduction, and enable it after acknowledged by NW. - Option 3-3: UE determines to apply parallel L3 measurement on multiple panels for serving L3 measurement delay reduction if a condition is fulfilled, e.g., at cell edge. NW may be aware of it by sending a ‘allowance’ signalling or not aware of it. - Option 4 (Nokia): Discuss FBS triggering conditions among the following options: - Option 4-1: Network configuration of FBS (similar as option 3-1) - Option 4-2: Mobility Event triggering FBS - Option 4-3: Conditional Handover configuration - FFS: UE has prior knowledge on the cell to be measured - Option 1 (CATT): on top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed - Option 2 (vivo, CTC, MTK): UE needs prior knowledge on the cell to be measured - Option 2a (CTC): It’s proposed to consider conditions of prior knowledge on the cell to be measured and discuss whether the conditions of prior knowledge are applicable - Option 3 (Ericsson): UE may only measure part of spatial directions with one panel out of multiple panels. It reduces L3 measurement delay as well, upon acquiring prior knowledge on the cell to be measured, e.g., - The UE has measured the cell before in a time period. - The UE has knowledge on the absolute/relative location of the cell to be measured. - The UE has knowledge on its moving state (including rotation). - FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - Option 1 (vivo, Samsung): Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - FFS: cell-centre UE or cell-edge UE - Option 1 (Nokia, MTK): RAN4 to consider L3 FBS targeting cell edge scenarios - FFS: DRX is configured or not (newly added issue in this meeting based on Xiaomi’s proposal) - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized - FFS: Simultaneous operation between L3 and L1 measurements - Option 1 (ZTE): Simultaneous operation between L3 and L1 measurements by optimizing Rx BSF, simultaneous operation between L3 measurement and data reception by optimizing Rx BSF - Option 2 (Ericsson): Scenario where L3 measurement is reduced using reduced beam sweeping and scenario where L1 measurement is reduced using multiple-reception from multi-TRP for DL measurement/data are different scenarios and not expected to operate simultaneously. - L3 measurement delay reduction may be influenced by L1 measurement/data transmission scheme, wherein L1 measurement may be: - Legacy requirements or, - Enhanced requirements for Multi-Rx in Rel-18 - We prefer legacy requirements between them. Once L1 measurement is chosen, we shall further check L1/L3 sharing scheme. - FFS: UE is in RRC CONNECTED mode (newly added issue in this meeting based on ZTE’s proposal) - Agreement: Only support multi-Rx L3 measurement for CONNECTED UE [Moderator]: Option 1 is agreeable, it’s already clearly stated in WID. Will be removed in the formal version.

For conditions to apply L3 measurement delay reduction by optimizing Rx BSF, UE shall support Rel-18 multi-Rx capability and multi-Rx simultaneous reception of UE is in active mode, but the conditions are not needed to be same as Rel-18 multi-Rx work item.

Proposal 3: For conditions to apply L3 measurement delay reduction by optimizing Rx BSF, UE shall support Rel-18 multi-Rx capability and multi-Rx simultaneous reception of UE is in active mode, but the conditions are not needed to be same as Rel-18 multi-Rx work item.

The discussion on L3 measurement delay reduction by optimizing Rx BSF can be focused on low mobility status, and the mobility of high speed condition is precluded.

Proposal 4: The discussion on L3 measurement delay reduction by optimizing Rx BSF can be focused on low mobility status, and HST is precluded.

It’s proposed to consider the condition whether UE has prior knowledge on the cell to be measured or not, which may have different impacts on BSF reduction.

Proposal 5: It’s proposed to consider the condition whether UE has prior knowledge on the cell to be measured or not, which may have different impacts on BSF reduction.

We support that Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment.

Proposal 6: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment.

Status in the WF [1]:

Issue 2-1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF [Moderator note]: The scenarios here means which UE behavior/activity/procedure(s) would be improved with this feature or which corresponding requirements in the existing RRM spec will be enhanced to accommodate this feature. FFS on the justifications to decide which of the following scenarios shall be considered. - Encourage companies to discuss the expected benefit to considering BSF reduction for the following scenarios. FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 1a (OPPO): For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply. - Option 2: No - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 2: No - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 1a (OPPO): For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply. - Option 2: No - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 2: No - Scenario 5: Handover - Option 1(NTT DCM, CATT, CMCC, ZTE, CTC, Ericsson, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, Xiaomi, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 6: PSCell addition - Option 1(NTT DCM, ZTE, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Option 1(NTT DCM, ZTE): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 8: SCell activation - Option 1(MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 9: SCG activation - Option 1(ZTE, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 10: CGI identification - Option 1 (NTT DCM, ZTE): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario. - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. - Option 1: Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario

For scenarios to use L3 measurement delay reduction by optimizing Rx BSF, in our views, the L3 measurement process of SSB based Intra-frequency/Inter-frequency measurement without/with MG can be considered, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra, TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter. Besides, unknown target FR2 cell delay requirements in Handover scenario can also be considered.

Proposal 7: For scenarios to use L3 measurement delay reduction by optimizing Rx BSF, the L3 measurement process of SSB based Intra-frequency/Inter-frequency measurement without/with MG can be considered, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra, TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter.

Proposal 8: For scenarios to use L3 measurement delay reduction by optimizing Rx BSF, unknown target FR2 cell delay requirements in Handover scenario can be considered.

Status in the summary [2]:

Issue 2-1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF - Proposal 1 (Apple): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 1a (CTC) - Rx beam sweeping factor methods in Rel-18 multi-Rx/ eFeRRM WI can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction. - Proposal 2 (Xiaomi): - UE multiple RX beams shall be used for the same cell’s measurement in order to reduce L3 measurement delay. - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 3 (LGE): - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 4 (QC): - RAN4 discuss whether NW needs to know whether UE is performing fast beam sweeping. - Proposal 5 (Intel): - The L3 measurement delay Beam Sweeping Factor can be cut half for simultaneous reception UE when only single carrier is configured. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Proposal 6 (Ericsson): - RAN4 to study whether to specify the switch between the L1 multi-Rx and L3 multi-Rx operation occasions, e.g. minimum time period between switching. - As a solution for L3 measurement delay reduction, apply the parallel L3 measurement on multiple panels at UE with less RX beam sweep factor on panels symmetrically, e.g., [4]. - Regarding the case of SSB based Intra/inter-frequency measurement, apply reduced Rx beam sweeping in the time period for subsequent operation(s) compared to the time period for prior operation(s), e.g., different Rx beam sweeping factor consequently in Tpss/sss_sync, TSSB_time_index_inter and Tssb_measurement_period.

In Rel-18 multi-Rx WI, fast beam sweeping method was introduced for FR2 L1 measurement delay reduction, which can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF.

Proposal 9: Fast beam sweeping method introduced for FR2 L1 measurement delay reduction in Rel-18 multi-Rx WI can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, with the following proposals:

Proposal 1: It’s proposed to consider UE supporting FR2-1 power class 3 as first priority.

Proposal 2: “For UE supporting multiple Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel”.

Proposal 3: For conditions to apply L3 measurement delay reduction by optimizing Rx BSF, UE shall support Rel-18 multi-Rx capability and multi-Rx simultaneous reception of UE is in active mode, but the conditions are not needed to be same as Rel-18 multi-Rx work item.

Proposal 4: The discussion on L3 measurement delay reduction by optimizing Rx BSF can be focused on low mobility status, and HST is precluded.

Proposal 5: It’s proposed to consider the condition whether UE has prior knowledge on the cell to be measured or not, which may have different impacts on BSF reduction.

Proposal 6: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment.

Proposal 7: For scenarios to use L3 measurement delay reduction by optimizing Rx BSF, the L3 measurement process of SSB based Intra-frequency/Inter-frequency measurement without/with MG can be considered, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra, TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter.

Proposal 8: For scenarios to use L3 measurement delay reduction by optimizing Rx BSF, unknown target FR2 cell delay requirements in Handover scenario can be considered.

Proposal 9: Fast beam sweeping method introduced for FR2 L1 measurement delay reduction in Rel-18 multi-Rx WI can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction by optimizing Rx BSF.

- References

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple, RAN WG4 Meeting #111, 20 – 24 May, 2024.

- R4-2408025, Topic summary for [111][228] NR_RRM_Ph5, Apple, RAN WG4 Meeting #111, 20 – 24 May, 2024.