# 3GPP TSG-RAN WG4 Meeting #112	R4-2412118

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.2.2

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN4 #111 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC issues captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization FFS: - Option 1 (Apple): Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently. - Option 2 (ZTE): Besides the case UE is not capable of R18 multi-Rx simultaneous reception, R19 CSSF optimization is also applied to the case that UE is capable of multi-Rx but not configured with GBBR report. - Option 3 (Ericsson): Clarify whether ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’. - Does it indicate that the UE only can use a single panel (and subsequent receiver including baseband) out of multiple panels for reception and measurement? If so, is there any degradation of reception and measurement from measuring on a single panel? - Option 4 (Nokia): Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios, independently of the UE support of multi-Rx capabilities. Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF FFS: - Option 1: the following scenarios in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - SSB based Intra-frequency measurement without MG (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Alt2: including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_inter and TSSB_measurement_period_inter - Alt2: including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Inter-RAT SSB measurement without MG (CMCC, Ericsson) - NeedForGaps measurement without MG, including both with and without interruption (Ericsson) - NCSG measurement without MG without interruption (Ericsson) Issue 2-2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF FFS: - Option 1 (Apple, CATT, OPPO, CMCC, ZTE): RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - EN-DC (Apple, CATT, OPPO, CMCC, ZTE, Ericsson): - Intel proposed to deprioritize EN-DC - NE-DC (Apple, CATT, OPPO, CMCC, ZTE, Intel, Ericsson): - SA (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel, HW): - FR2+FR2 CA (HW), FR1+FR2 CA (HW, Ericsson), FR2 only intra band CA (Ericsson), FR2 only inter band CA (Ericsson) - NR-DC (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel) - FR1+FR2 NR-DC (HW, Ericsson)

It’s preferred to decouple Rel-19 FR2-1 L3 measurement enhancement and Rel-18 multi-Rx work item, if there is no consensus on the description that “UE is capable of Rel-18 multi-Rx simultaneous reception but work in single Rx currently”, it’s proposed Rel-19 discussion on CSSF optimization can be focused on the case that UE is not capable of Rel-18 multi-Rx simultaneous reception.

Proposal 1: It’s preferred to decouple Rel-19 FR2-1 L3 measurement enhancement and Rel-18 multi-Rx work item, if there is no consensus on the description that “UE is capable of Rel-18 multi-Rx simultaneous reception but work in single Rx currently”, it’s proposed Rel-19 discussion on CSSF optimization can be focused on the case that UE is not capable of Rel-18 multi-Rx simultaneous reception.

For scenarios to use L3 measurement delay reduction by optimizing CSSF, the L3 measurement process of SSB based Intra-frequency/Inter-frequency measurement without MG can be considered, including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra, TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter.

Proposal 2: For scenarios to use L3 measurement delay reduction by optimizing CSSF, the L3 measurement process of SSB based Intra-frequency/Inter-frequency measurement without MG can be considered, including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra, TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter.

The modes of EN-DC, SA, NR-DC, NE-DC are included in the specification, if the workload is not very heavy, these four modes can be considered.

Proposal 3: If the workload is not very heavy, the modes of EN-DC, SA, NR-DC, NE-DC can be considered.

Status in the summary [2]:

Issue 2-2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF - Option 1 (Apple, CTC, Intel, MTK, Ericsson): RAN4 only consider the R19 enhancement based on 2 searchers. - Option 2 (CATT, CMCC): enhance R19 CSSF outside gap by considering both 2 searchers and 3 searchers. - Option 2a (HW): enhance R19 CSSF outside gap by considering 3 searchers for per FR gap capable UE. - Option 4 (Nokia): - RAN4 to confirm if one of the searchers is assumed for PCell measurement and the other is assumed for the measurements on all the SCells. - Recommended WF - Moderator note: if companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. Issue 2-2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC - Option 1 (Apple, CTC, Intel, Ericsson): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 1a (Apple): details of option 1 is: - If PCC in the band, measure PCC - Otherwise if PSCC in the band, measure PSCC - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO - Otherwise up to UE implementation - [Opponent view]Option 1b (HW, Ericsson): The candidate solution of optimizing CSSF (UE only needs to measure one serving CC per band if multiple serving CCs are in the same band) can be implemented by network configuration, e.g., Only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. - Option 1c(Ericsson): - If UE only measures one CC (e.g., determined by either UE or NW) out of multiple CCs, regarding measurement (including reporting) configurations to the CC and other CC(s), some promising approaches are listed as follows: - Option 1c-1: NW measurement configuration doesn’t cover the CC(s) not to be measured. - Option 1c-2: NW measurement configuration covers all CC, by further (e.g. dynamical) indication, - Option 1c-2.1. No measurement report even measurement configuration is configured for the CC(s) which not to be measured. - Option 1c-2.2: If measurement configuration is configured for the CC(s) which not to be measured, the report on the CC(s) reuses the measured result of the CC to be measured. - Option 2 (Apple, CTC, Ericsson): UE can reduce the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - The conditions can be FFS. - [Opponent view] (HW): Dynamical CSSF would complicate UE implementation. - Option 3 (OPPO): - RAN4 to discuss the optimization of counting the number of configured SCell(s) or MOs without MG for CSSFoutside_gap in FR2 L3 measurements. - Option 4 (Ericsson): - RAN4 to study different SMTC configurations in different CCs to optimize CSSF. - RAN4 to study the minimal CC number to apply CSSF enhancement. - Option 5 (Nokia): - To identify the scenarios where SSB-based measurements can be relaxed so that CSSF can be optimized. - To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements. - Option 6 (QC): - NW can group among intra-frequency layers from configured MO. UE can prioritize to measure one frequency layer in each group. - Option 7 (Huawei): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the CSSFoutsidegap can be optimized, as these MOs can share the three searchers. - Three searchers capability is naturally supported by UE who supports per FR gap and it is not a new or enhanced capability. - Recommended WF - Moderator note: - the option 1 has most support, to check if option 1 can be the starting point. - If issue 2-2-4 concluded on 3 searchers, then 3 searchers based solution can also be discussed.

For searcher assumption to apply L3 measurement delay reduction by optimizing CSSF, since the baseline assumption on number of searchers is 2 in WID, it’s proposed to consider the enhancement based on 2 searchers. The enhancement on the number of searchers can be considered in further release.

Proposal 4: For searcher assumption to apply L3 measurement delay reduction by optimizing CSSF, it’s proposed to consider the enhancement based on 2 searchers.

For solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, CSSF value is related to the number of serving cells, with large number of serving cells, L3 measurement delay would be long, thus the number of serving cells related enhancements in CSSF equation can be considered as a starting point. We support to allow UE to measure one serving CC per band if multiple serving CCs are in the same band, and the measurement result of this serving CC can be reused for other serving CCs. Besides, the solution of changing CSSF allocation (statically or dynamically) among PCC/PSCC/SCC can be considered.

Proposal 5: UE can measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 6: The solution of changing CSSF allocation (statically or dynamically) among PCC/PSCC/SCC can be considered.

The bands in FR1 are widely used in network deployment, there also exists the demand of L3 measurement delay reduction in FR1, from operator's perspective, the solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

Proposal 7: The solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, with the following proposals:

Proposal 1: It’s preferred to decouple Rel-19 FR2-1 L3 measurement enhancement and Rel-18 multi-Rx work item, if there is no consensus on the description that “UE is capable of Rel-18 multi-Rx simultaneous reception but work in single Rx currently”, it’s proposed Rel-19 discussion on CSSF optimization can be focused on the case that UE is not capable of Rel-18 multi-Rx simultaneous reception.

Proposal 2: For scenarios to use L3 measurement delay reduction by optimizing CSSF, the L3 measurement process of SSB based Intra-frequency/Inter-frequency measurement without MG can be considered, including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra, TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter.

Proposal 3: If the workload is not very heavy, the modes of EN-DC, SA, NR-DC, NE-DC can be considered.

Proposal 4: For searcher assumption to apply L3 measurement delay reduction by optimizing CSSF, it’s proposed to consider the enhancement based on 2 searchers.

Proposal 5: UE can measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 6: The solution of changing CSSF allocation (statically or dynamically) among PCC/PSCC/SCC can be considered.

Proposal 7: The solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

- References

- R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Apple, RAN WG4 Meeting #111, 20 – 24 May, 2024.

- R4-2408025, Topic summary for [111][228] NR_RRM_Ph5, Apple, RAN WG4 Meeting #111, 20 – 24 May, 2024.