# 3GPP TSG-RAN WG4 Meeting #112	R4-2412119

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.3

Source:	China Telecom

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN #103 meeting, the new WID on NR Radio Resource Management (RRM) Phase 5 was approved [1]. There are two objectives in the work item, one is FR2-1 SSB based L3 measurement delay reduction for connected mode, the other is fast SCell activation for UE supporting Rel-18 EMR.

In this paper, we present our views on fast SCell activation for UE supporting Rel-18 EMR described in the new WID [1].

- Discussion

The objectives in the new WID [1]:

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

Early Measurement Reporting was discussed in Rel-18 to support fast CA/DC setup, as for SCell activation, the delay of SCell activation is very long especially for unknown SCell, thus it’s proposed to reuse the measurement results for SCell activation if EMR report is valid when UE enters connected mode. SCell activation delay requirements with the L3 reporting during activation was specified in Rel-18 eFeRRM work item, in our views, the delay requirements with L3 reporting during activation can be the baseline for the definition of fast SCell activation delay requirements with valid EMR reporting.

For UE supporting Rel-18 EMR, the validity check is needed for the measurement results, if the conditions are not met for the validity check, the measurement results are considered invalid, and UE shall not report invalid measurement results. Therefore, the EMR reporting need to be valid for fast SCell activation, and the validity check can be discussed.

When UE reports valid EMR during SCell activation, probably the processes of AGC setting, cell detection, T/F tracking are not needed.

Proposal 1: The delay requirements with L3 reporting during activation can be the baseline for the definition of fast SCell activation delay requirements with valid EMR reporting.

Proposal 2: The EMR reporting need to be valid for fast SCell activation, and the validity check can be discussed.

Proposal 3: When UE reports valid EMR during SCell activation, probably the processes of AGC setting, cell detection, T/F tracking are not needed.

- Conclusion

The contribution presents our views on fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: The delay requirements with L3 reporting during activation can be the baseline for the definition of fast SCell activation delay requirements with valid EMR reporting.

Proposal 2: The EMR reporting need to be valid for fast SCell activation, and the validity check can be discussed.

Proposal 3: When UE reports valid EMR during SCell activation, probably the processes of AGC setting, cell detection, T/F tracking are not needed.

- References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator), RAN Meeting #103, March 2024.