# 3GPP TSG-RAN WG4 Meeting #112	R4-2412120

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.21.2

Source:	China Telecom

Title:	Discussion on Enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN #104 meeting, the Revised WID for Enhancements of network energy savings for NR was approved [1]. There are four objectives in the work item, including on-demand SSB SCell operation, on-demand SIB1, adaptation of common signal/channel transmissions, and the corresponding core requirements in RAN4. In this paper, we present our views on enhancements of network energy savings for NR.

- Discussion

The objectives in the new WID [1]:

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

For first objective to specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA, there are many aspects and issues to be discussed for the on-demand SSB SCell operation in RAN4 RRM session. For legacy SSB SCell operation, many SCell operation scenarios were introduced from Rel-15 to Rel-18, e.g., SCell activation/ deactivation, direct SCell activation, multiple SCell activation/ deactivation, PUCCH SCell activation/ deactivation, fast SCell activation, SCell activation with L3 reporting, etc. These scenarios for legacy SSB SCell operation can be the directions of discussion for on-demand SSB SCell operation. However, there are too many scenarios for legacy SSB SCell operation, thus it’s necessary to select a basic SCell operation scenario and discuss the baseline scenario firstly in Rel-19. In our views, it’s proposed to discuss and define requirements on single SCell activation supporting on-demand SSB SCell operation firstly. Other scenarios can be considered after baseline scenario being completed.

In details, regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, two cases are identified for further study in RAN1, Case #1 is no always-on SSB on the cell, and Case #2 is always-on SSB periodically transmitted on the cell. In RAN4 RRM session, legacy SCell activation requirements are considered with known SCell and unknown SCell, therefore, it’s proposed to consider Case #1 and Case #2 with known SCell activation and unknown SCell activation.

Observation 1: For legacy SSB SCell operation, many SCell operation scenarios were introduced from Rel-15 to Rel-18, e.g., SCell activation/ deactivation, direct SCell activation, multiple SCell activation/ deactivation, PUCCH SCell activation/ deactivation, fast SCell activation, SCell activation with L3 reporting, etc.

Observation 2: These scenarios for legacy SSB SCell operation can be the directions of discussion for on-demand SSB SCell operation. However, there are too many scenarios for legacy SSB SCell operation, thus it’s necessary to select a basic SCell operation scenario and discuss the baseline scenario firstly in Rel-19.

Proposal 1: It’s proposed to discuss and define requirements on single SCell activation supporting on-demand SSB SCell operation firstly.

Proposal 2: It’s proposed to consider Case #1 and Case #2 with known SCell activation and unknown SCell activation.

For second objective to study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, since the impacts are mainly in RAN1/2/3, it’s proposed to deprioritize on-demand SIB1 discussion in RAN4 only if the impacts in RAN4 RRM are identified.

Proposal 3: It’s proposed to deprioritize on-demand SIB1 discussion in RAN4 only if the impacts in RAN4 RRM are identified.

For third objective to specify adaptation of common signal/channel transmissions, there are many issues to be discussed including adaptation of SSB/ PRACH/ paging occasions, it’s proposed to start the discussion after RAN4 has sufficient information on adaptation of SSB/ PRACH/ paging occasions in RAN1.

Proposal 4: It’s proposed to start the discussion after RAN4 has sufficient information on adaptation of SSB/ PRACH/ paging occasions in RAN1.

- Conclusion

The contribution presents our views on RRM core requirements for enhancements of network energy savings for NR, with the following observations and proposals:

Observation 1: For legacy SSB SCell operation, many SCell operation scenarios were introduced from Rel-15 to Rel-18, e.g., SCell activation/ deactivation, direct SCell activation, multiple SCell activation/ deactivation, PUCCH SCell activation/ deactivation, fast SCell activation, SCell activation with L3 reporting, etc.

Observation 2: These scenarios for legacy SSB SCell operation can be the directions of discussion for on-demand SSB SCell operation. However, there are too many scenarios for legacy SSB SCell operation, thus it’s necessary to select a basic SCell operation scenario and discuss the baseline scenario firstly in Rel-19.

Proposal 1: It’s proposed to discuss and define requirements on single SCell activation supporting on-demand SSB SCell operation firstly.

Proposal 2: It’s proposed to consider Case #1 and Case #2 with known SCell activation and unknown SCell activation.

Proposal 3: It’s proposed to deprioritize on-demand SIB1 discussion in RAN4 only if the impacts in RAN4 RRM are identified.

Proposal 4: It’s proposed to start the discussion after RAN4 has sufficient information on adaptation of SSB/ PRACH/ paging occasions in RAN1.

- References

- RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson, Apple, RAN Meeting #104, June 2024.