# 3GPP TSG-RAN WG4 Meeting #112	R4-2412121

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #111, RRM core requirements for LP-WUS/WUR was discussed, the topic summary is provided in [1] and the way forward is approved in [2]. This contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR.

- Discussion

General aspects

Status in the WF [2] and summary [1]:

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON. FFS on other topics listed in [1] for sub-topic 1-1 Issue 1-1-3: Core requirements to be specified for MR RRM relaxation - Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple xiaomi oppo LG CT ZTE) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. If both LP-WUR and MR are ON, RAN4 to discuss whether UE uses LP-WUR measurement to decide MR relaxation or UE uses both LP-WUR and MR measurement to decide MR relaxation (Apple) - P1-1: RAN4 to specify MR RRM relaxation requirements for serving cell measurements in Idle/Inactive mode when (MTK) - LP-SS based LP-WUR RRM measurement is activated, or - PSS/SSS based LP-WUR RRM measurement is activated Not to consider RRM relaxation based on the hybrid measurements of both (LP-SS and SSS). Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation - Proposals - P1-1: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (Qualcomm) - The number of measurement samples needed for filtering. - The number of consecutive times the UE needs to meet the entry/exit criteria - P1-2: Define requirements on entry/exit criteria for LP-WUR measurement/ MR RRM measurement relaxation based on RAN1 and RAN2 design (CT) - P1-3: RAN4 to discuss the requirement about exit the LR by serving cell evaluation based on the LP-RSRP/LP-RSRQ simulation result and the requirement about entry the LR by serving cell evaluation based on the SSB RSRP/RSRQ. (Ericsson) Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (Apple oppo vivo) - P2: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition - P3: RAN4 to firstly discuss the differences between entry/exit conditions for LP-WUS monitoring defined in RAN1/RAN2 and conditions for LP-WUR measurement. RAN4 should at least consider LP-SS and PSS/SSS when discussing the entry/exit conditions for LP-WUR measurement. RAN4 to wait for RAN1/RAN2, if the entry/exit conditions for LP-WUS monitoring are introduced by RAN1/RAN2, the thresholds on serving cell measurement result can be further discussed in RAN4. (CATT) - P4: Both serving cell quality and UE mobility should be considered for the criteria(entry/exit conditions) for MR RRM measurement relaxation &LP-WUR measurement (State2) and only LP-WUR measurement (State 3). Whether network configure the high priority frequency carrier measurement or not should also be considered when defining the criteria (entry/exit conditions). (CMCC) - P5: When UE enters the State 3 (Measurement only based on LP-WUR), The serving cell quality should at least be Srxlev > SIntraSearchP and Squal > SIntraSearchQ (CMCC) - P6: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective (LG) - P7: RAN4 to discuss serving cell evaluation for entry/exit conditions for PSS/SSS and LP-SS based LR. PSS/SSS based entry/exit evaluation can be prioritised. (Nokia) Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (oppo vivo Apple) - RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2. (Apple) - P2: RAN4 should firstly discuss the meaning of MR RRM measurement relaxation on which phases and what purposes are included for RRM relaxation in the entire procedure, i.e. for cell selection/reselection or handover, for serving cell measurement or neighbor cell measurement, etc. (CATT) - P3: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation (LG) - P3a: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring - Proposals - P1: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary (Apple) - P2: For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ (Samsung) - P3-1: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring (xiaomi) - P3-2: The entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement should be considered jointly.(CMCC) - P3-3: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and LP-WUR RRM measurement. (vivo) - P3-4: RAN4 to confirm that same entry/exit condition for LP-WUS monitoring applies for serving cell RRM measurement offloading (from MR to LP-WUR). (HW) Notes: MR measurements results shall be larger than the threshold and satisfy the related MR relaxation conditions. (ZTE) - P4: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2 (LG) - P5: For the entry/exit conditions for LP-WUS monitoring, the threshold for the serving cell measurement performed by MR or LR shall be decided by RAN4 (ZTE) Recommendations: Suggest to check whether the following top level description: LP-WUR monitoring is triggered no later than one of cases identified in issue 1-1-2

Regarding cases/states to be considered for RRM relaxation, it’s proposed to consider case #2 MR has RRM relaxation on serving cell measurement (no neighboring cell measurement is needed) and LP-WUR performs serving cell measurement, and case #3 MR has RRM relaxation on serving cell and neighboring cell measurement and LP-WUR performs serving cell measurement. As for case #4, it can be FFS.

Proposal 1: Regarding cases/states to be considered for RRM relaxation, it’s proposed to consider case #2 and case #3.

For core requirements to be specified for MR RRM relaxation, since it was approved in the WID that RAN4 and RAN2 specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions, therefore, we support RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Proposal 2: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Regarding criteria (entry/exit conditions) for LP-WUR measurement/ MR RRM measurement relaxation/ LP-WUS monitoring, we support that RAN2 is the main group for criteria (entry/exit conditions) design.  Meanwhile, the requirements on verifying UE correct implementation of the entry/exit criteria need to be discussed and defined in RAN4.

Proposal 3: RAN2 is the main group for criteria (entry/exit conditions) design for LP-WUR measurement/ MR RRM measurement relaxation/ LP-WUS monitoring.

Proposal 4: RAN4 to discuss and define the requirements on verifying UE correct implementation of the entry/exit criteria based on RAN1 and RAN2 design.

LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the WF [2] and summary [1]:

Issue 1-2-6: Timeline on RRM requirement evaluation work - Proposals - P1: RAN4 start RRM requirement for LP-WUR at idle/inactive state after RAN4 has sufficient information on LP-SS design and LP-SS based measurement metric from other sources. (Apple CT)

For LP-WUR requirements at RRC_IDLE/INACTIVE state, regarding timeline on RRM requirement evaluation work, since LP-SS design and LP-SS based measurement metric are discussed in other working groups, it’s proposed to start RRM requirements for LP-WUR at idle/inactive state after RAN4 has sufficient information on LP-SS design and LP-SS based measurement metric from other sources.

Proposal 5: It’s proposed to start RRM requirements for LP-WUR at idle/inactive state after RAN4 has sufficient information on LP-SS design and LP-SS based measurement metric from other sources.

MR RRM relaxation

Status in the WF [2] and summary [1]:

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell - Proposals - P1: Regarding RRM relaxation of UE MR for both serving and neighbor cell measurements, RAN4 to discuss (Apple) - whether serving and neighbor cell measurements can share the same criteria for triggering relaxation or not and, - whether serving and neighbor cell measurements can share the same relaxation requirement (e.g., scaling factor) or not. - P2: RAN4 needs to further study serving cell measurement relaxation, and further neighboring cell measurement relaxation by MR (Samsung CATT Nokia) - P2-1 (CATT): What is the difference between MR RRM measurement relaxation and legacy RRM measurement relaxation (e.g., not-at-cell edge or low mobility in R16/R17)? Whether the legacy criteria for RRM relaxation can be reused for relaxing serving and neighbor cell measurements for UE MR ? Whether RRM measurement of MR is used for neighbor cell measurement when the UE is operating with LP-WUR can be discussed after further input from RAN2. After determining the meaning and the criteria (entry/exit conditions) for MR RRM measurement relaxation in RAN2/RAN4, the method and requirements for MR RRM relaxation will be further discussed in RAN4. - P3: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation (CT vivo ZTE) - P2-1: Equal or larger than 8 (CMCC) - P2-2: >=16 (Huawei) - P4: RAN4 to wait other WGs’ progress to clarify the MR RRM relaxation scenario before discussing the relaxation requirement. (Ericsson) - P5: The legacy SINR condition and accuracy are re-used(HW)

For MR RRM relaxation, MR RRM relaxation factor for serving cell/neighbor cell is an important issue to be discussed in RAN4. Referring to analysis of RRM for LP-WUR duty-cycled monitoring in TR 38.869, the average of power saving gain provided by LP-WUS is 4% when MR relaxed < 8 times, which is very limited. While the gain is 40% when 8 times<= MR relaxed <=16 times, and the gain is 60% when MR relaxed > 16 times. Therefore, it’s proposed to discuss relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for MR RRM relaxation.

Proposal 6: It’s proposed to discuss relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for MR RRM relaxation.

LP-WUR CONNECTED mode

Status in the WF [2] and summary [1]:

Issue 1-5-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is for connected mode in this WI. (Apple Docomo oppo LG CT vivo ZTE) - P2: Postponed until more progress (Ericsson) - P3: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress (Samsung)

For CONNECTED mode, it was described in the objectives of WID that in CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR. Therefore, in our views, there is no RRM objective for connected mode in LP-WUR/LP-WUS WI.

Proposal 7: There is no RRM objective for connected mode in LP-WUR/LP-WUS WI.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: Regarding cases/states to be considered for RRM relaxation, it’s proposed to consider case #2 and case #3.

Proposal 2: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR.

Proposal 3: RAN2 is the main group for criteria (entry/exit conditions) design for LP-WUR measurement/ MR RRM measurement relaxation/ LP-WUS monitoring.

Proposal 4: RAN4 to discuss and define the requirements on verifying UE correct implementation of the entry/exit criteria based on RAN1 and RAN2 design.

Proposal 5: It’s proposed to start RRM requirements for LP-WUR at idle/inactive state after RAN4 has sufficient information on LP-SS design and LP-SS based measurement metric from other sources.

Proposal 6: It’s proposed to discuss relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for MR RRM relaxation.

Proposal 7: There is no RRM objective for connected mode in LP-WUR/LP-WUS WI.

- References

- R4-2408026, Topic summary for [111][229] NR_LPWUS, Moderator (vivo), RAN4 #111, 20 - 24 May, 2024.

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4 #111, 20 - 24 May, 2024.