# 3GPP TSG-RAN WG4 Meeting #112	R4-2412122

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.19.3

Source:	China Telecom

Title:	Discussion on RRM core requirements for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN #104 meeting, the Revised WID for Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved [1]. The objectives were discussed in RAN1 in previous meetings. In this paper, we present our views on RRM core requirements for evolution of NR duplex operation.

- Discussion

The objectives in the revised WID [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

For the objectives of semi-static indication of time/ frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode, SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode, UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE, are mainly discussed in RAN1 and RAN2. The objectives of enhancements for CLI handling are related to RAN4 work, the study on L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework may have impact on RAN4 RRM.

Rel-19 enhancements for CLI handling are focused on L1 based UE-to-UE CLI measurement and reporting, in our views, Rel-16 CLI measurement requirements can be the starting point for the discussion on Rel-19 L1 CLI measurement requirements.

L1 based UE-to-UE CLI measurement and reporting are based on existing CSI framework, measurement resource can be periodic, semi-persistent, or aperiodic measurement resource, and the reporting is at least aperiodic reporting, periodic and semi-persistent reporting can be considered in addition to aperiodic reporting. Therefore, Rel-19 L1 CLI measurement reporting is different from Rel-16 CLI measurement reporting including periodic, event-triggered Periodic, event Triggered reporting, Rel-19 L1 CLI measurement reporting requirements need to be specified. In addition, the corresponding measurement period, measurement accuracy, side conditions, and scheduling restrictions need to be specified.

Since Rel-16 CLI measurement requirements are based on L3 framework while Rel-19 CLI measurement requirements are based on L1 framework, it’s proposed to define better measurement requirements for Rel-19 L1 CLI measurement compared to Rel-16 L3 CLI measurement.

Proposal 1: Rel-16 CLI measurement requirements can be the starting point for the discussion on Rel-19 L1 CLI measurement requirements.

Proposal 2: Rel-19 L1 CLI measurement requirements including measurement reporting, measurement period, measurement accuracy, side conditions, scheduling restrictions need to be specified.

Proposal 3: It’s proposed to define better measurement requirements for Rel-19 L1 CLI measurement compared to Rel-16 L3 CLI measurement.

- Conclusion

The contribution presents our views on RRM core requirements for evolution of NR duplex operation, with the following proposals:

Proposal 1: Rel-16 CLI measurement requirements can be the starting point for the discussion on Rel-19 L1 CLI measurement requirements.

Proposal 2: Rel-19 L1 CLI measurement requirements including measurement reporting, measurement period, measurement accuracy, side conditions, scheduling restrictions need to be specified.

Proposal 3: It’s proposed to define better measurement requirements for Rel-19 L1 CLI measurement compared to Rel-16 L3 CLI measurement.

- References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, RAN Meeting #104, June 2024.