# 3GPP TSG-RAN WG4 Meeting #112	R4-2412124

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.29.1

Source:	China Telecom

Title:	Discussion on RRM core requirements maintenance for NES

Document for:	Discussion

- Introduction

In RAN4 #111 meeting, RRM core requirements for inter-band SSB-less SCell activation for NES was discussed and the WF [1] was approved. In this paper, we present our views on RRM core requirements maintenance for NR network energy saving.

- Discussion

Status in the WF [1]:

Issue 1-1-1: Power difference conditions Agreement: Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Only change [9] dB to [12] dB in this meeting. Further discuss whether and how to support larger EPRE difference in next meeting: - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + ¦20*log (f1/f2)¦ - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS. - Option 2: Further increase [12] dB and the value is FFS - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference - Other options are no precluded. Issue 1-1-3: Multiple SSB-less SCells activation Agreements: - Also define requirements for following scenario: - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous. - All to-be-activated SCells have same QCL source cell. - P-TRS is used. - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR. - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay) - FFS A-TRS based requirements.

In last meeting, EPRE difference was changed from 9 dB to 12 dB, in our views, it’s no need to consider larger EPRE difference since 12 dB is large enough.

Proposal 1: It’s no need to consider larger EPRE difference since 12 dB is large enough.

When considering multiple SSB-less SCells activation, since all to-be-activated SCells are SSB-less on the same band and SCells are contiguous, and all to-be activated SCells have same QCL source cell, we think the AGC and T/F information of one of SCells can be reused for all other SCells activation. Therefore, the multiple SCell activation delay requirements can be based on TRS with the shortest periodicity.

Observation 1: When considering multiple SSB-less SCells activation, since all to-be-activated SCells are SSB-less on the same band and SCells are contiguous, and all to-be activated SCells have same QCL source cell, the AGC and T/F information of one of SCells can be reused for all other SCells activation.

Proposal 2: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity.

- Conclusion

The contribution presents our views on RRM core requirements maintenance for NR network energy saving, with the following observations and proposals:

Proposal 1: It’s no need to consider larger EPRE difference since 12 dB is large enough.

Observation 1: When considering multiple SSB-less SCells activation, since all to-be-activated SCells are SSB-less on the same band and SCells are contiguous, and all to-be activated SCells have same QCL source cell, the AGC and T/F information of one of SCells can be reused for all other SCells activation.

Proposal 2: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity.

- References

- R4-2410269, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #111, May 2024.