3GPP TSG-RAN WG4 Meeting #112	R4-2412125

Maastricht, Netherlands, August 19-23, 2024

Agenda Item:	8.8.2.1

Source:	Ericsson

Title:	On Co-existence considerations for NR and IoT NTN related to High Power UE (HPUE) for NTN

Document for:	Discussion

# 1	Introduction

In the last RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1].

The WI objectives for core part of Enhanced requirements and test methodology for NR and IoT NTN related to High Power UE (HPUE) for NTN have been listed below:

This contribution intends to trigger the discussion on co-existence studies assumptions for evaluation of ACLR and ACS corresponding to the specified power classes.

# 2	Discussion

## General

It has been stated in the WI objectives for co-existence studies to only evaluate impact on the UL performance of TN network for sample bands: n255 and 255 & n256 and 256, corresponding to the following power classes:

- Power Class 2 (PC2)

- Power Class 1.5 (PC1.5)

- Power Class 1 (PC1)

In the WI objective, it is clearly stated that Power class 2 (PC2, 26 dBm) is applicable to both handheld and non-handheld UEs for the co-existence study. However, PC1.5 and PC1 are mentioned for both NR and IoT NTN, where as PC1 only for IoT-NTN.

PC1.5 and PC1 are mentioned for both NR and IoT NTN, whereas PC1 only for IoT-NTN

In TS38.101-1 [3], the UE RF requirements for TN, PC1.5 is specified for TDD bands only. Unless there is a specific need to introduce PC1.5 for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), we propose to remove PC1.5 from the scope of this WI.

Since PC1.5 is specified for TN TDD bands only, unless there is a specific need to introduce PC1.5 for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), we propose to remove PC1.5 from the scope of this WI.

RAN4 to prioritize support of PC2 for NTN UE and support PC1 based on the feasibility studies.

When it comes to UE type, according to the WID objectives both handheld and non-handheld UEs are in the scope, where for other than PC2 high power classes “non-handheld UE is prioritized”. In TS38.101-1 [3] “Table 6.2-1: UE Power Class”, there is a note that:

“Generally, PC1 UE is not targeted for smartphone form factor.”

As the starting point, the same assumption should hold for FR1 NTN. Similar is the case with LTE bands as stated in TS36.101 [4]. Thus, unless NTN handheld UEs prove to be able to support PC1 and there is a specific need to introduce it, we propose to target PC1 for non-handheld UEs only.

Similar to FR1 TN, PC1 should be targeted for non-handheld UEs only for both FR1-NTN and IoT-NTN.

## Simulation Scenarios

In the Table 6.1.-1 extracted from TR 38.821 [5], scenarios for NTN-NTN/ TN co-existence have been discussed. The Dense Urban scenarios were not considered for simulations in previous studies. It is right to say that dense urban scenarios represent the most challenging environment, where higher transit power needs to be considered to get positive values of SNR to address coverage challenges. However, considering an extreme use-case, where due to a natural disaster NTN UE can still opt to connect to satellite network instead of TN network.

Table 6.1-1: Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  | NTN1,4,5 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 22 |  |  | HAPS |
|  |  | GEO3 | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural | X | X | X | X | X | X | X |
|  | Urban macro | X | X | X | X | X | X | X |
|  | Dense Urban6 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| HAPS |  | N/A | N/A | N/A | N/A | N/A | N/A | X |
| NOTE 1: Earth Fixed beam has been considered for co-existence studies. NOTE 2: Use Set 1 satellite antenna has been used as the starting point for co-existence studies. NOTE 3: GEO and LEO only operate at adjacent channel. NOTE 4: Use GEO and LEO@600km when TN is victim. NOTE 5: The satellite-to-satellite coexistence scenarios are not in the scope of this study considering this is already addressed by ITU (ITU RR Article 9 etc.) and regional regulations (e.g. FCC rules). NOTE 6: Dense Urban is not considered as it is expected NTN UE will connect to terrestrial networks rather than satellite networks in such scenario. |  |  |  |  |  |  |  |  |

RAN4 should discuss whether Dense Urban scenarios are needed for simulations, considering higher transmit power and higher antenna gain for UEs, to address coverage challenges.

## Non-handheld UE type

In the Table 6.1.1.1.1-3 extracted from TR 38.821 [5], the UE type: VAST, Handheld and other moving platforms (e.g. aircrafts, vessels, building mounted devices etc) have been discussed. However, the non-handheld UE characteristics still need to be discussed.

Table 6.1.1.1-3: UE characteristics for system level simulations [2]

| Characteristics | VSAT (Note 2) | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear: +/-45°X-pol | Linear: +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

RAN4 to collect information on Non-handheld UE characteristics needed for co-existence evaluation.

## Additional information from 5GAA for Non-handheld UE type

In addition to the above, there was a LS response [6] – “LS on Automotive Requirements for Non-Terrestrial Network (NTN) Enhancements in Rel-19” sent from 5G Automative Association (5GAA) WG4 to 3GPP TSG RAN, requesting to prioritize the definition of appropriate UE types as part of Rel-19 work.

Further details on automotive vehicular mounted NTN UE characteristics and additional parameters [6] are as follows:

RAN4 to consider the above parameters at LEO-600 for discussion on Non-handheld UE characteristics as starting point, and further investigate for GEO and LEO-1200 scenarios.

# Conclusion

In the previous sections we made the following observations:

Observation 1	PC1.5 and PC1 are mentioned for both NR and IoT NTN, whereas PC1 only for IoT-NTN

Based on the discussion in the previous sections we propose the following:

Proposal 1	Since PC1.5 is specified for TN TDD bands only, unless there is a specific need to introduce PC1.5 for FR1-NTN bands n256, n255 and n254 (which are all FDD bands), we propose to remove PC1.5 from the scope of this WI.

Proposal 2	RAN4 to prioritize support of PC2 for NTN UE and support PC1 based on the feasibility studies.

Proposal 3	Similar to FR1 TN, PC1 should be targeted for non-handheld UEs only for both FR1-NTN and IoT-NTN.

Proposal 4	RAN4 should discuss whether Dense Urban scenarios are needed for simulations, considering higher transmit power and higher antenna gain for UEs, to address coverage challenges.

Proposal 5	RAN4 to collect information on Non-handheld UE characteristics needed for co-existence evaluation.

Proposal 6	RAN4 to consider the above parameters at LEO-600 for discussion on Non-handheld UE characteristics as starting point, and further investigate for GEO and LEO-1200 scenarios.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2413365, “On UE RF Tx requirements for NTN HPUE”, 3GPP RAN4#112, Ericsson

- 3GPP TS 38.101-1 V18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- 3GPP TS 36.101 V18.6.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

- TR 38.821, “Solutions for NR to support Non-Terrestrial Networks (NTN)”, V16.2.0, 3GPP.

- RP-232733, “LS on Automotive Requirements for Non-Terrestrial Network (NTN) Enhancements in Rel-19,” Source 5GAA, 3GPP TSG RAN, December 2023