3GPP TSG-RAN WG4 Meeting #112	R4-2412127

Maastricht, Netherlands, August 19-23, 2024

Agenda Item:	8.2.4.1

Source:	Ericsson

Title:	On Urban Macro and Indoor Co-existence simulation results for 14800 to 15350 MHz frequency range

Document for:	Information

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz.

In 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] captured agreements on co-existence simulation assumptions for companies to present results in August meeting.

This contribution provides the simulation results for Urban Macro and Indoor scenario as per the latest agreements.

# 2	Discussion

The simulation assumptions for the Urban Macro scenario have been agreed in WF [3] and captured in TR [4]. In this section some preliminary simulation results for Urban macro and Indoor scenarios are presented, following the captured agreements, and the options listed in the sections below.

## 2. 1	Urban Macro Downlink

In the Urban macro scenario, the following options have been assumed for simulation:

- ISD = 450 m

- UE antenna: both options, FR1 and FR2 like options have been considered.

- UE max Transmission power = 23 dBm or 26 dBm (depending on if FR1 or FR2 UE antenna type has been considered)

- Uncoordinated deployment – to consider the worst-case scenario for requirements.

- For the BS antenna architectures, two arrays structures are considered, and they consider different sub-array structures compared to those proposed in the WF and are aligned with the proposal included in our companion contribution [5,6]

  - 2048 Antenna Elements (AE), 4x1 sub-array, 8x32 array

  - 1536 AE, 6x1 sub-array, 8x16 array

- Indoor probability: both options 0% and 20% have been considered

- Diversity gain: 0 dB

- Number of active UEs is 1

In the following sub-sections within Urban Macro, simulation results for Urban Macro scenario are presented, considering two BS antenna architectures with 1536 and 2048 AE, and for indoor probabilities 0% and 20%.

### 2.1.1	FR2 UE type

The following results are presented with the assumption of FR2 UE antenna type. The UE max power in this case is 23 dBm.

Figure 2.1.1-1 shows the DL SINR for a case without ACI (Adjacent Channel Interference), and it can be observed that under these assumptions of antenna arrays and UE model, the DL coverage is extremely good.

Figure 2.1.1-1: Urban Macro DL SINR for 1536 and 2048 AE, indoor probability 0% and 20%, FR2 UE antenna.

The mean user throughput loss and 5%tile throughput loss for 2 antenna architectures with 1536 and 2048 AE, and for indoor probability 0% and 20% are shown in Figure 2.1.1-2 and Figure 2.1.1-3.

Figure 2.1.1-2: Urban Macro DL 5%tile user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%, FR2 UE antenna.

Figure 2.1.1-3: Urban Macro DL mean user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%, FR2 UE antenna

From Figures 2.1.1-2 and 2.1.1-3, it can be observed that the mean user throughput loss does not lead to definition of any requirement, but the DL 5%tile user throughput loss is five percent between 17 and 21 dB, depending on the simulation combination. Increasing the number of AEs, increases the directionality of transmissions and receptions, and consequently the interference that is captured and, ultimately, the requirements. Also, the indoor 0% case, exposes the users to more interference and leads to higher requirements.

With FR2 UE type in Urban Macro DL, ACIR of approximate 21 dB and 17 dB is observed at Indoor probability of 0% and 20% respectively.

### 2.1.2	FR1 UE type

In the following, results assuming FR1 UE antenna model are presented.

It can be observed form Figure 2.1.2-1, depicting the DL SINR for a case without ACI, that the scenario with ISD 450m presents a small percentage of outage that reduces with the number of AEs and increases with the indoor probability.

Figure 2.1.2-1: Urban Macro DL SINR for 1536 and 2048 AE, indoor probability 0% and 20%, FR1 UE antenna.

The mean user throughput loss and 5%tile throughput loss for the two BS antenna architectures with 1536 and 2048 AE, and for indoor probability 0% and 20% are shown in Figure 2.1.2-2 and Figure 2.1.2-3.

Figure 2.1.2-2: Urban Macro DL 5%tile user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%

Figure 2.1.2-3: Urban Macro DL mean user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%

From Figures 2.1.2-2 and 2.1.2-3 it can be observed that the mean user throughput loss does not lead to definition of any requirement, but the DL 5%tile user throughput loss is five percent around 22 dB and 26 dB for the indoor 20% and indoor 0% cases, respectively. To increase the number of AEs, increases the directionality of transmissions and receptions, and consequently the interference that is captured and, ultimately, the requirements. Also, the indoor 0% case, exposes the users to more interference and leads to higher requirements

With FR1 UE type in Urban Macro DL, ACIR of approximate 26 dB and 22 dB is observed at Indoor probability of 0% and 20% respectively.

Based on the results, it can be seen that FR2 UE antenna leads to lower requirements, compared to FR1 UE antenna, because UEs can better filter the interference, and this results in lower impact from ACI.

In Urban Macro Downlink scenario, FR2 UE antenna architecture leads to lower ACIR requirements at 20% Indoor users.

In our companion contribution [5], we have discussed some important aspects related to BS antenna feasibility. In the calculations, we have considered TRP as 46 dBm (dual polarization), with which we can estimate a value for expected ACLR when considering different power amplifier solutions. Considering 26 dB as the worst case, based on the deduced ACIR values from the above simulation results.

RAN4 should consider both worst-case scenarios as well as BS antenna feasibility aspects of current semiconductor technologies when deciding ACLR/ ACS requirements.

## 2.2		Urban Macro Uplink

### 2.2.1	FR2 UE type

In the following, results assuming FR2 UE antenna model are presented.

Figure 2.2.1-1 shows this Urban Macro scenario with ISD 450 m, is characterized by an outage probability (around 5%), when indoor probability is 20%. For this case, requirements based on UL 5%tile throughput loss cannot be derived, since all the plots are below 5% throughput loss, as it can be observed from Figure 2.2.1-2 and Figure 2.2.1-3, or anyway would be very low, if we further reduce ACIR.

Figure 2.2.1-1: Urban Macro UL SINR for 1536 and 2048 AE, indoor probability 0% and 20%, FR1 UE antenna.

Figure 2.2.1-2: Urban Macro UL 5%tile user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%, FR2 UE antenna.

With FR2 UE type in Urban Macro UL, ACIR requirements cannot be derived as the UL 5%tile throughput loss plots are below 5%.

Figure 2.2.1-3: Urban Macro UL mean user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%, FR2 UE antenna.

### 2.2.2	FR1 UE type

In the following, results assuming FR1 UE antenna model are presented.

Figure 2.2.2-1 shows this Urban Macro scenario with ISD 450 m, is characterized by a high outage probability (around 10%), when indoor probability is 20%. As a result, for this case, the 5%tile user throughput is 0, and requirements based on UL 5%tile throughput loss cannot be derived.

It is necessary to locate all users outdoor to reach 5%tile user throughput statistics. The requirement in this case would be in the range of 16-18 dB. Similarly, as for the DL case, a higher number of AEs relaxes the requirement. The mean user throughput loss does not lead to any requirement.

To improve UL performance for the indoor 20% case, it is possible to study the case of ISD 350 m, or to improve the UE antenna capability with UE BF, and maximum transmission power of 26 dBm.

Figure 2.2.2-1: Urban Macro UL SINR for 1536 and 2048 AE, indoor probability 0% and 20%, FR1 UE antenna.

With FR1 UE type in Urban Macro UL, significantly high outage around 10% is observed assuming 20% Indoor users. Therefore, no requirements can be derived for such scenarios.

Urban Macro scenario UL coverage is challenged by the presence of indoor users, and requirements cannot be derived. For this, more studies should be conducted considering reduced ISD and/or higher maximum transmit power.

The mean user throughput loss and 5%tile throughput loss for the two BS antenna architectures with 1536 and 2048 AE, and for indoor probability 0% and 20% are shown in Figure 2.2.2-2 and Figure 2.2.2-3.

Figure 2.2.2-2: Urban Macro UL 5%tile user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%, FR1 UE antenna.

With FR1 UE type in Urban Macro UL, ACIR of approximate 16 to 18 dB is observed at Indoor probability of 0%.

Figure 2.2.2-3: Urban Macro UL mean user throughput loss, for 1536 and 2048 AE, indoor probability 0% and 20%, FR1 UE antenna.

## 2.3	Indoor Downlink

In the Indoor scenario, the following options have been assumed for simulation:

- UE max Transmission power = 23 dBm for both FR1 and FR2

- Antenna architectures: 4x4, aligned with the proposal included in our companion contributions [5,6]

- UE antenna: both options, FR1 and FR2 like options have been considered

- Diversity gain: 0 dB

- Number of active UEs is 1

In addition, we present simulations for both a 3-sector and 1-sector layouts, as described in TR 38.802 section A.2.1.

### 2.3.1	FR2 UE type

Figure 2.3.1-1 shows the DL SINR for the indoor scenario for FR2 UE type, both 1 sector and 3 sector deployments are considered. It can be observed that in both cases the coverage is good in this scenario.

Figure 2.3.1-1: Indoor DL SINR for 4x4 array, FR2 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.3.1-2: Indoor DL 5%tile and mean user throughput loss, for 4x4 array, FR2 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.3.1-2 shows the DL 5%tile and mean user throughout loss for both cases of 1 sector and 3 sector deployments, for FR2 UE antenna. The mean user throughout loss does not lead to any requirement, while for the DL 5%tile throughput loss we can observe similar requirements for the two layouts.

In both cases the requirements are quite low, so we propose to consider a 1 sector deployment for this scenario, since there is no need of more costly and complex deployments.

RAN4 to clarify how many sectors per node – 1 or 3 are considered for Indoor scenario deployments.

### 2.3.2	FR1 UE type

Figure 2.3.2-1 shows the DL SINR for the indoor scenario and FR1 UE type, both 1 sector and 3 sector deployments are considered. It can be observed that in both cases the coverage is good in this scenario.

Figure 2.3.2-1: Indoor DL SINR for 4x4 array, FR1 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.3.2-2: Indoor DL 5%tile and mean user throughput loss, for 4x4 array, FR1 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.3.2-2 shows the DL 5%tile and mean user throughout loss for both cases of 1 sector and 3 sector deployments, for FR1 UE antenna. The mean user throughout loss does not lead to any requirement, while for the DL 5%tile throughput loss we can observe that the requirements that we obtain are in the same magnitude.
Comparing the ACIR requirements in Indoor DL for 1 sector deployments based on FR2 and FR1 UE type behavior, It is observed that FR2 UE have high requirements as compared to FR1 UE. One reasoning could be that the internal interference is lower in FR2 UE due to better directionality of transmission and reception, however it is more sensitive to external interference. However, the overall requirements for both UE type are still low.

## 2.4	Indoor Uplink

### 2.4.1	FR2 UE type

Figure 2.4.1-1 shows the UL SINR for the indoor scenario and FR2 UE type, both 1 sector and 3 sector deployments are considered. It can be observed that in both cases the coverage is good in this scenario.

Figure 2.4.1-1: Indoor UL SINR for 4x4 array, FR2 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.4.1-2: Indoor UL 5%tile and mean user throughput loss, for 4x4 array, FR2 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.4.1-2 shows the UL 5%tile and mean user throughout loss for both cases of 1 sector and 3 sector deployments, for FR2 UE antenna. The mean user throughout loss does not lead to any requirement, while for the UL 5%tile throughput loss we can observe that the requirements that we obtain are similar for the two layouts.

In both cases the requirements are quite low, so we propose to consider a 1 sector deployment for this scenario, since there is no need of more costly and complex deployments.

### 2.4.2	FR1 UE type

Figure 2.4.2-1 shows the UL SINR for the indoor scenario and FR1 UE type, both 1 sector and 3 sector deployments are considered. It can be observed that in both cases the coverage is good in this scenario.

Figure 2.4.2-1: Indoor UL SINR for 4x4 array, FR1 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.4.2-2: Indoor UL SINR for 4x4 array, FR1 UE antenna, a) 1 sector, b) 3 sectors

Figure 2.4.2-2 shows the UL 5%tile and mean user throughout loss for both cases of 1 sector and 3 sector deployments, and FR1 UE antenna. The mean user throughout loss does not lead to any requirement, while for the UL 5%tile throughput loss we can observe that also in this scenario the requirements are low.

Comparing the ACIR requirements in Indoor DL for 1 sector deployments based on FR2 and FR1 UE type behavior, It is observed that FR2 UE have high requirements as compared to FR1 UE. One reasoning could be that the internal interference is lower in FR2 UE due to better directionality of transmission and reception, however it is more sensitive to external interference. However, the overall requirements are still low.

With FR2 and FR1 UE type in Indoor DL and UL, ACIR requirements are quite low for both 1 sector and 3 sector deployments.

# Conclusion

In the previous sections we made the following observations:

Observation 1	With FR2 UE type in Urban Macro DL, ACIR of approximate 21 dB and 17 dB is observed at Indoor probability of 0% and 20% respectively.

Observation 2	With FR1 UE type in Urban Macro DL, ACIR of approximate 26 dB and 22 dB is observed at Indoor probability of 0% and 20% respectively.

Observation 3	In Urban Macro Downlink scenario, FR2 UE antenna architecture leads to lower ACIR requirements at 20% Indoor users.

Observation 4	RAN4 should consider both worst-case scenarios as well as feasibility aspects of current semiconductor technologies when deciding ACLR/ ACS requirements.

Observation 5	With FR2 UE type in Urban Macro UL, ACIR requirements cannot be derived as the UL 5%tile throughput loss plots are below 5%.

Observation 6	With FR1 UE type in Urban Macro UL, significantly high outage around 10% is observed assuming 20% Indoor users. Therefore, no requirements can be derived for such scenarios.

Observation 7	Urban Macro scenario UL coverage is challenged by the presence of indoor users, and requirements cannot be derived. For this, more studies should be conducted considering reduced ISD and/or higher maximum transmit power.

Observation 8	With FR1 UE type in Urban Macro UL, ACIR of approximate 16 to 18 dB is observed at Indoor probability of 0%.

Observation 9	RAN4 to clarify how many sectors per node – 1 or 3 are considered for Indoor scenario deployments.

Observation 10	With FR2 and FR1 UE type in Indoor DL and UL, ACIR requirements are quite low for both 1 sector and 3 sector deployments.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2410741, “WF on IMT parameters study”, 3GPP RAN4#111, Ericsson

- R4-2410763, “3GPP TR 38.922 v01.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#111

- R4-2412128, “On Antenna feasibility and RF parameters for 14800 to 15350 MHz frequency range”, 3GPP RAN4#112, Ericsson

- R4-2412126, “On Co-existence simulation assumptions for 14800 to 15350 MHz frequency range”, 3GPP RAN4#112, Ericsson