3GPP TSG-RAN WG4 Meeting #112	R4-2412128

Maastricht, Netherlands, August 19-23, 2024

Agenda Item:	8.2.4.2

Source:	Ericsson

Title:	On Antenna feasibility and RF parameters for 14800 to 15350 MHz frequency range

Document for:	Discussion

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz.

In the previous 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] and TR 38.922 v0.1.0 [4] captured initial agreements on co-existence simulation assumptions

This contribution provides further discussion points on the feasibility of BS – UE antenna model and IMT parameters relevant for 14800 to 15350 MHz frequency range.

# 2	Discussion

## 2.1 	BS sub-array antenna size

### 2.1.1		Urban Macro scenario

In the previous RAN4 meeting, following BS array sizes were considered:

We consider that an 8x1 sub-array would be too large. A large sub-array would limit the coverage angular range (CAR) due to the narrow sub-array radiation pattern along the vertical axis. A narrow vertical range limits operation to deployment scenarios where UE only are spread out along the horizontal axis. In addition, a large sub-array will also introduce high transmission losses in the Radio Distribution Network (RDN) at 15 GHz. For sub-arrays larger than 4 elements the loss assumed to derive the peak element gain GE,max need to be reconsidered. As a result, we propose to consider sub-arrays of 6x1 or 4x1, while 8x1 would be more adequate for a rural deployment where UEs are on the same elevation plane.

The total number of array antenna elements is defined as:

NAE=2MNMsub

, where M is the number of sub-array rows, N is the number of sub-array columns, Msub is the size of the vertical sub-array. The factor 2 is to account for two supported polarizations (e.g., -45/+45 or V/H).

RAN4 to consider following sub-array structures: 6x1 and 4x1 for Urban Macro and drop 8x1 as a larger sub-array would limit the coverage angular range due to narrow sub-array radiation pattern in vertical axis.

### 2.1.2		Indoor Hotspot scenario

In [4], two options – 4x4 and 8x8 have been captured for Indoor scenarios. As per our pre-liminary simulation results presented in [6] assuming 4x4 array, it is observed that adequate coverage is achieved in both UL and DL scenarios. Thus, 8x8 array seems to be average large for such scenarios.

RAN4 to only consider 4x4 array for indoor office deployment.

## 2.2 	BS antenna feasibility

RAN4 in Release-16 studied the RF technology feasibility for the 7-24 GHz range documented in the report 3GPP TR 38.820 [5], provides in-depth analysis of power amplifiers based on different semi-conductor technologies. The study is essential to consider as it depicts the relation between ACLR and output power (dBm), for the relevant 14800 to 15350 MHz frequency range.

For 2048 AEs: 8x32 array and 4x1 sub-array,

Keeping that in mind, if for example we assume to aim for EIRP of 79.4 dBm per 100 MHz bandwidth and a conservative 2 dB transmission loss in RDN (probably higher transmission losses should be considered due to bigger sub-arrays assumed).

Antenna gain G (dBi) = 6.4 (Element gain in dBi) + 10*log10(8*32) + 10*log10(4*1) = 36.4

This will lead to, TRP (dBm) = EIRP – 36.4 = 79.4 – 36.4 = 43 dBm per polarization

Maximum output power per PA branch in dBm = TRP (both polarizations) – 10*log10(8*32*2) – 2 (transmission loss) = 46 – 27.09 – 2 = 16.9 dBm

For 1536 AEs: 8x16 array and 6x1 sub-array,

Keeping that in mind, if for example we assume to aim for EIRP of 78.1 dBm per 100 MHz bandwidth and a conservative 2 dB transmission loss in RDN (probably higher transmission losses should be considered due to bigger sub-arrays assumed).

Antenna gain G (dBi) = 6.4 (Element gain in dBi) + 10*log10(8*16) + 10*log10(6*1) = 35.1

This will lead to, TRP (dBm) = EIRP – 35.1 = 78.1 – 35.1 = 43 dBm per polarization

Maximum output power per PA branch in dBm = TRP (both polarizations) – 10*log10(8*16*2) – 2 (transmission loss) = 46 – 24.08 - 2 = 19.92 dBm

In the clause 5.4.3 stated in TR 38.820 [5], the study describes the current technological capability for different semi-conductor technologies to achieve a certain output power level at a prescribed ACLR level for different power amplifiers (PA). Figure 1 taken from clause 5.4.3 [5], shows the results for relation between output power and ACLR for 15 GHz.

If the assumption is 35 dB ACLR is required for 15 GHz, as per the latest findings from Figure 1 this accounts for:

- 20 dBm output power per PA branch for CMOS PA at 15 GHz

- 32 dBm output power per PA branch for GaN PA at 15 GHz

Figure 1: ACLR versus output power at 10, 15, and 20 GHz example frequencies, and 30 GHz based on 30 GHz PA model for reference [5]

RAN4 need to evaluate feasibility of current technology capabilities for output power per branch, ACLR and efficiency for power amplifiers based on available semiconductor technologies.

Based on the information in TR 38.820 and feasibility calculations presented in this section for array architecture, it is reasonable to assume that we may end up with CMOS and GaN PA technologies. However, there is a need to further investigate antenna feasibility aspects.

RAN4 to consider following BS array antenna architectures:

- 2048 AEs: 8x32 array and 4x1 sub-array

- 1536 AEs: 8x16 and 6x1 sub-array

## 2.3	UE antenna - FR1 vs FR2 type

For the UE antenna, it was agreed to consider the below two options for simulation purposes.

### FR1 like option:

In this approach, a UE will most likely have a conducted interface with an assumed isotropic radiation pattern antenna with [0 or 5] dB receive diversity gain and no beamforming.

### FR2 like option:

In this case, we assume UE beamforming, and the UE antenna element patter and array structure are described in Table 2.3-1.

Table 2.3-1: UE antenna element pattern and array structure

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

Further inputs are needed from UE vendors on feasibility aspects of both FR1 and FR2 UE type for a reasonable UE architecture.

## 2.3 	BS parameters

### 2.3.1	Transmitter characteristics

#### Power dynamic range

The power dynamic range requirement includes RE power dynamic range and total power dynamic range for BS. Keeping co-existence simulation study in perspective, BS is always in full RB transmission mode. It depends on whether power control is allowed for DL transmission. As we know for NR, there is no power control in DL so the power is allocated on average to the number of RBs. Hence, we propose for 0 dB value for this parameter.

RAN4 to agree on 0 dB as power dynamic range.

#### Spectral Mask

The spectral mask should be further discussed once ACLR is agreed based on system simulation results.

#### Spurious Emissions

For spurious emissions use limits in TR 38.921 for 10 to 10.5 GHz as baseline. According to ERC 74-01 report the upper frequency limit could be defined as 2nd harmonic.

Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

RAN4 to consider BS spurious emission limits in Table 2.2.3-1 and 2.2.3-2 as baseline for initial discussions.

#### Maximum output power and Average output power

There does not seem to be any difference between the two parameters, as Maximum output power is already an average power.

There is no upper limit specified for Wide Area Base Station, this shall be declared by the manufacturer. However, the BS maximum output power should be aligned with the antenna characteristics.

For BS operating within FR1 the maximum TRP output power values (specified in TS 38.104 [7]) for Medium Range and Local Area BS are:

- Medium Range: 47 dBm

- Local area: 33 dBm

### 2.3.2	Receiver characteristics

#### Noise Figure

Based on information in TR 38.820 and TR 38.921 the noise figure values in Table 2.2.3.1-1 can be considered.

Table 2.2.3.1-1: Typical noise figure values relevant for 14800 to 15350 MHz

| BS class | Noise figure (dB) |
| --- | --- |
| Wide Area | 8 |
| Medium Range | 13 |
| Local Area | 16 |

RAN4 to consider BS NF as 8 dB for WA BS, 13 dB for MR BS and 16 dB for LA BS respectively based on information in TR 38.820 and TR 38.921.

#### Sensitivity

In the TR 38.921, it has been specified that sensitivity is not a critical parameter for sharing and compatibility studies and “to be specified” in the previous LS sent for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 GHz.

The sensitivity requirement is calculated based on NF and antenna gain, as per below -

EISREFSENS=PkT+10*log10BW+NF+IM+SNR-G    (dBm)

Where:

-	BW is the configured bandwidth of the FRC,

-	NF is the noise figure,

-	IM is implementation margin not related to antenna array,

-	SNR is the required SNR to reach 95% throughput, and

-	G is the antenna gain including RF losses and 3dB off peak margin.

The sensitivity limits can be further discussed based on the agreements of BS NF and Antenna model, if should be agreed to mention any value for this parameter.

In the TR 38.921, it has been specified that sensitivity is not a critical parameter for sharing and compatibility studies and “to be specified” in the previous LS sent for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 GHz.

RAN4 to further discuss based on the agreements of BS NF and Antenna model, if should be agreed to mention any value for this parameter.

## 2.4	UE parameters

### 2.4.1	Transmitter characteristics

#### Power dynamic range

For the UE in sharing and compatibility studies, power dynamic range sets the range for Transmitter control output power. In TR 38.921, PC3 UEs were assumed for 6.425-7.025 GHz, 7.025-7.125 GHz, and 10.0-10.5 GHz study. From TS 38.101-1 [8], PC3 UEs have 23 dBm as Maximum output power and -33 dBm for 100 MHz Channel BW, therefore UE power dynamic range of 56 dB for 100 MHz can be considered as starting point.

#### Spectral Mask

The spectral mask should be further discussed once ACLR is agreed based on system simulation results.

#### Spurious Emissions

From TS 38.101-1 [8], the general spurious can be defined as per clause 6.5.3.1 for this frequency range.

-	1 GHz ≤ f ≤ 26 GHz: - 30 dBm / 1 MHz

RAN4 to consider UE spurious emission limits from TS 38.101-1, - 30 dBm / 1 MHz as baseline for initial discussions.

#### Maximum output power and Average output power

The UE maximum output power can be considered as 23 dBm as a starting point. Several other studies have been carried out for adoption of High power UEs, where the problem of UL coverage can be solved for these frequency ranges.

High power UEs adoption can be further studied to solve the problem of UL coverage for these frequency ranges.

RAN4 to consider as 23 dBm UE maximum output power as starting point.

### 2.4.2	Receiver characteristics

#### Noise Figure

From TR 38.820, subclause 5.5.1.3.3 a system noise figure of 10 dB can be considered for the frequency range 14800 to 15350 MHz.

RAN4 to consider UE NF as 8 dB based on information in TR 38.820.

#### Sensitivity

As per our proposal for BS Sensitivity, the UE sensitivity limits could be further discussed based on the agreements of UE NF and Antenna model, if should be agreed to mention any value for this parameter.

RAN4 to further discuss based on the agreements of UE NF and Antenna model, if should be agreed to mention any value for this parameter.

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN4 need to evaluate feasibility of current technology capabilities for output power per branch, ACLR and efficiency for power amplifiers based on available semiconductor technologies.

Observation 2	Further inputs are needed from UE vendors on feasibility aspects of both FR1 and FR2 UE type for a reasonable UE architecture.

Observation 3	In the TR 38.921, it has been specified that sensitivity is not a critical parameter for sharing and compatibility studies and “to be specified” in the previous LS sent for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 GHz.

Observation 4	High power UEs adoption can be further studied to solve the problem of UL coverage for these frequency ranges.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to consider following sub-array structures: 6x1 and 4x1 for Urban Macro and drop 8x1 as a larger sub-array would limit the coverage angular range due to narrow sub-array radiation pattern in vertical axis.

Proposal 2	RAN4 to only consider 4x4 array for indoor office deployment.

Proposal 3	RAN4 to consider following BS array antenna architectures:

1.	2048 AEs: 8x32 array and 4x1 sub-array

2.	1536 AEs: 8x16 and 6x1 sub-array

Proposal 4	RAN4 to agree on 0 dB as power dynamic range.

Proposal 5	RAN4 to consider BS spurious emission limits in Table 2.2.3-1 and 2.2.3-2 as baseline for initial discussions.

Proposal 6	RAN4 to consider BS NF as 8 dB for WA BS, 13 dB for MR BS and 16 dB for LA BS respectively based on information in TR 38.820 and TR 38.921.

Proposal 7	RAN4 to further discuss based on the agreements of BS NF and Antenna model, if should be agreed to mention any value for this parameter.

Proposal 8	RAN4 to consider UE spurious emission limits from TS 38.101-1, - 30 dBm / 1 MHz as baseline for initial discussions.

Proposal 9	RAN4 to consider as 23 dBm UE maximum output power as starting point.

Proposal 10	RAN4 to consider UE NF as 8 dB based on information in TR 38.820.

Proposal 11	RAN4 to further discuss based on the agreements of UE NF and Antenna model, if should be agreed to mention any value for this parameter.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2410741, “WF on IMT parameters study”, 3GPP RAN4#111, Ericsson

- R4-2410763, “3GPP TR 38.922 v01.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#111

- 3GPP TR 38.820 v16.1.0 (2021-03), “Study on 7-24 GHz frequency range (Release 16)”.

- R4-2412127, “On Urban Macro and Indoor Co-existence simulation results for 14800 to 15350 MHz frequency range”, 3GPP RAN4#112, Ericsson

- R4-2412126, “On Co-existence simulation assumptions for 14800 to 15350 MHz frequency range”, 3GPP RAN4#112, Ericsson