3GPP TSG-RAN WG4 Meeting # 112	R4-2412130

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.17.4

Source: 	National Taiwan University

Title: 	On AI/ML Interoperability and Standardization of Testing and Verification for CSI Use Case

Document for:	Approval

# Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

# Discussion

## New Proposal for Partially Specified Decoder

Based on the summary provided in Table 3 in [1], the options proposed to derive/specify partially specified decoder rely on a common dataset and/or a reference encoder/decoder pair. Those approaches can possibly achieve test repeatability, which is very important for standardized test and can provide good guidance for interoperability, but they also impose many limitations on encoder/decoder implementation since the training/design of the test decoder or DUT encoder is based on the reference encoder/decoder pair (or the data generated by the pair).

Such limitation may reduce the potential gain of using AI/ML models, or implicitly leads to a specialized DUT encoder for RAN4 test that deviates a lot from the encoders used in the field. Therefore, we want to carefully examine the mathematical structure of the CSI encoding/decoding function, and propose a new approach for partially specified decoder with much lower limitation on test decoder and DUT encoder limitation.

When we consider the complete CSI information, e.g, the precoder from SVD, as the input to the encoder, the encoder is a mapping between the precoder (from a real-value vector space) and the corresponding latent message (from a finite set of numbers); and the decoder is a mapping between the latent message (from a finite set of numbers) to the recovered precoder (from a real-value vector space but there are finite number of recovered precoders, limited by the size of the latent message set). When the encoder and decoder are jointly trained or trained by a common dataset, at least within the train dataset, we expect a recovered precoder is very close to the expectation of the subspace of the precoder that is mapped to the latent message corresponding to the recovered precoder. However, when the decoder and encoder are trained by different entities, i.e., we get the encoder from a jointly trained pair (pair A), and get the decoder from another pair (pair B), these pairs may not have aligned precoder subspaces to generate the latent message by the encoder. Therefore, even each pair have the optimal recovered precoder for each of their precoder subspace, the recovered precoder from the decoder in pair B may not be the optimal recovered precoder when the latent message is encoded by the encoder in pair A since the subspaces in pair A and B are different. The even worse scenario is that the decoder may map the latent message to a recovered precoder very far away from the subspace of precoders mapped to the latent message by the encoder if the same latent message are mapped to two very far apart subspaces by the two encoders (from pair A and B).

We illustrate the two types of errors in the following figure. When the pairs are jointly trained, like ENC-A and DEC-A, ENC-B and DEC-B, ENC-C and DEC-C, we expect the recovered precoder is very close to the (probability weighted) center of the subspace of the precoder linked by the corresponding latent message (sub-figure 1, 2, 3). However, when we connect the encoders and decoders trained separately, as explained above, the recovered messages are not ideal. The first type of error due to subspace boundary misalignment from different models are illustrated in sub-figure 4, when we connect ENC-A to DEC-B. We can see that the recovered precoder (e.g., the dark red dot) is no longer at the probability center of the precoder subspace linked by the corresponding latent message (e.g., the dark green circle), but is still very close to it and might be within the precoder subspace. However, the second type of error due to mapping order difference to the latent message, as illustrated in sub-figure 5 (connecting ENC-A to DEC-C), is very harmful. We can observe that the dark green precoder subspace is mapped to the light yellow recovered precoder through the dark blue latent message instead of the dark yellow recovered precoder, which is much closer to the dark green precoder subspace. Now the recovered precoder is very far away from the precoder space it is representing, and therefore we expect to have a very bad performance. If we can ensure that DEC-A and DEC-C are (to some extent) consistent: any latent message is mapped to close by recovered precoders by DEC-A and DEC-C, we can prevent the second type of error.

ENC-A

DEC-D

5

ENC-A

DEC-B

4

ENC-C

DEC-C

3

ENC-B

DEC-B

2

ENC-A

DEC-A

1

In our proposal, we want to first resolve the second issue illustrated in the previous paragraph: even if different pairs trained separately may have subspaces that are not completely aligned, we want to propose a set of decoder specifications that can guarantee nearby subspaces from different pairs mapping to the same latent message. Therefore, we can eliminate the mismatch due to far apart subspaces mapped to the same latent message, and the residual mismatch are only from subspace mismatches between different pairs.

We first formulate the mathematical problem corresponding to the above requirement in the following. First, we assume the encoder input space X is common across all the encoders, and use the following mapping to describe two encoder and decoder pairs:

Pair 0:X→Z:Z=z0,…,zk, X0=x0,0,…,x0,k

Pair 1:X→Z:Zz0,…,zk,X1=x1,0,…,x1,k

Note that zm→x0,m in pair 0 and zm→x1,m in pair 1.

We want to have any two pairs of encoder and decoder satisfy the following property: for any z∈Z, denote decoder output of pair 0: z→x0 and pair 1: z→x1, the decoders has to satisfy: x0-x1=minx∈X1x0-x. To fit in the test decoder standardization framework, we can consider pair 0 as the reference encoder/decoder pair, and we require the pair 1 (with the test decoder) satisfying the constraint listed above.

We provide a novel approach below to guide the derivation of the test decoder satisfying the above constraints. We can start with a reference decoder, or any mapping between latent messages and the recovered precoders, e.g., e-Type II precoding can be an option. Denote this mapping by

Z→Xref, Z:z0,…,zk, Xref:xref,0,…,xref,k

Any test decoder, Z: z0,…,zk→X: x0,…,xk, has to follow the constraints below

x0=argminXxref,0-x

x1=argminX\x0xref,1-x…

xk-1=argminX\x0,…,xk-2xref,k-1-x

xk-1=X\x0,…,xk-2,xk-1

The above approach can be considered as a greedy algorithm to sort a achieve the proposed test decoder requirements: x0 is guaranteed to satisfy the proposed requirement, but if x0=minXxref,1-x, we can’t guarantee that x1 satisfies the proposed requirements. Based on the above observation, we can conclude that the greedy algorithm can satisfy the requirements under an additional constraint:

argminx∈X1xn-x≠argminx∈X1xm-x

For any n≠m.

Proposal 1: Study the feasibility of the partial specified test decoder with partial mapping alignment

For any latent message z∈Z, where Z is the latent message space, denote the decoder output of the reference enc/dec pair as xref(z) and the decoder output of the test decoder (from another pair) as xtest(z), the test decoders has to satisfy: xref(z)-xtest(z)=minx∈Xtestxref(z)-x, where Xtest is the space of the test decoder output.

The above approach, if proven to be feasible, has the following advantages:

Observation 1: The advantages of partial mapping alignment approach for partial specified test decoder option are:

- A standardized training data free approach: 3GPP standard doesn’t have to capture the training data.

- If the reference decoder can be described analytically with a set of equations, e.g., e-Type II CSI, RAN4 doesn’t have to agree a reference encoder/decoder pair to derive the test decoders.

- Best forward compatible: either dataset or reference encoder/decoder pair approaches require UE vendors to train the decoder based on the encoder or dataset from a specific model designed based on the currently available AI/ML model structures. When new and superior models are developed by AI research communities, the dataset or reference encoder may limit the implementation flexibility of the UE vendors to adopt the new models.

Since we don’t require the test decoder output matching exactly to the reference decoder output, we need take the mismatches between the test decoder, the reference decoder, and the decoder UE vendors training their encoders into consideration. However, the constraint in proposal 1 significantly reduces the mismatches between the decoders by ensuring that the decoder output mapped from the same latent message are the closest within the decoder output space.

Proposal 2: Additional margin to account for the mismatch xref(z)-xtest(z)=minx∈Xtestxref(z)-x need to be considered when determining requirement.

## Test Decoder Derivation Procedure

In RAN4 #111, the group agrees the following for the reference decoder derivation discussion for study phase [2]:

Parameters agreed are just for the feasibility study of testing options. If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

However, we don’t understand how agreeing a reference decoder structure that may not be used in the work item stage can prove the feasibility of the CSI use case from the interoperability and verification perspectives. Since companies are agreeing on a reference decoder under the assumption that it may not go into specification, we are not sure if companies still can agree on a reference decoder when the reference decoder will be captured in the specification. Therefore, we have the following proposal to move forward.

Proposal 3: RAN4 either agrees multiple options of test decoder model structures and other parameters, or agrees a reference decoder derivation procedure for work item phase, in the study phase to demonstrate the feasibility of CSI use case from the interoperability and verification perspectives.

In fact, many companies have good proposals for decoder derivation procedure, we summarize them in the following, by first list out the steps and cites the related contributions (from Apple[3], Vivo[4],Samsung[5],Oppo[6]), and provide summary of open issues and suggested WF based on all the companies’ contributions.

- Step 1: Determine simulation setup and encoder input dataset generation procedure

Apple (R4-2407236): For generating data for training the pair of UE encoder/Test Decoder, use a system-level simulator. The proposed baseline system simulation assumptions are shown in Table 2. (partial quote for the relevant part) Build a database by agreeing channel model(s) and aggregate data from UE vendors.

Vivo (R4-2408294): align the dataset containing only channel information, by agreeing simulation assumptions and data sample format etc. Multiple dataset can be merged

Samsung (R4-2405653): standardization procedure start: consider a certain use case, e.g., CSI compression under certain configuration

Oppo (R4-2405187): determine test condition; determine test data, considering generation method, collect/merg data from different companies

  - Summary:

    - Open issue for simulation setup:

- System level or link level simulation for determining test decoder

- Using setup in TR as starting points, what are the parameters need to be updated

    - Open issue for data collection/merge: whether RAN4 needs to collect and merge encoder input generated by different companies

- Step 2: Determine model type and parameter

Apple: identify model architectural parameters for test decoder, e.g., backbone type, model depth/width, layer type/size, quantization

Vivo: determine the model hyperparameters that need to be aligned

Samsung: identify necessary model architecture parameters

Oppo: determine the test model structure, how to setup and merge into one

Summary: WF in RAN4#110 (R4-2403712) can be used as a starting point for WI discussion on which parameters to be agreed in this step

  - Summary of open issues:

- Whether further update is needed for the table in R4-2403712

- Whether multiple options can be listed as the outcome of this step, if RAN4 experiences difficulties to pick one before evaluation step: companies can pick their favorite models to come up with their proposals for evaluation in step 4

- Step 3: Determine training hyper parameters

Apple: specify training model parameters (loss function, hyper-parameters, regularization 	etc). Build a dataset (from step 1) with encoder output

Samsung: identify necessary model training parameters (training procedure, loss function etc)

Qualcomm: remove training procedure from “the parameter needed to be agreed”

  - Summary of open issues:

- whether RAN4 needs to agree training related parameters, and if needed, what parameters to be agreed

- Whether RAN4 needs to agree on dataset including encoder output

(After step 3, companies can train and propose (encoder and) decoder)

- Step 4: Determine the test decoder(s) by first determine evaluation methodology and selection/merge criterion, and then decide the test decoder(s) based on companies’ evaluation

Apple: Each vendor evaluates their own encoder/decoder pair. Performance alignment between companies (based on metrics, e.g., NMSE, SGCS etc). RAN4 agrees on test decoder.

Vivo: Define the evaluation method for model complexity and performance. Select the best structure based on the agreed evaluation method and dataset. Select or merge the test 	decoders derived from the best structure from company proposals.

Samsung: performance comparison based on different companies’ proposals, and based on 	that RAN4 agree on the (fully specified) test decoder.

Oppo: determine the test model parameter: based on reference dataset and the agreed structure. Consider the possibility of merge difference parameter proposals.

  - Summary:

    - Open issue on evaluation method and criterion: whether to consider one more factors listed in the following

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

    - Open issue on selecting and merging the decoder proposals: for each testing scenario, whether RAN4 directly selects one decoder proposal from all the proposals (after step 3) or RAN4 can merge parameters from multiple proposals with the same structure into on decoder based on further studies.

    - Open issue on iterations: if RAN4 can’t agree or merge proposals to one decoder, whether RAN4 goes back to step 2/3 to agree more model/training parameters and repeat step 4.

Proposal 4: RAN4 uses the following steps, summarized from contribution [3,4,5,6] to derive the reference decoder (and encoder) in the WI phase and captures them in the TR. The open issues listed in each of the steps can be discussed in study or work item phase.

- Step 1: Determine simulation setup and encoder input dataset generation procedure

  - Summary:

    - Open issue for simulation setup:

- System level or link level simulation for determining test decoder

- Using setup in TR as starting points, what are the parameters need to be updated

    - Open issue for data collection/merge: whether RAN4 needs to collect and merge encoder input generated by different companies

- Step 2: Determine model type and parameter

  - Summary of open issues:

- Whether further update is needed for the table in R4-2403712

- Whether multiple options can be listed as the outcome of this step, if RAN4 experiences difficulties to pick one before evaluation step: companies can pick their favorite models to come up with their proposals for evaluation in step 4

- Step 3: Determine training hyper parameters

  - Summary of open issues:

- whether RAN4 needs to agree training related parameters, and if needed, what parameters to be agreed

- Whether RAN4 needs to agree on dataset including encoder output

(After step 3, companies can train and propose (encoder and) decoder)

- Step 4: Determine the test decoder(s) by first determine evaluation methodology and selection/merge criterion, and then decide the test decoder(s) based on companies’ evaluation

  - Summary:

    - Open issue on evaluation method and criterion: whether to consider one more factors listed in the following

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

    - Open issue on selecting and merging the decoder proposals: for each testing scenario, whether RAN4 directly selects one decoder proposal from all the proposals (after step 3) or RAN4 can merge parameters from multiple proposals with the same structure into on decoder based on further studies.

    - Open issue on iterations: if RAN4 can’t agree or merge proposals to one decoder, whether RAN4 goes back to step 2/3 to agree more model/training parameters and repeat step 4.

# Conclusion

Proposal 1: Study the feasibility of the partial specified test decoder with partial mapping alignment

For any latent message z∈Z, where Z is the latent message space, denote the decoder output of the reference enc/dec pair as xref(z) and the decoder output of the test decoder (from another pair) as xtest(z), the test decoders has to satisfy: xref(z)-xtest(z)=minx∈Xtestxref(z)-x, where Xtest is the space of the test decoder output.

Observation 1: The advantages of partial mapping alignment approach for partial specified test decoder option are:

- A standardized training data free approach: 3GPP standard doesn’t have to capture the training data.

- If the reference decoder can be described analytically with a set of equations, e.g., e-Type II CSI, RAN4 doesn’t have to agree a reference encoder/decoder pair to derive the test decoders.

- Best forward compatible: either dataset or reference encoder/decoder pair approaches require UE vendors to train the decoder based on the encoder or dataset from a specific model designed based on the currently available AI/ML model structures. When new and superior models are developed by AI research communities, the dataset or reference encoder may limit the implementation flexibility of the UE vendors to adopt the new models.

Proposal 2: Additional margin to account for the mismatch xref(z)-xtest(z)=minx∈Xtestxref(z)-x need to be considered when determining requirement.

Proposal 3: RAN4 either agrees multiple options of test decoder model structures and other parameters, or agrees a reference decoder derivation procedure for work item phase, in the study phase to demonstrate the feasibility of CSI use case from the interoperability and verification perspectives.

Proposal 4: RAN4 uses the following steps, summarized from contribution [3,4,5,6] to derive the reference decoder (and encoder) in the WI phase and captures them in the TR. The open issues listed in each of the steps can be discussed in study or work item phase.

- Step 1: Determine simulation setup and encoder input dataset generation procedure

  - Summary:

    - Open issue for simulation setup:

- System level or link level simulation for determining test decoder

- Using setup in TR as starting points, what are the parameters need to be updated

    - Open issue for data collection/merge: whether RAN4 needs to collect and merge encoder input generated by different companies

- Step 2: Determine model type and parameter

  - Summary of open issues:

- Whether further update is needed for the table in R4-2403712

- Whether multiple options can be listed as the outcome of this step, if RAN4 experiences difficulties to pick one before evaluation step: companies can pick their favorite models to come up with their proposals for evaluation in step 4

- Step 3: Determine training hyper parameters

  - Summary of open issues:

- whether RAN4 needs to agree training related parameters, and if needed, what parameters to be agreed

- Whether RAN4 needs to agree on dataset including encoder output

(After step 3, companies can train and propose (encoder and) decoder)

- Step 4: Determine the test decoder(s) by first determine evaluation methodology and selection/merge criterion, and then decide the test decoder(s) based on companies’ evaluation

  - Summary:

    - Open issue on evaluation method and criterion: whether to consider one more factors listed in the following

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

    - Open issue on selecting and merging the decoder proposals: for each testing scenario, whether RAN4 directly selects one decoder proposal from all the proposals (after step 3) or RAN4 can merge parameters from multiple proposals with the same structure into on decoder based on further studies.

    - Open issue on iterations: if RAN4 can’t agree or merge proposals to one decoder, whether RAN4 goes back to step 2/3 to agree more model/training parameters and repeat step 4.

# Reference

[1] R4-2407334

[2] R4-2410570

[3] R4-2407236

[4] R4-2408294

[5] R4-2405653

[6] R4-2405187