3GPP TSG-RAN WG4 Meeting # 112															R4-2412135

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.18.2

Source:	Samsung

Title:	Views on UE RF impact of NR MIMO Phase 5

Document for:	Discussion

1	Introduction

The latest WID on Rel-19 NR MIMO Phase 5 includes following objectives in the WID [1]. RAN4 is mentioned to specify necessary core requirements for the enhancements.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

As highlighted above, from our point of view, the WID includes following objectives which may need UE RF discussions and requirements:

- Non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions (3Tx)

- Asymmetric DL sTRP/UL mTRP deployment scenarios

Given that RAN1 discussions are still ongoing, the potential impact on the UE RF requirements shall be further evaluated until the RAN1 design is clear except some objectives, which have clear RAN4 impacts and sufficient progress. In this respect, we would like to focus on the topic of non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions (3Tx) in this contribution.

2	Discussion

2.1	RAN1 agreements (3Tx)

For 3Tx UL, RAN1 has reached following agreements for their last three meetings.

RAN1#116 Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for single-layer transmission. 13100,13010,13001 Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for two-layer transmission. 13100100, 13100001, 13 001001 Agreement For non-coherent uplink precoding by a 3TX UE, at least following precoders are supported for three-layer transmission. 13100010001 Agreement For codebook-based uplink transmission by a 3TX UE, support full-power Mode 0, subject to UE capability. Conclusion There is no consensus in RAN1 to support antenna switching for 3TX UE in Rel-19 Agreement For SRS configuration supporting codebook-based UL transmission by a 3TX UE, one SRS resource set is configured for single TRP operation.

RAN1#116-bis Agreement To support codebook-based UL transmission by a 3TX UE, the agreed rank1 precoders in RAN1#116 can also be used when transform precoding is enabled (DFT-s-OFDM ). Agreement To indicate precoding information for codebook-based UL transmission by a 3TX UE, - Reuse legacy TPMI indication framework where TPMI and TRI are jointly indicated - TPMI field is 2 or 3bits for 3-antenna-port transmission - For maxRank equals to 1, TPMI field is 2 bits for DFT-s-OFDM and CP-OFDM - For maxRank equals to 2 or 3, TPMI field is 3 bits for CP-OFDM Agreement - For codebook-based UL transmission by a 3TX UE, when 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows: Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. - Note: PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT_RS port 0, and PUSCH antenna port 1001 is associated with PT_RS port 1 - Number of bits used for the indication - 1 bit Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, - Option 3: Always a same port is muted, e.g., the 4th port Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, UE splits a linear SRS power equally across the 3 unmuted antenna ports of the 4-port SRS resource. Agreement For 3-port codebook-based PUSCH transmission for a 3TX UE, scale factor s should be the ratio of the number of antenna ports with a non-zero PUSCH transmission power to 3 (except for full-power Mode 0). - FFS: Whether specification needs to be updated to reflect the above Agreement For codebook-based UL transmission by a 3TX UE, when 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, PTRS-DMRS association indication is as follows: - Alt2: 2-bit indication PTRS-DMRS association when 1 PT-RS port is configured Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port Reserved. Agreement For a 3TX UE, support Rel-17 M-TRP PUSCH repetition where, - Two SRS resource sets, each with up to 2 of 4-port SRS resources are configured, Note: The configured 4 port SRS resources are used to enable 3-port SRS transmission

RAN1#117 Agreement Update the agreement made in RAN1 #116bis as the following. Agreement For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, Option 3: Always a same port is muted, e.g., i.e., the 4th port. Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2, - Reuse Rel-17 M-TRP PUSCH repetition design, where the second precoding information field only indicates TPMI index, and applies same rank as indicated by the first precoding information field. Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, scheduled by DCI format 0_1/0_2, - Introduce new tables as Table I, II, III for the second precoding information field, for maxRank=1 or 2 or 3, respectively. - Table I: Second precoding information for 3 antenna ports if maxRank=1 Bit field; codebookSubset=NonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; Reserved. - Table II: Second precoding information for 3 antenna ports if maxRank=2 Bit field; codebookSubset=NonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; 1 layer: Reserved. 0; 2 layer: TPMI=0. 1; 2 layer: TPMI=1. 2; 2 layer: TPMI=2. 3; 2 layer: Reserved. - Table III: Second precoding information for 3 antenna ports if maxRank=3 Bit field; codebookSubset=NonCoherent. 0; 1 layer: TPMI=0. 1; 1 layer: TPMI=1. 2; 1 layer: TPMI=2. 3; 1 layer: Reserved. 0; 2 layer: TPMI=0. 1; 2 layer: TPMI=1. 2; 2 layer: TPMI=2. 3; 2 layer: Reserved. 0; 3 layer: TPMI=0. 1~3; 3 layer: reserved. Agreement For codebook-based M-TRP PUSCH repetition by a 3TX UE, for indication of PTRS-DMRS association, - When 1 PTRS port is configured by maxNrofPorts in PTRS-UplinkConfig, reuse Rel-17 multi-TRP TDM repetition, - When 2 PTRS ports are configured by maxNrofPorts in PTRS-UplinkConfig, and maxRank = 2 or 3, - A second PTRS-DMRS association field (1 bit) is used to indicate the association between PTRS ports and DMRS ports for the 2nd SRS resource set (i.e.,2nd TRP). Agreement For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

According to the summary of RAN1 agreements above, it shows that the 3Tx discussion is on the fair way toward the current goal in the WID [1]. Also, given that RAN4 has limited TUs to specify all relevant requirements within the WI period, it is reasonable to initiate RAN4 discussion with the 3Tx UE RF requirements, which are the first try for RAN4 to discuss 3Tx on a single carrier for handheld devices.

Observation 1:	3Tx discussion is on the fair way toward the current goal in the WID in RAN1.

Proposal 1:		RAN4 should first initiate the discussion with the 3Tx UE RF requirements

2.2	RAN4 backgrounds

In Rel-18, RAN4 had a WI on 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC [2]. For the 3Tx in inter-band UL CA and EN-DC, the targeted scenario was 1Tx in one band and 2Tx in the other band. The corresponding requirements included inter-band UL CA with UL MIMO or TxD, and inter-band EN-DC with UL MIMO or TxD. Meanwhile, new per FS TxD capability was introduced.

Observation 2:	RAN4 had a WI on 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC.

Also, as a Rel-18 WI on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1), RAN4 discussed requirements to enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [3]. UE power class was PC1.5, and possible UE PA configurations assumed during the discussion were 4x23 dBm, 2x23 dBm + 2x26 dBm and 4x26 dBm. The main requirements included MPR, Pcmax,c tolerance, MIMO configurations including those for ULFPTx with 4 layers and coherent UL MIMO for 4Tx. In addition, new per FS TxD capability was introduced for 4Tx, which is the indication for UE to apply the Tx diversity requirements.

Observation 3:	RAN4 discussed requirements to enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices.

As RAN4 has such valuable experiences such as 3Tx (two bands) and 4Tx (CPE/FWA/vehicle/industrial devices) as described above, in our view, RAN4 should take the results of previous studies for Rel-19 3Tx (single band) discussions as much as possible.

Proposal 2:		RAN4 should take the results of previous 3Tx (two bands) and 4Tx (CPE/FWA/vehicle/industrial devices) studies for Rel-19 3Tx (single band) discussions as much as possible.

2.3	Work scope for UE RF

Regarding the work scope that must precede to start the 3Tx discussion, the WID can be an important guidance on what RAN4 should assume during the discussion [1]. RAN4 can first draw some basic assumptions from the justification part.

“NR supports 1-port, 2-port, 4-port and 8-port PUSCH. Nonetheless, current commercial mobile UEs are generally equipped with only 1 or 2 Tx antennas. To enhance the UL performance and anticipating advancements in hardware and design technology, the emergence of UEs equipped with 3Tx antennas is a foreseeable development in the near future. Consequently, it is advantageous to augment the NR standards to support 3-port PUSCH supporting up to 3 layers.”

As highlighted, it can be simply considered that the target device is not only FWA (larger form factor) devices, but also handheld (small form factor) devices from the fact that “the current commercial mobile UEs are generally equipped with only 1 or 2 Tx antennas”. It also can be inferred that RAN4 can concentrate on FR1 requirements based on our initial discussion on STxMP in Rel-18 and market demands.

Observation 4:	RAN4 can first draw some basic assumptions from the justification part of the WID.

Proposal 3:		Target devices of the WI are not only FWA (larger form factor) devices, but also handheld (small form factor) devices.

Proposal 4:		RAN4 can concentrate on FR1 requirements based on our initial discussion on STxMP in Rel-18 and market demands.

Regarding other technical assumptions, which are basic assumptions applied also to precedent RAN1 discussion, the WID has the clear guideline in its objective as below [1].

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported.

As marked in yellow, the first assumption to consider for UE RF requirements would be non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions. That might mean each antenna port can represent single MIMO layer, and it would be up to 3 layers for 3-antenna-port codebook-based transmissions, which also says that there is no enhancement on non-codebook PUSCH with 3Tx. Also, there would be no change for 3Tx on the existing SRS resource and UL full power transmission mode 1 and 2.

Observation 5:	WI considers non coherent UL codebook only (1-port/layer up to 3 layers).

Observation 6:	There is no enhancement on NCB PUSCH with 3Tx

Observation 7:	There is no enhancement on SRS resource

Observation 8:	UL full power transmission mode 0 (full power mode) is considered only

2.4	Power class and PA configurations

In order to specify necessary core requirements, UE architecture assumptions should be discussed to support 3Tx UL on single band. Figure 1 and Table 1 show every possible scenario of PA configuration and candidate power class to support the 3-antenna-port transmission, respectively.

Figure 1: Possible PA configurations for 3Tx

Table 1: Applicable power classes with PA configurations

| PA configuration | Achievable power level | Candidate power class |
| --- | --- | --- |
| 3x23 dBm | 27.8 dBm | PC3, PC2 |
| 1x23 + 2x26 dBm | 29 dBm | PC2, PC1.5 |
| 2x23 + 1x26 dBm | 30 dBm | PC2, PC1.5 |
| 3x26 dBm | 31 dBm | PC1.5 |

As shown in Table 1, for the single carrier requirements with 3Tx, RAN4 first needs discussions for the baseline assumptions such as the relationship between PA configurations and applicable power classes, and the maximum power class/level. In our view, based on the previous 4Tx discussion in Rel-18, RAN4 can consider a single PA configuration for each power class to specify the UE RF requirements of 3Tx as an initial stage.

Observation 9:	RAN4 first needs discussions for the baseline assumptions such as the relationship between PA configurations and candidate power classes, and the maximum power class/level.

Proposal 5:		It is proposed to consider a single PA configuration for each power class to specify the UE RF requirements of 3Tx as an initial stage.

2.5	MPR for 3Tx

Regarding maximum power reduction (MPR), RAN4 has specified the requirement for each power class. The number of Tx ports and device type have also been considered for the MPR requirements. Table 2 summarizes what RAN4 has defined for UL MIMO related MPR so far.

Table 2: Existing UL MIMO related MPR requirements

| Power class | Number of Tx | Requirements (for |
| --- | --- | --- |
| PC3 | 1 | Table 6.2.2-1 |
| PC2 | 2 | Table 6.2D.2-1, Table 6.2.2-2 |
| PC1.5 | 2 | Table 6.2D.2-2, Table 6.2D.2-3 (FWA) |
|  | 4 | Table 6.2D.2-4 (vehicular), 6.2D.2-5 (FWA) |

For the MPR requirement with 3Tx, RAN4 is supposed to define new MPR with 3Tx for target power classes to be decided. However, taking into account the existing MPR gap between 2 Tx and 4Tx for PC1.5, up to 1.5 dB gap for FWA devices as an example, it is reasonable for 3Tx to reuse existing MPR tables for dual Tx (handheld UE) as much as possible. For example, RAN4 can simply add 0-1 dB to the existing 2 Tx MPR requirements for 3Tx given our previous experience and efficient discussion in Rel-19.

Observation 10:	Existing MPR gap between 2 Tx and 4Tx for PC1.5 is up to 1.5 dB gap for FWA devices.

Proposal 6:		It is proposed to reuse existing MPR tables for dual Tx (handheld UE) as much as possible given our previous experience and efficient discussion in Rel-19.

3	Conclusion

This contribution focuses on the topic of non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions (3Tx), which has clearer RAN4 impacts and more sufficient progress than other topics in RAN1. Following observations and proposals were derived.

Observation 1:	3Tx discussion is on the fair way toward the current goal in the WID in RAN1.

Proposal 1:		RAN4 should first initiate the discussion with the 3Tx UE RF requirements

Observation 2:	RAN4 had a WI on 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC.

Observation 3:	RAN4 discussed requirements to enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices.

Proposal 2:		RAN4 should take the results of previous 3Tx (two bands) and 4Tx (CPE/FWA/vehicle/industrial devices) studies for Rel-19 3Tx (single band) discussions as much as possible.

Observation 4:	RAN4 can first draw some basic assumptions from the justification part of the WID.

Proposal 3:		Target devices of the WI are not only FWA (larger form factor) devices, but also handheld (small form factor) devices.

Proposal 4:		RAN4 can concentrate on FR1 requirements based on our initial discussion on STxMP in Rel-18 and market demands.

Observation 5:	WI considers non coherent UL codebook only (1-port/layer up to 3 layers).

Observation 6:	There is no enhancement on NCB PUSCH with 3Tx

Observation 7:	There is no enhancement on SRS resource

Observation 8:	UL full power transmission mode 0 (full power mode) is considered only

Observation 9:	RAN4 first needs discussions for the baseline assumptions such as the relationship between PA configurations and applicable power classes, and the maximum power class/level.

Proposal 5:		It is proposed to consider a single PA configuration for each power class to specify the UE RF requirements of 3Tx as an initial stage.

Observation 10:	Existing MPR gap between 2 Tx and 4Tx for PC1.5 is up to 1.5 dB gap for FWA devices.

Proposal 6:		It is proposed to reuse existing MPR tables for dual Tx (handheld UE) as much as possible given our previous experience and efficient discussion in Rel-19.

Reference

[1] 	RP-240087, WID revision: NR MIMO Phase 5, RAN#104

[2] 	RP-231719, WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, RAN#101

[3] RP-232366, Revised WID on on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1), RAN#101