3GPP TSG-RAN WG4 Meeting # 112															R4-2412136

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.1.1.3.4

Source:	Samsung

Title:	Views on SRS IL imbalance issue based on performance measurements

Document for:	Discussion

1	Introduction

As one of objectives, ”6Rx for handheld and FWA UE” under Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1], the issue of insertion loss imbalance across SRS ports has been discussed to study whether the corresponding solution is required or not.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

However, even though there were more than 16 contibutions proposing 17 candidate solutions for the SRS IL imbalance issue, they could not been discussed, but continued encouraging companies to bring more analysis for the justification of the work [2, 3]. Since an initial anlysis of the SRS IL imbalance impact on NW performance degradation, which is captured in the WF [3], had already been provided in [4], in this contribution, we would like to re-submit our initial studies and measurement results with various networks based on [4].

2 Discussion

In RAN4#111, in our understanding, RAN4 focused on drafting the way forwards on SRS IL imbalance without sufficient discussions on the proposed solutions or issues such as UE behaviours or network performance degradation as below [3].

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

Observation 1:	The proposed solutions or issues of SRS IL imbalance such as UE behaviours or network performance degradation has not been discussed sufficiently in RAN4.

However, as mentioned in the WID [1], the most important thing is that RAN4 should identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would be repeated to find out the solution for the undefined problem as previous discussions in Rel-18. Therefore, before taking any options in the WF [2], RAN4 should discuss and conclude whether the IL imbalance issue needs to be handled in our specifications based on the real network behaviour, for example.

Observation 2:	The most important thing is that RAN4 should identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would be repeated to find out the solution for the undefined problem as previous discussions in Rel-18.

To seek the justification, the problem that needs to be solved should be clarified first as there are a lot of sources affects both downlink and uplink performance related to the multiple SRS ports, e.g., IL imbalance, low SRS power, Rx interference in CA/DC, etc. Also, after the clarification, the problems should be confirmed by checking the impact on the actual performance in practical deployment. Without such confirmation, RAN4 does not have to repeat the same discussion as in Rel-18, nor need to continue the discussion for the SRS IL as noted in the WID.

Observation 3:	RAN4 first needs to justify what the issue is regarding the IL imbalance across SRS ports as there are a lot of sources affects both downlink and uplink performance related to the multiple SRS ports.

Proposal 1:		RAN4 should discuss and conclude whether the IL imbalance issue needs to be handled in the specification based on the real network check and measurements before moving forward to the solutions.

Theoretically, it is possible that the SRS IL imbalance could affect overall uplink and downlink performance due to the low SRS power if one PA based antenna switching is conducted. As a preliminary check for the potential issue, we have conducted some measurements under the various networks in the field to find out the best implement solution to compensate the imbalance. As one of such measurement results, the downlink throughput impact of SRS power imbalance between 4-port can be found in Figure 1.

Figure 1: DL performance by SRS power imbalance

However, unexpectedly, the impact of SRS power imbalance is pretty much dependent on the networks as the downlink performance varies according to the measurement location with the same condition as shown in Figure 1. In our understanding, the result implies that the issue can be handled by implementation as some networks have the same downlink performance regardless of the imbalance level. In other words, it would be not so meaningful capability for some UEs and networks even if RAN4 defines a solution to compensate the gap.

Observation 4:	SRS power imbalance impact is pretty much dependent on the networks, which implies that the issue can be handled by implementation as some networks have the same downlink performance regardless of the imbalance level.

Observation 5:	It would be not so meaningful capability for some UEs and networks even if RAN4 defines a solution to compensate the gap.

On top of the network dependency, the measurements also have a similar result across networks that the meaningful performance degradation has not been seen even from the large imbalance gap between antennas. Since the threshold point starting the performance degradation is even much higher than what RAN4 had thought about, e.g., 7.5 dB for t1r4 PC2, it can be said that the SRS IL imbalance is actually not an issue in the real networks. Furthermore, if SRS transmission power does not reach to its maximum power, the issue did not occur, or had a marginal impact across measurement sites and networks.

Observation 6:	In our measurements, it has not been seen the meaningful performance degradation even from the large imbalance gap between antennas under various scenarios with various networks.

In addition, even though the SRS power reduction happened, the networks cannot simply distinguish whether it is from IL imbalance, channel fading, hand grip loss, or other issues. Although some reporting schemes or capabilities can be introduced expecting the better network behavior, the fundamental issue cannot be removed because many other underlying factors like path differences between antennas and LPAF (LNA/PA/Filter/switch) are still there affecting the imbalance. It also should be noted that the level of the imbalance could be changed instantly as shown in our measurements that may make the reporting useless. In this regard, introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation.

Observation 7:	Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation.

Therefore, at the current stage, we believe that the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Proposal 2:		At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

3	Conclusion

In this contribution, following the direction of the approved WF in the last meeting [3], we focus on the issue of insertion loss imbalance across SRS ports based on our initial studies and measurement results with various networks, which are provided already in [4]. Following observations and proposals are provided to be considered by this meeting.

Observation 1:	The proposed solutions or issues of SRS IL imbalance such as UE behaviours or network performance degradation has not been discussed sufficiently in RAN4.

Observation 2:	The most important thing is that RAN4 should identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would be repeated to find out the solution for the undefined problem as previous discussions in Rel-18.

Observation 3:	RAN4 first needs to justify what the issue is regarding the IL imbalance across SRS ports as there are a lot of sources affects both downlink and uplink performance related to the multiple SRS ports.

Proposal 1:		RAN4 should discuss and conclude whether the IL imbalance issue needs to be handled in the specification based on the real network check and measurements before moving forward to the solutions.

Observation 4:	SRS power imbalance impact is pretty much dependent on the networks, which implies that the issue can be handled by implementation as some networks have the same downlink performance regardless of the imbalance level.

Observation 5:	It would be not so meaningful capability for some UEs and networks even if RAN4 defines a solution to compensate the gap.

Observation 6:	In our measurements, it has not been seen the meaningful performance degradation even from the large imbalance gap between antennas under various scenarios with various networks.

Observation 7:	Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation.

Proposal 2:		At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Reference

[1] RP-240828	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

[2] R4-2408937	Topic summary for [111][126] NR_ENDC_RF_Ph4_part3

[3]	R4-2410751	WF on requirements for 6Rx

[4] R4-2409660	Views on SRS IL imbalance