3GPP TSG-RAN WG4 Meeting #112															R4-2412137

Maastricht, Netherland, 19th – 23rd August, 2024

Agenda item:			8.2.3

Source:	Samsung

Title:	Views on IMT parameters for 7125 to 8400 MHz frequency range

Document for:	Discussion

1 Introduction

In RAN4#110, RAN4 set up the following estimated completion dates for the work although those are challenging and may need more time to complete parameters at 14.8 – 15.35 GHz [1]:

- 4 400-4 800 MHz	Estimated date for completion: May 2024 (RAN WG4#111)

- 7 125-8 400 MHz	Estimated date for completion: August 2024 (RAN WG4#112)

- 14.8-15.35 GHz	Estimated date for completion: November 2024 (RAN WG4#113)

In RAN4#111, the first reply LS was sent out for the range of 4400 to 4800 MHz as scheduled [2]. Similarly, in RAN4#112, RAN4 is supposed to agree and send the second response to ITU-R WP 5D on IMT parameters relevant for sharing and compatibility for 7125 to 8400 MHz frequency range. In this contribution, we would like to provide our views on the remaining issues for the frequency range to support concluding the reply LS for their sharing study in 7125 to 8400 MHz.

2 Discussion

For 8 GHz, frequency range of 7125-8400 MHz, it was agreed that existing n104 requirements should be a baseline for future discussions, while checking the parameters if they could differ from n104. Also, looking through the previous WF on the 8 GHz range, it is shown that most parameters were discussed and reached agreements based on existing parameters of n104 as summarized in [3].

Observation 1:	For 8 GHz, frequency range of 7125-8400 MHz, it was agreed that existing n104 requirements should be a baseline for future discussions.

Observation 2:	Most parameters were discussed and reached agreements based on existing parameters of n104.

Given the WF, following topics needs further discussions [3].

Issue 2-3: Typical channel bandwidth Agreement: - Mention 100MHz as typical bandwidth in the LS - FFS whether to mention wider bandwidths may be possible in future in the LS Describe other possibilities for bandwidth in the TR, mentioning that additional specification effort needed for >100MHz Issue 2-4: Typical signal bandwidth Outcome of discussion (Not agreed): Option 1: 273 RB, 30k SCS as 38.104 Option 2: Quote formula of RBs * SCS without number of RBs Option 3: Directly quote the signal bandwidth in MHz Issue 2-14: BS antenna array parameters Issue 2-15: Maximum output power Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

Typical channel/signal bandwidth

Regarding typical channel bandwidth and its signal bandwidth, it was agreed that the LS would mention 100 MHz as ‘typical’ channel bandwidth. However, there is still FFS whether to mention wider bandwidths in the LS, or just to leave the possibilities in the TR. A concern during the last discussion was that adding such possibilities to the LS can be confusing WP 5D for their study, and this is the same argument as what provided in 4 GHz for SBFD. Also, it has been said that new value/scheme different with the previous LS may affect the WP 5D study and/or WRC conclusion.

However, as mentioned before, in our view, it is not clear that such supplementary text/note affects the result/process of the sharing study in WP 5D even though the parameter still has a single typical value/scheme in the LS. Also, based on our experience there, the additional note is normally nothing to do with the study or interference level of IMT to legacy services in a certain frequency range since WP 5D usually picks up a single typical value among the received parameters from various SDOs during their discussion. It is also true that such non-technical subjective debate can happen anytime and anywhere in ITU-R regardless of the ‘confusing’ or ‘uncertain’ information from 3GPP.

Observation 3:	Supplementary text/note may not affect the result/process of the sharing study in WP 5D as long as the parameter has a single typical value/scheme in the LS.

Observation 4:	Non-technical subjective debate can happen anytime and anywhere in ITU-R regardless of the ‘confusing’ or ‘uncertain’ information from 3GPP.

In addition, as RAN4 is discussing new frequency range targeting for the next generation, given the regional spectrum plan and WRC-27 study period, there is no need to limit any possibilities of larger channel bandwidth introduction in the future by ourselves as long as we have a single typical value there for ITU-R. In this regard, it would be better to describe the possibilities of wider channel bandwidths both in the TR and LS.

Observation 5:	There is no need to limit any possibilities of larger channel bandwidth introduction in the future by ourselves as long as we have a single typical value there for ITU-R.

Proposal 1:		It would be better to describe the possibilities of wider channel bandwidths both in the TR and LS.

For the ‘typical’ signal bandwidth based on ‘typical’ channel bandwidth, it can be taken by either option with existing n104 parameter for 100 MHz as we have agreed 100 MHz for the typical channel bandwidth. However, since the current spec does not have the requirement for the new frequency of 8 GHz range, i.e., number of RBs or SCS support 100 MHz channel bandwidth to calculate the signal bandwidth, it is preferred to quote directly in MHz based on 100 MHz for n104 (Option 3) rather than other indirect options.

Proposal 2:		It is preferred to quote directly in MHz based on 100 MHz for n104 (Option 3) rather than other indirect options since the current spec does not support the new frequency of 8 GHz range.

BS antenna array parameters

For the next issue regarding BS antenna array parameters for suburban macro and urban macro, there remain two options as below.

In our understanding from the discussion, each option has the reasonable point. Option 1 was derived by the same parameters for 4 GHz range, which is a half frequency of 8 GHz with 4x8 for the antenna array configuration [2]. So, doubled H and V are required or possible to support the doubled frequency range. On the other hand, Option 2 considered the parameter in TR 38.921 for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 GHz, which is 8x16 without sub-array. Although we still prefer the option we proposed before, i.e., 8x8x3, other options such as 8x16x3 would also be fine for the sake of progress.

Proposal 3:		Other options such as 8x16x3 would also be fine for the sake of progress.

UE maximum output power

For the last topic about UE maximum output power, it has been discussed which power class should be captured as typical maximum output power in the LS to WP 5D. In our view, RAN4 needs to take a different way with the previous SBFD or typical channel bandwidth discussion given the fact that UE maximum output power is limited by not only the enhancement of hardware components but also regional regulation that would not be a matter of enhancements or negotiations. Therefore, in our view, 23 dBm should be adopted in the LS for the typical maximum output power of normal UE unless 26 dBm can always be achieved without a solution to handle the SAR issue.

Proposal 4:		23 dBm should be adopted in the LS for the typical maximum output power of normal UE.

In conclusion, taking into account observations and proposals for the tricky but resolvable remaining issues above, it is better for RAN4 to send out 8 GHz parameters in this meeting. In considerations of the both schedule of RAN4 and WP 5D. By doing so, it would also help RAN4 to concentrate the study efforts on the new range, i.e., 14800 to 15350 MHz given the challenging schedule.

Proposal 5:		Taking into account observations and proposals for the tricky but resolvable remaining issues above, it is better for RAN4 to send out 8 GHz parameters in this meeting.

3	Conclusion

This contribution provides following observations and proposals on the remaining issues for the frequency range of 7125-8400 MHz to be considered by this meeting.

Observation 1:	For 8 GHz, frequency range of 7125-8400 MHz, it was agreed that existing n104 requirements should be a baseline for future discussions.

Observation 2:	Most parameters were discussed and reached agreements based on existing parameters of n104.

Observation 3:	Supplementary text/note may not affect the result/process of the sharing study in WP 5D as long as the parameter has a single typical value/scheme in the LS.

Observation 4:	Non-technical subjective debate can happen anytime and anywhere in ITU-R regardless of the ‘confusing’ or ‘uncertain’ information from 3GPP.

Observation 5:	There is no need to limit any possibilities of larger channel bandwidth introduction in the future by ourselves as long as we have a single typical value there for ITU-R.

Proposal 1:		It would be better to describe the possibilities of wider channel bandwidths both in the TR and LS.

Proposal 2:		It is preferred to quote directly in MHz based on 100 MHz for n104 (Option 3) rather than other indirect options since the current spec does not support the new frequency of 8 GHz range.

Proposal 3:		Other options such as 8x16x3 would also be fine for the sake of progress.

Proposal 4:		23 dBm should be adopted in the LS for the typical maximum output power of normal UE.

Proposal 5:		Taking into account observations and proposals for the tricky but resolvable remaining issues above, it is better for RAN4 to send out 8 GHz parameters in this meeting.

Reference

[1]	R4-2403653	LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the
 				frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	R4-2410576	LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and
 				compatibility studies in preparation for WRC-27

[3]	R4-2410741	WF on IMT parameters study