3GPP TSG-RAN WG4 Meeting #112															R4-2412138

Maastricht, Netherland, 19th – 23rd August, 2024

Agenda item:			8.2.4.1

Source:	Samsung

Title:	Initial simulation results of co-existence scenarios for 14800 – 15350 MHz

Document for:	Discussion

1 Introduction

Based on the LS from ITU-R WP 5D [1], a Study Item was approved in RAN#103 to task RAN4 to study the IMT parameters relevant for sharing and compatibility for the frequency ranges, and to send LS reply to ITU-R WP 5D directly (cc to RAN) [2].

For 15 GHz range, RAN4 agreed to conduct coexistence studies to derive interference levels on adjacent channels before the IMT parameters for ITU-R sharing study. Although it still has multiple options due to lack of consensus, simulation assumptions are agreed for the co-existence study in RAN4#111 [3]. This contribution provides our initial results of DL-to-DL co-existence studies for 15 GHz frequency range based on the current assumptions in [3].

2 Discussion

In [3], initial simulation scenarios are captured in Table 1 with the priority level for each scenario as below.

Table 1: Summary of initial simulation scenarios for 14800 - 15350 MHz [3]

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, TBD MHz | NR, TBD MHz | DL to DL | 15 GHz | Indoor hotspot | Second |
| 2 | eMBB | NR, TBD MHz | NR, TBD MHz | DL to DL | 15 GHz | Urban macro | First |
| 3 | eMBB | NR, TBD MHz | NR, TBD MHz | DL to DL | 15 GHz | Dense urban | Third |
| 4 | eMBB | NR, TBD MHz | NR, TBD MHz | UL to UL | 15 GHz | Indoor hotspot | Second |
| 5 | eMBB | NR, TBD MHz | NR, TBD MHz | UL to UL | 15 GHz | Urban macro | First |
| 6 | eMBB | NR, TBD MHz | NR, TBD MHz | UL to UL | 15 GHz | Dense urban | Third |

As shown in Table 1, the urban macro scenario is the first priority with both scenario 2 (DL) and scenario 5 (UL). However, given the fact that RAN4 still has not a few simulation parameters under discussion, only scenario 2 is selected in this contribution. It is also considered that this is the first meeting to discuss the co-existence simulation. Following assumptions in Table 2 are considered for some parameters having multiple options.

Table 2: Simulated cases for DL-to-DL scenario

|  | UE Ant | UE # | UL power | Indoor ratio | BS Ant | ISD | Bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FR1 like | 1 | 23 | 0 | 64x32x8 | 450 | 400MHz |
| 2 | FR1 like | 1 | 23 | 0 | 64x32x8 | 350 | 400MHz |
| 3 | FR1 like | 1 | 23 | 0 | 64x32x8 | 450 | 100MHz |
| 4 | FR1 like | 1 | 23 | 0 | 64x24x4 | 450 | 100MHz |
| 5 | FR1 like | 1 | 23 | 0 | 64x24x4 | 350 | 100MHz |

2.1 Case 1: 64x32x8, 450m, 400MHz

Table 3: Simulation results for throughput loss (%)

| ACIR [dB] | Collocated |  |  |  |  | Non-collocated |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 5 | 10 | 15 | 20 | 25 |
| Average | 6% | 3% | 1% | 1% | 0% | 5% | 3% | 1% | 0% | 0% |
| 5%-tile | 26% | 17% | 10% | 6% | 4% | 56% | 20% | 12% | 7% | 4% |

| <Collocated> | <Non-collocated> |
| --- | --- |

Figure 1: Simulation results for throughput loss (%)

2.2 Case 2: 64x32x8, 350m, 400MHz

Table 4: Simulation results for throughput loss (%)

| ACIR [dB] | Collocated |  |  |  |  | Non-collocated |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 5 | 10 | 15 | 20 | 25 |
| Average | 2% | 1% | 1% | 0% | 0% | 2% | 1% | 1% | 0% | 0% |
| 5%-tile | 17% | 8% | 6% | 4% | 3% | 17% | 8% | 6% | 4% | 3% |

| <Collocated> | <Non-collocated> |
| --- | --- |

Figure 2: Simulation results for throughput loss (%)

2.3 Case 3: 64x32x8, 450m, 100MHz

Table 5: Simulation results for throughput loss (%)

| ACIR [dB] | Collocated |  |  |  | Non-collocated |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 25 |
| Average | 2% | 1% | 1% | 0% | 2% | 1% | 1% | 0% | 0 |
| 5%-tile | 15% | 8% | 6% | 5% | 14% | 6% | 4% | 3% | 2% |

| <Collocated> | <Non-collocated> |
| --- | --- |

Figure 3: Simulation results for throughput loss (%)

2.4 Case 4: 64x24x4, 450m, 100MHz

Table 6: Simulation results for throughput loss (%)

| ACIR [dB] | Collocated |  |  |  | Non-collocated |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 25 |
| Average | 5% | 4% | 2% | 1% | 2% | 1% | 1% | 0% | 0% |
| 5%-tile | 45% | 32% | 17% | 6% | 15% | 12% | 8% | 5% | 2% |

| <Collocated> | <Non-collocated> |
| --- | --- |

Figure 4: Simulation results for throughput loss (%)

2.5 Case 5: 64x24x4, 350m, 100MHz

Table 7: Simulation results for throughput loss (%)

| ACIR [dB] | Collocated |  |  |  | Non-collocated |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 5 | 10 | 15 | 20 | 25 |
| Average |  |  |  |  |  |  |  |  |  |
| 5%-tile |  |  |  |  |  |  |  |  |  |

| <Collocated> | <Non-collocated> |
| --- | --- |

Figure 5: Simulation results for throughput loss (%)

Based on the tables and figures above, it is observed that the average throughput loss for most cases is low enough compared to other frequency range with the smaller antenna array elements. It is our understanding that the lower average throughput comes from the massive antenna pattern assumptions, e.g., 64x24 or 64x32, which have much lower side lobe gain of the antenna pattern with narrow beams. Also, the edge at 5%-tile throughput loss is higher than average throughput loss that could determine the ACIR requirement, which can be in the range of 14-26 dB.

Observation 1:	Average throughput loss for most cases is low enough compared to other frequency range with the smaller antenna array elements thanks to the massive antenna pattern assumptions, e.g., 64x24 or 64x32, which have much lower side lobe gain of the antenna pattern with narrow beams.

Observation 2:	ACIR requirement can be in the range of 14-26 dB.

3	Conclusion

This contribution provides our initial results of DL-to-DL co-existence studies for 15 GHz frequency range based on the assumptions under discussion. Based on the simulation results summary in Table 8, following observations are derived to be discussed in the meeting.

Table 8: Summary of simulation results

|  | Collocated |  | Non-collocated |  |
| --- | --- | --- | --- | --- |
| ACIR | Average | Edge | Average | Edge |
| Case 1 | 6 | 26 | 4 | 21 |
| Case 2 | 0 | 18 | 0 | 18 |
| Case 3 | 6 | 22 | 6 | 23 |
| Case 4 | 0 | 18 | 0 | 14 |
| Case 5 | 0 | 18 | 0 | 16 |

Observation 1:	Average throughput loss for most cases is low enough compared to other frequency range with the smaller antenna array elements thanks to the massive antenna pattern assumptions, e.g., 64x24 or 64x32, which have much lower side lobe gain of the antenna pattern with narrow beams.

Observation 2:	ACIR requirement can be in the range of 14-26 dB.

Reference

[1] R4-2400333	Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency
 				bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	RP-240787	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to
 				15350 MHz

[3]	R4-2410592	TP to TR 38.922: System level simulation methodology and assumptions for coexistence study for
 				14800 - 15350 MHz frequency range