3GPP TSG-RAN WG4 Meeting #112															R4-2412139

Maastricht, Netherland, 19th – 23rd August, 2024

Agenda item:			8.2.4.2

Source:	Samsung

Title:	Views on UE type for 14800 to 15350 MHz frequency range

Document for:	Discussion

1 Introduction

Based on the LS from ITU-R WP 5D [1], a Study Item was approved in RAN#103 to task RAN4 to study the IMT parameters relevant for sharing and compatibility for the frequency ranges, and to send LS reply to ITU-R WP 5D directly (cc to RAN) [2].

In RAN4#111, RAN4 discussed the UE type support frequency range of 14800 to 15350 MHz, which is the range in-between FR1 UE (omnidirectional without beamforming) and FR2 UE (directional with beamforming). However, due to limited time for sufficient productive discussion between two options, both UE types were agreed for the coexistence simulation only as noted in the WF [3].

Although required link budget or UL performance can be further determined based on the coexistence study, it is also important for RAN4 to check the possibilities from implementation and RF/antenna hardware perspective. In this contribution, we would like to continue providing our initial views on the possible UE type for 15 GHz based on our last contribution [4].

2 Discussion

For the 15 GHz range, RAN4 agreed to conduct coexistence studies and leave them on the TR on top of radio and antenna parameters at the frequency ranges. It is also tried to have initial simulation assumptions for the 15 GHz study in RAN4#111, which are captured in TP [5]. As mentioned above, during the UE type discussion, both options were agreed without sufficient discussion for the coexistence simulation as below:

Issue 3-8: UE type Agreement: - For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP

As shown in the agreement above, FR2 like UE type with UE beamforming antennas can provide higher directional gain on EIRP such as 5dBi element gain. However, based on the experience in FR2 and previous studies in the 7-24 GHz [6], the first question or consideration can be raised whether such beamforming is beneficial even for the handheld UE form factor at 15 GHz frequency range. For example, given that 1x4 phase array antennas have been assumed for the FR2 requirements from Rel-15, and is widely adopted in FR2 handheld UEs up to now, it could be foreseen that smaller antenna arrays, e.g., 2×1, 2x2, or individual antenna element could be used taking into account the limited form factor of the handheld UE. Therefore, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be reconsidered.

Observation 1:	For FR2 like UE type, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be reconsidered.

Similar to the Rel-16 study in [6], we have checked the potential UE antenna characteristics on the frequency range. In terms of the array antenna size, the typical spacing between elements of the array antenna can be half wavelength, and further decrease in spacing will cause the negative impact on the antenna gain and coupling problems among each element. Therefore, the total size of array can be determined according to the arrangement of the antennas as shown in Figure 1.

Also, for the possible number of antenna modules/panels at 15 GHz, it should be noted that it is not even easy to mount single patch antennas on four edges or the backside of the smartphone considering the larger array size and the already-packed smartphone space as shown in Figure 2.

Figure 1: Estimated antenna size at 15 GHz

Figure 2: Main PCB size example of a foldable smartphone

Therefore, in our understanding, it would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and number of antenna modules/panels.

Observation 2:	It would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and number of modules/panels.

Regarding FR1 like UE type with isotropic UE antennas, it has been discussed how many antennas should be considered for the transmission and reception to achieve better uplink and downlink performances at the higher frequency range than FR1. Theoretically, in the 15 GHz band, the higher number of receive antennas, e.g., 6Rx or 8Rx, than currently used for FR1 could be implemented taking into account the smaller antenna size, required diversity/throughput gain against the path loss. Also, according to the PA performance in [6] and studied recently, it should be considered that both Psat and PAE could have reduced much compared with PAs operating in FR1, e.g., 6 dB lower than PC3 with legacy waveform, even they would be higher than those in FR2. In that sense, multiple Tx operation with FR1 like antennas may be appropriate for 15 GHz uplink.

However, it should be also noted that increasing the number of antennas is not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor. For example, implementing additional antennas, e.g., up to 8Rx, will result in the increased coupling among antennas, leading to a degradation in MIMO performance. Since the insertion loss per unit length is much higher in 15 GHz, it would be not easy to optimize all the antenna feeding lines to ensure the better performance.

Observation 3:	For FR1 like UE type, it should be also noted that increasing the number of antennas is not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor.

Therefore, RAN4 needs more discussions to find out the optimal balance between two options with observations above for the better performance than legacy UEs in FR1 and FR2. At the same time, for 6G or future technologies, it could be one possible solution for RAN4 to have both options for the frequency range in-between FR1 and FR2 so that UE can apply or declare either option based on the UE type. For example, FWA UE with larger form factor can adopt FR2 like beamforming antennas while the handheld smartphone with limited form factor can implement FR1 like multi-antenna. Detailed can be further discussed in future releases.

Observation 4:	RAN4 needs more discussions to find out the optimal balance between Option 1 and Option 2 for the better performance than legacy UEs in FR1 and FR2.

Observation 5:	One possible solution for RAN4 is to have both options for the frequency range in-between FR1 and FR2 so that UE can apply or declare either option based on the UE type.

Given the pros and the cons of each UE type, i.e., FR1 like or FR2 like, and the alternative introduced above, both options could be considered together in the LS and TR based on the existing parameters in TS 38.101-1 and TS 38.101-2. Otherwise, if only one parameter required for the LS, we prefer FR1 like option for the LS at this stage based on the feasibility discussed above.

Proposal:			Both UE type options could be considered together in the LS and TR. Otherwise, if only one parameter required for LS, FR1 like option is preferred for the LS at this stage.

3	Conclusion

This contribution provides following observations and proposals to be considered by this meeting.

Observation 1:	For FR2 like UE type, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be reconsidered.

Observation 2:	It would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and number of modules/panels.

Observation 3:	For FR1 like UE type, it should be also noted that increasing the number of antennas is not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor.

Observation 4:	RAN4 needs more discussions to find out the optimal balance between Option 1 and Option 2 for the better performance than legacy UEs in FR1 and FR2.

Observation 5:	One possible solution for RAN4 is to have both options for the frequency range in-between FR1 and FR2 so that UE can apply or declare either option based on the UE type.

Proposal:			Both UE type options could be considered together in the LS and TR. Otherwise, if only one parameter required for LS, FR1 like option is preferred for the LS at this stage.

Reference

[1] R4-2400333	Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency
 				bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	RP-240787	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to
 				15350 MHz

[3]	R4-2410741	WF on IMT parameters study

[4]	R4-2409463	Views on IMT parameters for 14800 to 15350 MHz frequency range

[5]	R4-2410592	TP to TR 38.922

[6]	3GPP TR 38.820 V16.1.0