3GPP TSG-RAN WG4 Meeting #112	R4-2412143

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.16.1

Source:	Ericsson

Title:	On general issues for demodulation requirement with interference

Document for:	Discussion

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, the following are objectives for UE and BS parts:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provided our view on general issues for UE and BS demodulation requirement with interference.

# 2	Discussion

## 2.1	UE demodulation and CSI requirement

In this section, we plan to discuss the scope and discussing flow that RAN4 may follow to derive the final PDSCH and CQI reporting test case and requirement. We propose to first reach agreements on the interference model before stepping into the detail of parameter assumption.

Proposal 1: RAN4 should first reach agreements on the interference model before dig into the detailed parameter assumption.

Following the objectives, for inter-cell interference cancellation, it’s expected to introduce PDSCH and CQI reporting requirement with 8Rx reception.

The following aspects related to the interference model should be taken into consideration:

- Number of interfering cell

- Interference profile

  - Whether to reuse the DIP method and INR level

- Transmission rank

- Time/frequency offset

- SSB configuration

Proposal 2: Propose RAN4 to discuss and agree at least on the configurations of the following interference model aspects for inter-cell interference mitigation

- Number of interfering cell

- Interference profile

  - Whether to reuse the DIP method and INR level

- Transmission rank

- Time/frequency offset

- SSB configuration

As for the scenario of intra-cell inter-user interference, it is expected to introduce PDSCH demodulation requirement. Similarly, we propose to discuss the interference model first with the following aspects to be considered:

- Number of paired UE

- Rank for target and interference

- Precoder codebook type

- PMI selection and precoding

- MCS for interference

- DMRS configurations

Proposal 3: Propose RAN4 to discuss and agree at least on the configurations of the following interference model aspects for intra-cell inter-user interference cancellation:

- Number of paired UE

- Rank for target and interference

- Codebook type

- PMI selection and precoding

- MCS for interference

- DMRS configurations

After reaching agreements on the interference model and profile, it is suggested RAN4 to discuss the detailed parameter assumptions and encourage companies to bring initial simulation results so that further updates on the assumption (if needed) can happen to help facilitate the final configuration and derive the requirement.

## 2.2	BS demodulation requirement

The MMSE-IRC receiver is widely used in wireless communication network to migrate the interference from unexpected directions. The benefit is obvious compared to MRC receiver in high traffic load scenario and heterogenous network. In LTE Rel-13, the demodulation gain is studied and BS demodulation requirements for MMSE-IRC are introduced for both synchronous and asynchronous deployments.

In previous Rel-17 and Rel-18, NR MMSE-IRC UE demodulation requirements are defined. In NR deployment, high power UE would be also used and could give higher interference on UL than LTE deployment. This is mentioned in justification of WID. In that case, companies have consensus that it is worthy to study the BS demodulation requirements for NR MMSE-IRC receiver in Rel-19.

The current scenario scope focus on inter-cell interference under homogenous and heterogenous with FDD and TDD synchronous deployment.

To evaluate the performance of BS MMSE-IRC for NR, a proper interference profile is necessary. To have such a profile, there are two possible approaches can be considered. The first option is following similar study procedures as TR36.884 to discuss system level and link level simulations, and a TR might be needed to capture evaluation output.

- Typical NR deployment configurations for system level simulation.

- Interference modeling.

- Interference profile definition.

- IRC assumptions and link level simulations.

The interference profile is the key configuration for IRC performance simulation, but the system level simulations would take a large effort to discuss parameters and align the results. Another option is to use artificial values for interference levels to avoid system level simulations. The following subsection would discuss the interference model used in 3GPP and discuss the possibility of avoid system level simulations.

Interference model definition

There are two modeling method discussed in LTE BS MMSE-IRC requirements. One method is DIP (Dominant interferer proportion) model and the other one is INR (Interference Noise Ratio) model.

- DIP model

This model was firstly introduced for UE MMSE-IRC receiver in 3GPP. The DIPx is defined by the ratio of the power of xth strongest interference to the power of all interference and noise. If we assume there are NUE in total UE transmitting simultaneously and the received signal power of wanted UE is I0, and I1, I2, …, INUE-1 are interferences. The thermal noise power of a certain channel bandwidth for all UE is the same as σ2. Then,

DIPi=IiIoc, where Ioc=i=1NUE-1(Ii+σ2), i=0,1,2,…,NUE-1 	     	            (2.2-1)

And UL SINR is,

SINR= I0Ioc             	                            (2.2-2)

It should be noted that the DIPx  values won’t change if the total number of interferences are fixed. That would be beneficial for performance comparison between different number of interferences. It is more suitable for full buffer system simulation which interferers are always transmitting.

There are two methods mentioned in TR 36.884 to derive DIPx based on system level simulations.

-	Methodology 1

Step 1: Decide DIP1. First obtain the distribution of unconditional DIP1 values from all the simulated samples. The DIP1 value at 85 %-tile of the DIP1 distribution is taken.

Step 2: Decide DIP2. For the DIP1 value at 85 %-tile, the median of the conditioned DIP2 are obtained, where the median DIP2 is obtained from all DIP2 whose corresponding DIP1 fall within ±5 %-tile of 85 %-tile (i.e., 80~90 %-tile).

-	Methodology 2

Step 1: Identify the SINR value at 5 %-tile of UL wideband SINR distribution as the SINR of interest.

Step 2: For each simulated sample, if the UL wideband SINR fall within +/- 0.2 dB of 5 %-tile UL wideband SINR, the DIP1/2 values are saved for this sample.

Step 3: After saving all the conditional DIP1/2 values, the median values of DIP1/DIP2 distribution are taken.

Based on the output of two methods, DIP1 values are very similar and DIP2 values have 1~2dB difference. Considering DIP2 values are over 10dB less than DIP1, two methods would show similar performance by link level simulations. In that case, companies agree to use methodology 1 for the interference profile definition. In NR part, similar methodologies could be reused if DIP model is agreed.

- INR model

This model is also widely used for interference level definition.

INRi=j=1NRxIor(i)(j)j=1NRxNoc(j) 		                         (2.2-3)

Where, Ior(i)(j) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario on the j-th antenna connector and Noc(j) is the average power spectral density of a white noise source on the j-th antenna connector. In 3GPP NR MMSE-IRC PDSCH inter-cell interference requirements, INR is used for interference model definition.

Comparing DIP model and INR model, we can see that  Ii=j=1NRxIor(i)(j) and σ2= j=1NRxNoc(j) if we assume interference and wanted signal have same channel bandwidth and white noise are same on all UE.  Then we can get INRi =Iiσ2  and  Iocσ2=i=1NUE-1(Iiσ2+1)=i=1NUE-1(INRi+1), so DIP could be expressed as

DIPi=IiIoc=INRii=1NUE-1(INRi+1)    		              (2.2-4)

On the other direction, we can also express INR by DIP by evaluation on (2.2-4).

i=1NUE-1DIPi=i=1NUE-1INRii=1NUE-1(INRi+1)=1-NUE-1i=1NUE-1(INRi+1)                      (2.2-5)

Then we can get

INRi=DIPi*i=1NUE-1(INRi+1)=DIPiNUE-11-i=1NUE-1DIPi            (2.2-6)

For 1 interferer only scenario, INR1=DIP11-DIP1

For 2 interferers scenario, INR1=2DIP11-DIP1-DIP2,  INR2=2DIP21-DIP1-DIP2

In that case, two models can be expressed to each other and the basically same. From the link level perspective, INR model would be much easier for interference level configuration than DIP model due to it is UE independent.

Observation 1: The DIP model and INR model could be expressed by each other. The INR model would be much easier for link level simulation.

Proposal 4: Use INR for NR BS MMSE-IRC interference model.

The purpose of system simulation is to derive the proper interference levels in typical network deployment, then the corresponding requirement could be more practical. But from the receiver algorithm perspective, an artificial interference level could also check the performance. In that case, it is possible to avoid system simulations by using a proper artificial interference profile which can achieve stable performance and get alignment between companies.

According to LTE BS MMSE-IRC study report [4], DIP1 level could be dominant for whole performance since DIP2 level is much smaller. In homogenous and heterogenous deployments, (DIP1, DIP2) are (-1.11, -10.91) dB and (-0.43, -13.78) dB separately. The corresponding performances of homogenous and heterogenous are quite similar. The reason behind might come from the evaluation method of DIP. If DIP1 is higher, DIP2 will be smaller because they are compared to same total interference power. From the performance checking perspective, it is not clear which one would be better between DIP1 >> DIP2 and DIP1 slightly higher than DIP2 for two interferers cases.

Note that, we can convert LTE DIP values above to INR model by formula (2.2-6) as (INR1, INR2) are (10.29, 0.5) dB and (15.37, 2.04) dB for homogenous and heterogenous deployments separately.

If artificial value is considered, companies could directly do trial simulations by different interference level combinations. For example, some companies have concern on PC1 or PC2 HPUE which have 7 or 3dB higher Tx power than legacy PC3 UE, so in trial link level simulations, INR1 could take 7dB or 3dB higher than LTE INR1 and check the performance. Companies could furtherly discuss how to modify the values based on their own simulation results.

Proposal 5: Companies to check if artificial interference profile based on LTE MMSE-IRC BS interference profile is feasible for NR MMSE-IRC BS demodulation requirement derivation.

# 3	Summary

In this contribution, we shared our views on general issues related to UE and BS demodulation requirement with interference. Our proposals and observations are summarized as follows:

Proposal 1: RAN4 should first reach agreements on the interference model before dig into the detailed parameter assumption.

Proposal 2: Propose RAN4 to discuss and agree at least on the configurations of the following interference model aspects for inter-cell interference mitigation

- Number of interfering cell

- Interference profile

  - Whether to reuse the DIP method and INR level

- Transmission rank

- Time/frequency offset

- SSB configuration

Proposal 3: Propose RAN4 to discuss and agree at least on the configurations of the following interference model aspects for intra-cell inter-user interference cancellation:

- Number of paired UE

- Rank for target and interference

- Codebook type

- PMI selection and precoding

- MCS for interference

- DMRS configurations

Observation 1: The DIP model and INR model could be expressed by each other. The INR model would be much easier for link level simulation.

Proposal 4: Use INR for NR BS MMSE-IRC interference model.

Proposal 5: Companies to check if artificial interference profile based on LTE MMSE-IRC BS interference profile is feasible for NR MMSE-IRC BS demodulation requirement derivation.

# References

[1]	RP-241297: Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN#104, Shanghai, China

[2]	TS38.101-4, v18.5.0

[3]	TS38.104, v18.5.0

[4] 	TS36. 884, v13.1.0