3GPP TSG-RAN WG4 Meeting #112	R4-2412144

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	8.16.2

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx

Document for:	Discussion

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, following are the objectives for UE part:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

In this contribution, we provided our view on how to define the PDSCH demodulation and CQI reporting requirements with interference cancellation using MMSE-IRC receiver for 8Rx UEs.

# 2	Inter-cell interference

In this section, we discuss the scenario of inter-cell interference cancellation with 8Rx reception.

## 2.1	Inter-cell interference model

As for the inter-cell interference model, we propose to reuse the model used for Rel-17 NR performance enhancement that contains two interfering cells for initial evaluation purpose.

In Rel-17, the interference model that RAN4 agreed to use was defined in B.6 in TS38.101-4 [2], where the interference to noise ratio (INR) is defined as follows:

Each interfering cell involved in PDSCH performance requirements with intercell interference is characterized by its associated interferer to noise ratio (INR) value: INRi=j=1NRXIor(i)(j)j=1NRXNoc(j) where Ior(i)(j) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario on the j-th antenna connector and Noc is the average power spectral density of a white noise source consistent with definition provided in section [4.4.3/4.5.3].

Reuse the Dominant Interferer Proportion (DIP) method that considered back in Rel-17 to determine the interference profile. Following exact INR values for two different interference cells can be reused for performing initial evaluation:

| Parameter | Unit | Interference Cell 1 | Interference Cell 2 |
| --- | --- | --- | --- |
| INR | dB | 7.77 | 2.29 |

Specifically, as described in B.6.2 in TS38.101-4 [2], the interference model can be assumed as that in each subframe, each interfering cell shall transmit 16 QAM based randomly modulated data over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel. Transmitted physical channels shall include SSB and TRS/CSI-RS as specified in requirements section.

Regarding the transmission rank for the interference cells, legacy configurations are shown as follows:

| Parameter | Unit | interference Cell 1 | Interference Cell 2 |
| --- | --- | --- | --- |
| Transmission rank | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |

In Rel-19 with 8Rx reception, we propose to also consider rank 1 and rank 2 mixed with probabilities as specified above for the transmission rank of the interfering cells since higher rank interference tend to appear as white noise.

As for the precoding of interfering PDSCH, we propose to reuse the method that is to assume random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2.

We also propose to use 16QAM randomly modulated symbols for the modulation order of interfering PDSCH.

Proposal 1: Assume the following as the interference model:

- Number of interference cell: 2

- Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2

- Interference model: Reuse the assumption of the Rel-17 inter-cell interference requirement (as specified in B.6.2)

- INR: 7.77 for interference cell 1, 2.29 for interference cell 2

- SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

- Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

## 2.2	PDSCH requirement and parameters

Following the objectives, we propose to take the legacy Rel-17 inter-cell interference parameter configurations as baseline and do the evaluation. Then, make adjustment if it is needed to update some assumptions to define PDSCH requirement with inter-cell interference for 8Rx reception. In Rel-18, RAN4 defined UE demodulation requirements with 8Rx for both FR1 FDD and TDD, we propose to define the requirements for both FDD SCS=15kHz and TDD SCS=30kHz. Specifically, we propose the following assumptions for evaluation:

Proposal 2: Consider following parameter assumptions for inter-cell interference scenario

- Duplex CBW/SCS

  - FDD 10MHz/15kHz

  - TDD 40MHz/30kHz, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Antenna configuration

  - 2T8R ULA medium B

  - 4T8R ULA low

  - 8T8R ULA low

- Rank SCS

  - Rank 2, MCS19

  - Rank 4, MCS17

  - Rank 8, MCS17

- Channel model

  - TDLA30-10

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

- DMRS configuration

  - For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interference

UE capability and test applicability can be discussed if necessary and decide after RAN4 reaches agreements on the scope and basic parameter assumptions.

## 2.3	CQI reporting

As indicated in the objective of WID, we think it’s also necessary to define CQI reporting requirement for inter-cell interfering cancellation using 8Rx MMSE-IRC receiver. With this new requirement, the UE’s ability of tracking the channel variations and selecting the largest transport format possible using 8Rx reception under the inter-cell interference condition can be tested.

In Rel-17, RAN4 defined CQI reporting test in fading condition for inter-cell interference scenario. We therefore to define CQI reporting test in fading condition for 8Rx UE also. Regarding the interference model, we propose to reuse following assumptions as a starting point:

- One interfering cell

- CSI-IM overlaps with PDSCH from interference

- NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference

We suggest reusing the test metric as well:

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ;

- when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

We propose to consider the assumption assumed in Rel-17 CQI reporting fading tests with inter-cell interference as baseline, e.g., 6.2.3.1.2.3 (4Rx) in TS38.101-4 [2]

Proposal 3: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline

# 3	Intra-cell inter-user interference

In this section, we discuss the scenario of intra-cell inter-user interference cancellation with 8Rx reception.

## 3.1	Inter-user interference model

As for the intra-cell inter-user interference model, we propose to first reconsider the model used for Rel-17 NR performance enhancement and do some evaluations. Following aspects are within the pool of priority that need to be discussed so that to determine the overall interference model.

Number of paired UEs

Reuse the legacy pairing UE model to set up one target UE and one co-scheduled (interfering) UE. Since we are taking MMSE-IRC receiver as the baseline receiver, the target UE does not need to know the number of co-scheduled UEs and their ranks when it is processing in demodulation. Based on this, we propose to consider only 1 co-scheduled UE but with different layers in different test cases.

Proposal 4: Consider only 1 co-schedule UE in the interference model

Rank for target and interference PDSCH

Previously in Rel-17 intra-cell interference discussion, RAN4 agreed to consider rank combination 1+1 for 2Rx and 2+2 for 4Rx cases. As we are considering 8Rx reception, it’s possible to have additional rank 4 and rank 8 configurations on top of rank 2 configuration of Rel-17. Considering the real network deployment scenario, it’s also likely to configure a hybrid rank combination for target and interfering UE(s) if the Network can hardly find two paired UEs with the same rank configuration. Thus, we propose to evaluate following two possible rank combinations and further check the results before making any down-selection for the purpose of introducing requirements.

Proposal 5: Consider following rank configuration for initial evaluations

- For 2 layers

  - Target UE + interference UE:

    - 1+1

- For 4 layers

  - Target UE + interference UE:

    - 2+2

- For 8 layers

  - Target UE + interference UE:

    - 2+6

    - 4+4

Antenna configuration

As indicated in the objectives of the WID, new PDSCH requirement will be introduced for 8Rx UEs. Similar to the antenna configuration used in Rel-18 UE RF enh WI, we propose to consider 2Tx, 4Tx and 8Tx so that the test case design is suggested to be based on 2T8R, 4T8R and 8T8R.

Proposal 7: Consider following antenna configurations:

- 2T8R

  - For rank 1+1

- 4T8R

  - For rank 2+2

- 8T8R

  - For rank 2+6 and/or 6+6

Codebook type

As for the codebook type, we prefer to reuse the assumption of Rel-17 intra-cell interference cancellation that is Type I single panel codebook so that the precoding configuration can be assumed as randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [3].

Proposal 8: Consider Type I Single Panel only

PMI selection and precoding matrix generation

We think the precoding method used for Rel-18 NR demodulation performance evolution WI can be reconsidered, which is stated as follows:

- Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Proposal 9: Reuse the PDSCH & PDSCH DMRS Precoding configuration of Rel-18 NR demodulation performance evolution WI

PRB bundling size and precoding granularity

As for the PRB bundling size and precoding granularity, we prefer to 2PRBs for frequency domain and per slot for time domain, which is the same assumption as the existing UE demodulation requirements.

Proposal 10: 2PRBs for PRB bundling size and precoding granularity

MCS for interfering PDSCH

We propose to reuse the assumption for Rel-17 NR demodulation performance evolution WI which is the following:

- Random 16QAM symbols

Proposal 11: Assume Random 16QAM symbols for interfering PDSCH

## 3.2	PDSCH requirement and parameters

Following the objective, we propose to take the legacy Rel-17 intra-cell inter-user interference parameter as the baseline and do the evaluation, then update it with proper parameter assumptions if needed for introducing PDSCH requirement with inter-cell interference with 8Rx reception. We propose to consider following parameter assumptions as a starting point:

Proposal 12: Consider following parameter assumptions for intra-cell inter-user interference scenario

- Duplex CBW/SCS

  - For FDD 15kHz SCS: Cover 10MHz CBW

  - For TDD 30kHz SCS: Cover 40MHz CBW, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Antenna configuration

  - 2T8R ULA low

  - 4T8R ULA low

  - 8T8R ULA low

- MCS for target UE for evaluation

  - MCS17, MCS13

- Channel model

  - TDLA30-10

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

# 4	Summary

In this contribution, we shared our views on how to introduce PDSCH and CQI reporting requirement for two scenarios. Our proposals are summarized as follows:

Proposal 1: Assume the following as the interference model:

- Number of interference cell: 2

- Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2

- Interference model: Reuse the assumption of the Rel-17 inter-cell interference requirement (as specified in B.6.2)

- INR: 7.77 for interference cell 1, 2.29 for interference cell 2

- SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

- Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

Proposal 2: Consider following parameter assumptions for inter-cell interference scenario

- Duplex CBW/SCS

  - FDD 10MHz/15kHz

  - TDD 40MHz/30kHz, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Antenna configuration

  - 2T8R ULA medium B

  - 4T8R ULA low

  - 8T8R ULA low

- Rank SCS

  - Rank 2, MCS19

  - Rank 4, MCS17

  - Rank 8, MCS17

- Channel model

  - TDLA30-10

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

- DMRS configuration

  - For both serving and interfering cells, DMRS Type 1 with single symbol front loaded and 1 additional DMRS, with FDM applied between DMRS and data (number of CDM groups without data is equal to 1), i.e., overlapped DMRS between target and interference

Proposal 3: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline

Proposal 4: Consider only 1 co-schedule UE in the interference model

Proposal 5: Consider following rank configuration for initial evaluations

- For 2 layers

  - Target UE + interference UE:

    - 1+1

- For 4 layers

  - Target UE + interference UE:

    - 2+2

- For 8 layers

  - Target UE + interference UE:

    - 2+6

    - 4+4

Proposal 6: Consider following antenna configurations:

- 2T8R

  - For rank 1+1

- 4T8R

  - For rank 2+2

- 8T8R

  - For rank 2+6 and/or 6+6

Proposal 7: Consider Type I Single Panel only

Proposal 8: Reuse the PDSCH & PDSCH DMRS Precoding configuration of Rel-18 NR demodulation performance evolution WI

Proposal 9: 2PRBs for PRB bundling size and precoding granularity

Proposal 10: Assume Random 16QAM symbols for interfering PDSCH

Proposal 11: Consider following parameter assumptions for intra-cell inter-user interference scenario

- Duplex CBW/SCS

  - For FDD 15kHz SCS: Cover 10MHz CBW

  - For TDD 30kHz SCS: Cover 40MHz CBW, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Antenna configuration

  - 2T8R ULA low

  - 4T8R ULA low

  - 8T8R ULA low

- MCS for target UE for evaluation

  - MCS17, MCS13

- Channel model

  - TDLA30-10

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

# References

[1]	RP-241297: Revised WID on NR demodulation performance: Phase 5, China Telecom, NTT DOCOMO, RAN#104, Shanghai, China

[2]	TS38.101-4

[3]	TS38.214