3GPP TSG-RAN WG4 Meeting #112	R4-2412146

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	4.6

Source:	Ericsson

Title:	On the reference channel for CQI reporting requirement for mobile IAB

Document for:	Discussion

# 1	Introduction

This contribution discussed the solution to address the missing reference channel for CQI reporting requirement for mobile IAB.

# 2	Discussion

While reviewing the TS38.174 v18.5.0, we found that the measurement channel (parameter in the Table 11.2.3B.1.3.1-1 [1] as shown below) for the requirement of Sub-band Channel Quality Indicator (CQI) under fading conditions is ‘TBD’.

Table 11.2.3B.1.3.1-1: Test parameters Parameter; Unit; Test 1; Test 2. Bandwidth; MHz; 40. Subcarrier spacing; kHz; 30. Duplex Mode; TDD. Default TDD UL-DL pattern (Note 1); 7D1S2U, S=6:4:4. SNR; dB; 8; 9; 14; 15. Propagation channel; Two tap model specified in Annex B.2.4 with a=1, fD = 5Hz, and τd=0.1125μs. Antenna configuration; 2×2. Correlation configuration; As per Annex B.1. Beamforming Model; As specified in Annex I.3.1. ZP CSI-RS configuration; CSI-RS resource Type; Periodic. Number of CSI-RS ports (X); 4. CDM Type; FD-CDM2. Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0); Row 5,4. First OFDM symbol in the PRB used for CSI-RS (l0); 9. CSI-RS periodicity and offset; slot; 10/1. NZP CSI-RS for CSI acquisition; CSI-RS resource Type; Periodic. Number of CSI-RS ports (X); 2. CDM Type; FD-CDM2. Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0); Row 3,(6). First OFDM symbol in the PRB used for CSI-RS (l0); 13. NZP CSI-RS-timeConfig periodicity and offset; slot; 10/1. CSI-IM configuration; CSI-IM resource Type; Periodic. CSI-IM RE pattern; 0. CSI-IM Resource Mapping (kCSI-IM,lCSI-IM); (4, 9). CSI-IM timeConfig periodicity and offset; slot; 10/1. ReportConfigType; Aperiodic. CQI-table; Table 2. reportQuantity; cri-RI-PMI-CQI. timeRestrictionForChannelMeasurements; Not configured. timeRestrictionForInterferenceMeasurements; Not configured. cqi-FormatIndicator; Subband. pmi-FormatIndicator; Wideband. Sub-band Size; RB; 16. csi-ReportingBand; 1111111. CSI-Report periodicity and offset; slot; Not configured. Aperiodic Report Slot Offset; 8. CSI request; 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0. reportTriggerSize; 1. CSI-AperiodicTriggerStateList; One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM. aperiodicTriggeringOffset; Not configured. Codebook configuration; Codebook Type; typeI-SinglePanel. Codebook Mode; 1. (CodebookConfig-N1,CodebookConfig-N2); Not configured. CodebookSubsetRestriction; 000001. RI Restriction; N/A. Physical channel for CSI report; PUSCH. CQI/RI/PMI delay; ms; 9.5. Maximum number of HARQ transmission; 1. Measurement channel; TBD. Note 1: The same requirements are applicable to with different UL-DL patterns. Note 2: SSB, TRS, CSI-RS, and/or other unspecified test parameters with respect to TS 38.101-4 [28] are left up to test implementation, if transmitted or needed. Note 3: If the IAB-MT reports in an available uplink reporting instance at slot #n based on CQI estimation at a downlink slot not later than slot#(n-4), this reported CQI cannot be applied at the gNB downlink before slot#(n+4)..

The reason is that there is no correct reference channel defined in the current version of TS38.174. Our proposed solution is to add a new clause containing the correct reference channel for this test. Details of this solution is shown below:

### A.3B.3 Fixed Reference Channels for CSI reporting performance (NEW) This clause defines the DL signal applicable to the reporting of channel status information. Tables in this clause specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [11] and with MCS definition specified in clause 5.1.3 of TS 38.214 [11]. Table A.3B.3-1: Fixed Reference Channels for FR1 CSI reporting with CQI table 2 and MCS table 2 Reference channel; M-FR1-A.3B.3-1. Number of allocated PDSCH resource blocks; 16. Number of consecutive PDSCH symbols; 12. Number of PDSCH MIMO layers; 1. Number of DMRS REs (Note 1); 24. Overhead for TBS determination; 0. Available RE-s for PDSCH; 1920. CQI index; Spectral efficiency; MCS index; Modulation; Information Bit Payload per Slot. 0; OOR; OOR; OOR; N/A. 1; 0.1523; 0; QPSK; 456. 2; 0.3770; 1; 736. 3; 0.8770; 3; 1736. 4; 1.4766; 5; 16QAM; 2856. 5; 1.9141; 7; 3752. 6; 2.4063; 9; 4608. 7; 2.7305; 11; 64QAM; 5248. 8; 3.3223; 13; 6400. 9; 3.9023; 15; 7424. 10; 4.5234; 17; 8712. 11; 5.1152; 19; 9736. 12; 5.5547; 21; 256QAM; 10760. 13; 6.2266; 23; 12040. 14; 6.9141; 25; 13320. 15; 7.4063; 27; 14088. NOTE 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data NOTE 2: PDSCH is only scheduled on slots which are full DL.

Similar issue has been found in TS38.176-2 as well. Our corresponding CRs have been submitted to address this issue in two different specifications.

Proposal 1: Add a new clause to include the correct reference channel for Sub-band CQI reporting requirement for mobile IAB

# 3	Summary

In this contribution, we discussed the solution to address the missing reference channel for Sub-band CQI reporting requirement for mobile IAB. Our proposal is summarized below:

Proposal 1: Add a new clause to include the correct reference channel for Sub-band CQI reporting requirement for mobile IAB

# References

[1]	TS38.174, v18.5.0

[2]	TS38.176-2, v18.5.0