3GPP TSG-RAN4 Meeting # 112	R4-2412151

# Maastricht, Netherlands, 19th  – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0108 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-5: Correction on UE demodulation requirement for NTN FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test purpose is incorrect for FR2 PDSCH requirement. There are still square brackets on the SNR values. The reference channel for PDCCH requirement is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the test purpose - Remove the square brackets - Correct the reference channel for PDCCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The minimum requirement of PDSCH and PDCCH for NTN FR2 are not completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.2.1, 11.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1------------------------------------------------------------

11.2.2.1.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 11.2.2.1.1.1-3 with the addition of test parameters in Table 11.2.2.1.1.1-2 and the downlink physical channel setup according to Annex A.3.

The test purposes are specified in Table 11.2.2.1.1.1-1.

Table 11.2.2.1.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 1 receive antenna conditions and with different channel models and MCS | 1-1, 1-2, 1-3, 1-4, 2-1, 2-2, 2-3, 2-4 |

Table 11.2.2.1.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4. |
| Number of HARQ Processes |  |  | 16 for Test 1-1, Test 1-2 32 for Test 1-3 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes for Test 1-4 in which 4 disabled processes are randomly selected at test configuration |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 80 for Test 1-1, Test 1-2, Test 1-3 and Test 1-4 2080 for Test 2-1, Test 2-2, Test 2-3 and Test 2-4 |
| Maximum number of HARQ transmission |  |  | 4 for Test 1-1, Test 1-2, Test 1-3 1 for Test 1-4 (re-Tx disabled for all HARQ processes) |

Table 11.2.2.1.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 1-2 | R.PDSCH.3-4.1 FDD | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | 11.2 |
| 1-3 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 1-4 | R.PDSCH.3-3.1 FDD (Note 1) | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 4.2 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Table 11.2.2.1.1.1-4: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 2-2 | R.PDSCH.3-4.1 FDD | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-1200 | 1x1 | 70 | 11.2 |
| 2-3 | R.PDSCH.3-3.1 FDD | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 3.5 |
| 2-4 | R.PDSCH.3-3.1 FDD (Note 1) | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-1200 | 1x1 | 70 | 4.2 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

-----------------------------------------------------------End of change 1---------------------------------------------------------------

----------------------------------------------------- Beginning of Change 2------------------------------------------------------------

##### 11.2.3.1.1	Minimum requirements with 1Tx Antenna

For the parameters specified in Table 11.2.3.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 11.2.3.1.1-1. The downlink physical setup is in accordance with Annex C.5.1.

Table 11.2.3.1.1-1: Minimum performance requirements with 1 Tx Antenna

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR(dB) |
| 1-1 | 200 | 132 | 1 | 8 | R.PDCCH.1-1.1 | NTN-TDLC5-1200 | 1x1 Low | 1 | 4.6 |
| 1-2 | 200 | 132 | 2 | 16 | R.PDCCH.1-1.2 | NTN-TDLC5-1200 | 1x1 Low | 1 | 3.2 |

-----------------------------------------------------------End of change 2---------------------------------------------------------------