3GPP TSG-RAN4 Meeting # 112	R4-2412156

# Maastricht, Netherlands, 19th  – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.176-1 | CR | 0058 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.176-1: Correction on mIAB-MT requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reference channel index is incorrect for IAB CSI reporting requirement |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the reference channel index |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSI reporting requirement for IAB is not configured with correct reference measurement channel |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR … CR… |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Mirror CR to R4-2413550 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1------------------------------------------------------------

## A.3.5	Fixed Reference Channels for CSI reporting performance requirements

This clause defines the DL signal applicable to the reporting of channel state information

Tables A.3.5-1 specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [24] and with MCS definition specified in clause 5.1.3 of TS 38.214 [24].

The parameters for the reference measurement channels are specified in table A.3.5-2 for FR1 PMI reporting performance requirements

Table A.3.5-1: Fixed Reference Channels for CSI reporting

| Reference channel |  |  |  | M-FR1-A.3.5-1 | M-FR1-A.3.5-2 | M-FR1-A.3.5-3 | M-FR1-A.3.5-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 3 | 4 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 12720 | 12720 | 12720 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 2976 | 5896 | 8976 | 11784 |
| 2 | 0.3770 | 1 |  | 4744 | 9480 | 14344 | 18976 |
| 3 | 0.8770 | 3 |  | 11016 | 22536 | 33816 | 45096 |
| 4 | 1.4766 | 5 | 16QAM | 18960 | 37896 | 56368 | 75792 |
| 5 | 1.9141 | 7 |  | 24576 | 49176 | 73776 | 98376 |
| 6 | 2.4063 | 9 |  | 30728 | 61480 | 92200 | 122976 |
| 7 | 2.7305 | 11 | 64QAM | 34816 | 69672 | 104496 | 139376 |
| 8 | 3.3223 | 13 |  | 42016 | 83976 | 127080 | 167976 |
| 9 | 3.9023 | 15 |  | 49176 | 98376 | 147576 | 196776 |
| 10 | 4.5234 | 17 |  | 57376 | 114776 | 172176 | 229576 |
| 11 | 5.1152 | 19 |  | 65576 | 131176 | 196776 | 262376 |
| 12 | 5.5547 | 21 | 256QAM | 69672 | 139376 | 213176 | 278776 |
| 13 | 6.2266 | 23 |  | 79896 | 159880 | 237776 | 319784 |
| 14 | 6.9141 | 25 |  | 88064 | 176208 | 262376 | 352440 |
| 15 | 7.4063 | 27 |  | 94248 | 188576 | 278776 | 376896 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |  |  |  |

-----------------------------------------------------------End of change 1---------------------------------------------------------------