3GPP TSG-RAN WG4 Meeting # 112	R4-2412190

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.12.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on PUR requirements for IoT NTN

Document for:	Discussion

# Introduction

In Rel-18 IoT NTN, there is on remaining issue on PUR validation conditions, which is not treated for quite a long time. In this contribution, we provide our views on this issue.

# Discussion

In current requirements for PUR in IoT NTN, there is on FFS point for the TA validation conditions, which is shown as follows:

#### 4.6A.3.3 Requirements on TA validation for transmission using PUR The UE is allowed to transmit using PUR provided that TA is valid. [ FFS for the validation conditions] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

The history of the discussion is also shown as follows:

RAN4#104 R4-2214350 #### __Issue 2-6: Transmission using preconfigured uplink resources (PUR) - Continue discussion the following aspect in the next meeting - whether to reuse the legacy RSRP-based TA validation for PUR in IoT NTN - The UE update the uplink timing for transmitting on PUR using the configured TA command according to TS 36.211 v17.2.0 i.e. transmission of uplink radio frame number from the UE starts TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs - The UE assumes TA is valid provided that the following conditions are met, otherwise the TA is considered invalid: - Satellite assistance information (SAI) is valid i.e. T317 has not expired and - Current time of the UE is at least DT seconds earlier than t-Service. RAN4#104b R4-2217264 #### __Issue 2-9-1: PUR, RSRP-based TA validation - Option 1: The legacy RSRP-based TA validation is not applicable for PUR in IoT NTN. - Option 2a: The legacy RSRP-based TA validation is applicable for PUR in IoT NTN, for both GEO and LEO. - Option 2b: The legacy RSRP-based TA validation is applicable for PUR in IoT NTN for GEO but not LEO. #### __Issue 2-9-2: PUR in NGSO, TA validation for NTN based on t-service The following proposals can be FFS in next meeting - Proposal 1a (new): The UE assumes TA is valid provided that the following conditions are met, otherwise the TA is considered invalid (Ericsson) - Current time of the UE is at least DT seconds earlier than t-Service, where DT is the configured PUR periodicity. RAN4#105 R4-2220362 #### Issue 2-3-1: PUR, RSRP-based TA validation - Agreement - Do not define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO

Based on the discussion above, there were mainly two points about PUR TA validation:

- Whether to define legacy RSRP-based TA validation

- Whether to introduce TA validation based on t-Service

For the first issue, RAN4 has discuss it and reached agreements that do not define legacy RSRP-based TA validation. The reason is that RSRP is to tell the changing TA implicitly in TN. In NTN, UE can tell the change explicitly by GNSS and ephemeris.

#### 5.3.3.19 Timing alignment validation for transmission using PUR The UE shall consider the timing alignment value for transmission using PUR to be valid when the following conditions are fulfilled: 1> either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers; and 1> either pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is not configured or the following conditions are fulfilled: 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not increased by more than increaseThresh; and 2> compared to the stored serving cell reference (N)RSRP value, the serving cell (N)RSRP has not decreased by more than decreaseThresh;

It could be observed that, there is two ways to verify the TA. One is the timer and the other is the RSRP-based solution. Since RAN4 has agreed that no RSRP-based TA validation requirements, it is reasonable that NW configure timer to control the TA validation. Meanwhile, there is no need to introduce additional validation based on t-service since the PUR timer can already serve the same purpose which is more accurate and controllable.

Observation 1: Without RSRP-based TA validation, NW can guarantee the TA is valid by PUR timer.

Observation 2: TA validation for PUR can be guaranteed by existing PUR timer.

Observation 3: TA validation could be directly referred to existing RAN2 spec (e.g. 36.331 5.3.3.19).

Thus, we propose following TP to fix the remaining issue:

Proposal 1: Update the requirements for TA validation using PUR as follows:

#### 4.6A.3.3 Requirements on TA validation for transmission using PUR The UE is allowed to transmit using PUR provided that TA is valid provided that pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as defined in [2] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

The above change could be also be found in our accompanied CR in this meeting.

# Conclusions

Observation 1: Without RSRP-based TA validation, NW can guarantee the TA is valid by PUR timer.

Observation 2: TA validation for PUR can be guaranteed by existing PUR timer.

Observation 3: TA validation could be directly referred to existing RAN2 spec (e.g. 36.331 5.3.3.19).

Proposal 1: Update the requirements for TA validation using PUR as follows:

#### 4.6A.3.3 Requirements on TA validation for transmission using PUR The UE is allowed to transmit using PUR provided that TA is valid provided that pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as defined in [2] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

# References