3GPP TSG-RAN WG4 Meeting # 112	R4-2412196

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.14.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on requirements maintenance for Rel-18 eFeRRM SCell activation

Document for:	Discussion

# Introduction

The core part and performance part of Rel-18 WI of even further RRM enhancement was completed with remaining issues captured in WF [1]. In this contribution, we further provide our views on remaining issues.

# Discussion

Regarding the applicable scenarios and requirements multiple SCells activation, the progress are summarized as follows:

Issue 1-1: Applicability of multiple SCell activation with L3 reporting on FR1 and FR2 band Agreement: - For FR1, L3 reporting based multiple SCell activation requirements are applicable to unknown target SCell activation when there is no contiguous active serving cell or there is no contiguous known SCell(s) to the unknown to-be-activated SCell on the FR1 band. - This condition will be added section 8.3.18. - For FR2, L3 reporting based multiple SCell activation requirements are applicable to unknown target SCell activation when there is no active serving cell or there is no known SCell(s) on the same band. - This condition will be added section 8.3.18. Issue 1-2: Multiple SCell activation requirement with L3 reporting based on issue 1-1 - Discuss directly in the CR.

The above agreements are captured in the spec as follows:

In EN-DC, NE-DC, standalone NR, or in one CG of NR-DC, the requirements in this clause shall apply when the following conditions are met: - UE only receives one single MAC command for multiple SCell activation within the activation period defined in this clause, and - in each single CG, there are no other SCell activation, deactivation, addition or release before activation is completed for all the SCells activated by the single MAC CE in this clause, and - in EN-DC and NE-DC, there are no E-UTRAN SCell activation, deactivation, addition or release before multiple SCell activation is completed in this clause, and - all to-be-activated SCells are unknown on the same FR2 band, and there is neither active serving cell(s) nor known SCell(s) on the same band, or, - all to-be-activated SCells are unknown on the same FR1 band, and there is neither active serving cell contiguous to the SCell nor known SCell(s) contiguous to the to-be-activated SCell on the same band, and - the UE reports valid L3 measurement results after receiving the SCell activation command for the to-be-activated SCell in FR1, or at least one unknown SCell in the same FR2 band In two CGs of NR-DC, the requirements in this clause shall apply when the following conditions are met: - UE receives one MAC command per CG for multiple SCell activation within the activation period defined in this clause, and - UE supports per-FR measurement gap capability, and - all to-be-activated SCells are unknown on the same FR2 band, and there is neither active serving cell(s) nor known SCell(s) on the same band, or, - all to-be-activated SCells are unknown on the same FR1 band, and there is neither active serving cell contiguous to the SCell nor known SCell(s) contiguous to the to-be-activated SCell on the same band, and - the UE reports valid L3 measurement results after receiving the SCell activation command for the to-be-activated SCell in FR1, or at least one unknown SCell in the same FR2 band Otherwise, Clause 8.3.7 is applied.

During the discussion in last meeting, we don’t have enough time to solve all issues. Companies only focused the requirements applicability rules. The following question is how to modify the equation as now it is exactly the same as single SCell activation requirements. As mentioned in previous meeting, it should be divided into FR1 and FR2 separately.

For FR2, the situation is much clearer that the applicable case can be interpreted as:

- All to-be-activated SCell are unknown without known to-be-activated SCell or active serving Cell;

- One or more to-be-activated SCells has valid L3 report;

Based on above, the requirements defined for single CC in clause 8.3.17 can apply to each target SCell with some modification as:

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17 except the definition of Tuncertainty_MAC and Tuncertainty_RRC are replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

-	Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

Proposal 1: For FR2 target SCell, the requirements can be updated as follows:

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17 except the definition of Tuncertainty_MAC and Tuncertainty_RRC are replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

-	Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

For FR1, the situation is much more complicated. Similar as the analysis for FR2, the scenario could be summarized as follows:

- All to-be-activated SCell are unknown without known to-be-activated SCell or active serving Cell;

- The target FR1 SCell has valid L3 reprot;

However, for FR1 multiple SCells activation, based on the principles in legacy requirements, UE may still need to perform AGC using SMTC_Max even the target FR1 SCell is known. If looking at the legacy requirements, the exact number of samples for AGC are further differentiated considering following aspects:

- Measure period for the target Cell

- Whether there is unknown to-be-activated SCell

- The measurement period for the parallel to-be-activated SCell

Following the requirements for single SCell activation in 8.3.17, there is no further differentiation with respect to the measurement period for the target Cell. Thus, follow the same principle, we only need to consider whether there is pure unknown to-be-activated SCell. The cases can be summarized as follows:

- On the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation.

- All parallel to-be-activated SCells are unknown with valid L3 report.

Based on the analysis above, for the second case, single CC requirements can apply directly for each target SCell. For the first case, the SMTC_MAX shall be considered in the equation, which makes the procedure more complicated. We use following fig to elaborate the case.

Fig.1 FR1 multiple SCell activation with L3 report.

The requirements are proposed as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

Max(AGC, L3 report + PDCCH TCI)

Max(L3 report + P/SP CSI-RS, Max() + fine timing )

Based on the analysis above, the requirements for FR1 are proposed to be modified as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation. TFirstSSB_MAX_multiple_scells, TSMTC_MAX_multiple_scells is defined in 8.3.7; if on the same band, UE does not have any parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation, requirements in 8.3.17 apply.

Proposal 2: For FR1 target SCell, the requirements can be updated as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation. TFirstSSB_MAX_multiple_scells, TSMTC_MAX_multiple_scells is defined in 8.3.7; if on the same band, UE does not have any parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation, requirements in 8.3.17 apply.

The above proposed changes can also be found in our companied CR in this meeting.

# Conclusions

Proposal 1: For FR2 target SCell, the requirements can be updated as follows:

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17 except the definition of Tuncertainty_MAC and Tuncertainty_RRC are replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

-	Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

Proposal 2: For FR1 target SCell, the requirements can be updated as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation. TFirstSSB_MAX_multiple_scells, TSMTC_MAX_multiple_scells is defined in 8.3.7; if on the same band, UE does not have any parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation, requirements in 8.3.17 apply.

# References

[1] R4-2410259 WF for [111][205] NR_RRM_enh3_part1