3GPP TSG-RAN WG4 Meeting # 112		R4-2412201

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.29.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for R18 NES

Document for:	Discussion

# Introduction

The performance part for R18 NES were completed in last meeting with agreements and remaining issues captured in WF [1]. In this, paper, we further provide our views on remaining issues.

# Discussion

One remaining issue is about the TC case introduced for Cell DTX, where UE is expected to fulfilled the requirements as legacy one. Companies proposed two issues as shown as follows:

Issue 3-1-1: NES Cell DTX test case Proposals: - Option 1: For cell DTX test case, the TAT is set to 1280ms, and the UE shall be scheduled with PUSCH at every cell DTX cycle. (vivo) - Option 2: RAN4 confirms whether TE can allocate UL resources based on UE’s PRACH or SR in cell DTX inactive time. (vivo)

As analysis in the paper [2], per RAN1 specification, UE is not expected to received UE specific PDCCH, which means gNB can only schedule PUSCH for L3 report within on-duration. As Cell DTX cycle is configured with 640ms, even UE sends SR for L3 report, UE has to wait for next on-duration to get UL grant. It means, the delay caused by waiting PUSCH shall be considered in the test requirements. Another issue is about the TAT configuration, which is configured as Infinity in current spec. From our understanding, if the TAT is set to non-infinite value, UE needs to get TAC before the timer expires, otherwise, UE will not be able to transmit UL. As TAT is configured as infinity, no matter whether there is TAC command or not, UE is always to transmit UL (L3 report) as long as UL resource is scheduled. In Cell DTX test cases, SSB is transmitted and UE is able to maintain the DL timing accordingly, with infinite TAT timer and constant TA, we see no problem for UL transmission in the test cases.

Observation 1: With Cell DTX, UE can only be scheduled with PUSCH for L3 reporting in on-duration, which will increase measurement reporting delay.

Proposal 1: For Cell DTX test, UE can be scheduled with PUSCH for L3 reporting during on-duration of Cell DTX. The delay uncertainty for next available PUSCH should be considered in the test requirements.

# Conclusions

Observation 1: With Cell DTX, UE can only be scheduled with PUSCH for L3 reporting in on-duration, which will increase measurement reporting delay.

Proposal 1: For Cell DTX test, UE can be scheduled with PUSCH for L3 reporting during on-duration of Cell DTX. The delay uncertainty for next available PUSCH should be considered in the test requirements.

# References

R4-2410269 WF on RRM requirements for NR network energy saving

R4-2407777 Discussion on test cases for R18 NES